3GPP TSG-RAN WG4 Meeting # 104bis-e											R4-2217760

Electronic Meeting, 10– 19 October 2022

Title: 	WF on NS mapping for intra-band CA

Agenda Item:	9.4

Source: 	Huawei, HiSilicon

Document for:	Approval

# Background

In RAN4#104 meeting, WF [1] on clarification of the NS mapping from RAN2 to RAN4 was approved with the following interpretations and way forward.

It can be seen that the RAN2 specification only refers back to Table 6.2.3.1-1A for 38.101-1 (ignoring the 38.101-2 specification for the context of this discussion) which contains the single carrier NS.  The RAN2 specifications do NOT refer back to clause 6.2A.3.1.1 for CA_NS or clause 6.2A.3.1.2 for CA_NC_NS and further state that a common additionalSpectrumEmission is applicable for all uplink carriers in the NC or CCA configuration.

Two possible interpretations exist.

- Only the single carrier NS mapping from the PCC applies (cell specific NS signalled in SIB). The mapping to CA_NS and CA_NC_NS must conform to the NS mapping as defined in Table 6.2.3.1-1A. For example, assume that the basestation indicates additionalSpectrumEmission = 1 in Band n41.  According to Table 6.2.3.1-1A, this maps to NS_04. When CA is configured, the same additionalSpectrumEmission = 1 applies and checking Table 6.2.3.1.1-2 for CCA, it can be seen that CA_NS_04 applies to the CA_n41 configuration.  However, if instead additionalSpectrumEmission = 2 had been indicated, the single carrier mapping would be to NS_47 but there is no mapping for CA_NS and CA_NC_NS. The RAN4 specification should be corrected to add the mappings for CA_NS and CA_NC_NS corresponding to additionalSpectrumEmission = 2; for example, this might mean to define CA_NS_47 and CA_NC_NS_47. All future CA_NS and CA_NC_NS should ensure there is a 1:1 mapping according to the NS mapping.  Alternative, since the mapping is 1:1, the CA_NS and CA_NC_NS designations are redundant and can be removed referring instead only to the NS (of course, there are different A-MPR tables for CCA and NCCA and potentially different emissions, but they can still be indexed by NS rather than CA_NS or CA_NC_NS).

- It is the CA_NS or CA_NC_NS that applies when the SCC is configured and activated.  It was the intention that a separate and distinct additionalSpectrumEmission is conveyed to the UE for CA (via dedicated signalling) when the SCC is configured and therefore RAN4 defined independent tables for CA_NS and CA_NC_NS. This SCC additionalSpectrumEmission is mapped according to Table 6.2A.3.1.1-2 (38.101 spec has an error and incorrectly refers to this table as 6.2.3.1.1-2 without the “A” suffix for CA) and Table 6.2A.3.1.2-2 for CCA and NC CA, respectively. RAN2 specification should be updated to point to clause 6.2A.3.1.1 for CA_NS or clause 6.2A.3.1.2 for CA_NC_NS.

Separately, it has been reported that RAN2 has made the following agreement with respect to Band n77 NS signaling

-- For UL CA in n77 with at least one cell in DoD-band and at least one cell in C-band, the network may configure either NS_55 or NS_01 for UL carrier(s) in DoD-band, and NS_01 for the remaining uplink carrier(s) in this band.

This agreement can be accommodated by either option 1 or option 2 although the implementation may slightly differ.

<Way forward/Agreement>:

- There has been no consensus on option 1 or option 2 in RAN4.  The WF is therefore to continue discussion between these options at RAN4 #104-bis-e.

Option 1: The additionalSpectrumEmission associated with PCC applies for NS, CA_NS, and CA_NC_NS.  RAN4 specification update needed.

Option 2: The additionalSpectrumEmission associated with configuration/activation of the SCC applies (analogous to LTE with a separate parameter) for CA_NS and CA_NC_NS.  RAN2 specification update needed.

In this meeting, companies provided some contributions [2~5] to address this issue. Based on the 1st round discussion summary, it seems that some consensus can be reached. This way forward will capture them and find a solution.

# Way forward

Qualcomm: the signalling are redundant. There is no difference between CA_NS_xx and CA_NC_NS. We think they are misleading. The point that requirements are different is agreeable. But it should be a single pointer.

T-Mobile USA: Qualcomm comment is aligned with RAN2 signaling. Having a single reference makes sense. We proposed replacing CA_NS_XY and CA_NC_NS_XY with NS_XY in the tables. The reason is that CA_NS_XY and CA_NC_XY do not exist in the RAN2 specs.

OPPO: the different views last. We can divide WF 1 into two parts

Huawei: the framework of additional spectrum emission of NR is different from LTE. CA_NS.. and CA_NC_NS refer to RF requirements. They did not impact RAN2 IEs. Thus RAN4 defines the tables associated with the RAN2 notation. Removing CA notation would cause confusion in RAN4. We need CA notation to differentiate the requirements.

Mediatek: we think that we have to discuss the A-SEM and A-MPR separately. The A-MPR requirements could be impacted by notation.

Ericsson: agree with Huawei and support the way forward. We would like to know NS value is RAN4 only notion and does not exist in RAN2 specification. It is for single serving cell in uplink. CA_NS value is also RAN4 notation and indicate two uplink cases for both serving cells. We should not remove them. Otherwise, it is not clear which requirements will be applied. The notation indicates different requirements sometimes A-MPR is allowed.

Apple: understand that from RAN2 signalling there is only one parameter. The requirement should be the same no matter whether it is single carrier or CA. From RAN4 perspective, we agree with Ericsson. A-MPR requirement could be different between single carrier and contiguous/non-contiguous CA. We slightly keep them in RAN4 spec.

Intel: RAN2 signaling does not provide the flexibility to indicate different cases. In the way forward, we have common understanding. I do not see the reason why we should re-write them since they are there for a long time.

Nokia: we do not agree with Huawei comment. We understand that there is only spectrum emission IE. But the containers can be different in RAN2 spec. RAN2 spec can refer to suitable tables in RAN4 spec if going with Qualcomm. Qualcomm way is more friendly to new reader for RAN4 spec.

T-Mobile USA: we provide the different slightly alternative, i.e., keeping the able and replacing CA_NS_XY and CA_NC_NS_XY with NS_XY in the tables. Different requirements will apply but there is no different signalling.

Mediatek: in RAN4 spec, the requirements of A-SEM are the same for NS_XY and CA_NS. Therefore the modification in RAN2 spec is not necessary. Only difference in RAN4 is about how to address A-MPR issue and indicator.

Skyworks: from RAN4 spec point of view, it may be enough make NS_XY and CA_NS corresponding to one signalling IE.

Huawei: Agree with Skyworks. For WF2, it can be up to UE implementation.

Ericsson: we can explain in RAN4 spec what CA_NS means, and it should be different from NS. That would be enough. That would be different from NS_XY which is for single uplink. If we mix the NS and CA_NS, it is confusing to reader. For PCell, you could have more than one NS value but for SCell you have just one.

Nokia: regarding PCell and SCell discussions, we agree with Ericsson. In some cases, multiple NS can be indicated. These discussions should be conducted in RAN2.

Mediatek: We have different view from Nokia. In RAN2 and RAN4, the spec has already indicated the per band capability of additional spectrum emission for all uplink carriers no matter PCell and SCell under consideration. Also in RAN4 CA_NC_NS_04 also follows the NS_04.

Nokia: I do not see all NS values are the same across carriers in the CA does not matter to me. If people believes there is problem for PCell and SCell, the issue should be discussed in RAN2.

AT&T: Ericsson suggestion of explanation in RAN4 can work.

Qualcomm: Emission requirements should be the same. Is it the common understanding?

Skyworks: there should be difference, i.e., Intra-band CA vs single carrier. They are in different tables.

Mediatek: we check the tables. The requirements do not conflict with each other.

Qualcomm: Fundamentally there is no difference between CA and single carrier, since the requirements are based on regulation.

<Way forward 1>:

- CA_NS_XX and CA_NC_NS_XX in RAN4 spec are associated with specific RF requirements

  - A-SEM and A-MPR requirements

- FFS whether to keep them in RAN4’s spec and FFS on solutions for the issue.

<Way forward 2>: Whether additionalSpectrumEmission for CA_NS_XX, CA_NC_NS_XX or HO is associated with PCell or SCell should be discussed in RAN2, if the issue is identified.

Mediatek: we wonder why WF2 is needed.

# Reference

[1] R4-2214409, WF on clarification of the NS mapping from RAN2 to RAN4, Qualcomm Incorporated

[2] R4-2215334, Impact of NS mapping between RAN2 and RAN4 specs on n77 issue, Nokia, Nokia Shanghai Bell

[3] R4-2215794, Discussion on additionalSpectrumEmission for NS mapping and CRs associated with Canada and US band n77, Mediatek

[4] R4-2216063, Discussion on NS mapping for intra-band UL CA, Huawei, HiSilicon

[5] R4-2216649, Remove network signalling labels for CA_NS and CA_NC_NS, Qualcomm