3GPP TSG-RAN WG4 Meeting # 104bis-e											R4-22xxxx

Electronic Meeting, October 10 – October 19, 2022

Title: 	WF on Measurement without gaps for UEs reporting NeedForGapsInfoNR

Agenda Item:	6.10.3.1

Source: 	Intel Corporation

Document for:	Approval

# Topic #1: Measurement without gaps for UEs reporting NeedForGapsInfoNR (AI 6.10.3.1)

## Sub-topic 1-1: Interruption

### Issue 1-1-1: Whether interruption is expected when UE reports ’no-gap’ in ‘NeedForGapsInfoNR'

< Way forward/Agreement >:

- Option 1:

  - Yes

- Option 2

  - No

- Option 3:

  - Introduce additional UE capability or the new indication of the existing UE capability (e.g. as part of needForGap) to differentiate whether UE needs interruption

| Company | Comments |
| --- | --- |
| Ericsson | Option 3 We’re fine to introduce an additional indication to further explain the ‘no gap’s meaning in current UE reporting. |
| CMCC | Option 2. In LTE spec, it is clearly stated that the measurement is conducted without gaps and without interruption. Same appraoch can be used for NR. It is not expected that NR has worse performance than LTE. |
| Xiaomi | Support option 1 and option 3. For option 3, we prefer to introduce additional UE indication to differentiate whether interruption is expected based on the current NeedForGap signalling framework. |
| ZTE | Support Option 1 and 3. Regarding to Option 3, similar view as Ericsson and Xiaomi, we are fine to indicate an additional UE indication to tell NW whether interruption is needed or not, such indication can be some extension of NeedForGap signalling framework. |
| Apple | Support option1 and option 3. On option 3 we are open to further discuss the new indication, e.g. whether as part of NeedForGap or a new capability. On option 2. We think it is necessary to emphasize that “to allow interruption” doesn’t mean “NR has worse performance than LTE”. Whether interruption is needed depends on UE architecture and target band combo. Since it has been comprehensively studied during NCSG design, we don’t need to repeat the study again. The outcome is that on some bands in certain combo interruption is needed while other are not needed. For example, when both serving cell and target MO are both in low band, the two RF chains may share some common components. Interruption may be needed when UE switching on/off one of the chains. If we go with LTE approach (always no interruption), UE probably indicate ‘gap’ on the bands requiring interruption. Thus for this particular case, the improvement in NR is that, UE could indicate ‘no-gap’ but with interruption, compared with the case wherein UE has to indicate ‘gap’. |
| Qualcomm | Since the requirement is missing when UE report ‘no-gap’ in R16 requirement for inter-frequency measurement, we thought the requirement may need to define when UE report no-gap but UE does not support interFrequencyMeas-Nogap-r16 or the SSB is not completely contained in the active BWP of the UE. Thus, we switch our position to define the requirement. However, if option1 is also only appliable for R18 UE, we will keep the original position that no need to work for needforgap. NCSG framework will cover both interruption and no interruption cases. So, we would like to ask companies to share their understanding whether this WI is considering Rel-16 UE does not support interFrequencyMeas-Nogap-r16 or the SSB is not completely contained in the active BWP. Because NW cannot differentiate UE is R16 or R18 when UE reports no-gap. |
| Intel | Support Option 1 and Option 3. In our understanding, such requirements are targeted to Rel18 UE. For Rel18 UE, there is ambiguity from ‘no-gap’ in ‘needForGap’. To answer QC’s question: For NW side, ‘no-gap’ reported from UE could be differentiate indeed because NW can also know UE’s capability to support NCSG or not. For Rel16 UEs which need not support NCSG, when they report ‘no-gap’ it is clearly indicated no any gaps (legacy gap or NCSG) will be used. So the interruption is not allowed in Rel16. But for Rel18 UE, if UE report ‘no-gap’ at least from the current spec(TS38.331), it is possible that not legacy ‘measurement gap’ but NCSG can be used. For the later case (NCSG used when report ‘no-gap’), the interruption shall be allowed. |
| vivo | Ok with option 3 |
| OPPO | Support option 3. we are also fine with the suggestion from Ericsson and Xiaomi, additional indication to differentiate ‘no-gap’. |
| Huawei | We can compromise to option 3. The details of the indication can be further discussed. |

### Issue 1-1-2: Requirements on the interruption length , if allowed

< Way forward/Agreement >:

- Option 1:

  - As a starting point, the interruption length can be same as these defined for NCSG,e.g.

    - When UE reporting “no-gap” in NeedForGapInfoNR  the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

    - When UE reporting “others[TBD]” in NeedForGapInfoNR no interruption allowed

- Option 1a:

  - As a starting point, the interruption length can be same as these defined for NCSG,e.g.

    - When UE reporting “no-gap” in NeedForGapInfoNR  the interruption length can be specified based on the same RTT assumption(1ms in FR1 and 0.75ms in FR2)  as for NCSG interruption occasion.

    - When UE reporting “others[TBD]” in NeedForGapInfoNR no interruption allowed

- Option 1b:

  - As a starting point, the interruption length can be same as these defined for NCSG,e.g.

    - When UE signals that interruption is needed for gap-less measurements the interruption length can be VIL=1 ms in FR1 and VIL=0.75 ms in FR2.

- Option 2:

  - Consider smaller interruption length than VIL1+VIL2 from NCSG for a UE that requires additional interruptions for measurements without gaps.

- Option 3(Harmonization):

  - As a starting point, when UE reporting “no-gap” in NeedForGapInfoNR, the interruption length can be specified based on the same RTT assumption as for NCSG (0.5ms in FR1 and 0.25ms in FR2) interruption occasion.

| Company | Comments |
| --- | --- |
| Ericsson | Fine with option 3. |
| Xiaomi | Fine with option 3. |
| ZTE | Fine with option 3. |
| Apple | Support option 1a and 1b. technical discussion on the possible interruption has already been comprehensively discussed in NCSG design. It took several meetings for RAN4 to converge on this issue. We don’t expect RAN4 spends too much effort to re-open the discussion. |
| Qualcomm | It depends on Issue 1-1-4. We prefer option1b without using VIL wording as reference interruption length to define interruption ratio. However we first need to get clarification of the scenario and scope as we raised question in issue 1-1-1. |
| Intel | Support Option 1a and Option 3. To Qualcomm’s comments above: yes, these issues depended issue1-1-1. But as we mentioned in GTW, they can be discussed in parallel. |
| vivo | Ok with option 1 and option 3 |
| OPPO | Support option 3. |
| Huawei | Support option 1/1a/1b. We have similar view as Apple that conclusion on the interruption length was extensively discussed in Rel-17 NCSG, and we prefer to re-use it when defining interruption for NFG. We also agree with QC that the key part in option 1/1a/1b is the interruption length (1ms in FR1 and 0.75ms in FR2), and we may not need to use the term VIL. |

### Issue 1-1-3: Requirements on the interruption location , if allowed

< Way forward/Agreement >:

- Option 1:

  - Interruption location needs to be specified.

  - FFS on the specific location of interruption allowed

- Option 2:

  - No need to define the specific interruption location but the total interruption ratio

| Company | Comments |
| --- | --- |
| Ericsson | FFS |
| Xiaomi | Prefer Option 1 |
| ZTE | Support Option 1. |
| Apple | FFS. Open to further study which option is better. |
| Qualcomm | Support Option 2. Interruption is not same as small gap in NCSG. Interruption can be placed anywhere and interrupt data reception per scenario. So, we support not specify location of interruption but ratio. However we first need to get clarification of the scenario and scope as we raised question in issue 1-1-1. |
| Intel | Can be FFS. Prefer Option 1. |
| OPPO | Prefer option 1. |
| Huawei | Suggest FFS |

### Issue 1-1-4: Requirements on the interruption ratio, if allowed

< Way forward/Agreement >:

- Option 1:

  - RAN4 to further discuss how to control the total interruption ratio for NeedForGaps capability

- Option 1a:

  - RAN4 needs to define the total interruption ratio if no specific interruption location was required.

- Option 2:

  - RAN4 needs not to define total interruption ratio but the interruption

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. From our understanding, even if RAN4 agrees to define the specific interruption location, we should still follow the rule to control the total interruption ratio. |
| Xiaomi | Prefer option 2. If the interruption location is to be specified, we prefer not to define the interruption ratio, similar as NCSG. |
| ZTE | Prefer Option 2. Once the interruption location and length identified, the ratio can be controlled. Do not need additional discussion on interruption ratio. |
| Apple | FFS. Depends on issue 1-1-3. |
| Qualcomm | Option2. Same comment in Issue1-1-3. |
| Intel | Can be FFS. |
| vivo | Prefer option 2. |
| OPPO | Prefer option 2. And we agree this should depends on issue 1-1-3. |
| Huawei | Suggest FFS, should be discussed together with issue 1-1-3. @Moderator, it seems option 2 is incomplete. |

### Issue 1-1-5: Other aspect on whether to allow interruption

< Way forward/Agreement >:

- Proposal 1:

  - If interruption is allowed when UE reports ‘no-gap or others [TBD]’, the interruption should be allowed for each of intra- and inter-frequency measurements for which UE reports ‘no-gap’.

    - The interruption will impact all the serving cells if UE does not support per-FR gap, and all the serving cells in the same FR as the measurement if UE supports per-FR gap.

- Proposal 2:

  - The switching from ‘gap’ to ‘no-gap’ would be accompanied by the change of UE RF BW and/or MO, so interruption can be allowed when UE reports ‘no-gap’ through NeedForGap

| Company | Comments |
| --- | --- |
| Ericsson | We support P1-1: - When UE reports ‘no-gap’ and further indicates ‘no gap’ means ‘nogap-noncsg’ and ‘ncsg’, the interruption should be allowed for each of intra- and inter-frequency measurements for which UE reports ‘no-gap’. - The interruption will impact all the serving cells if UE does not support per-FR gap, and all the serving cells in the same FR as the measurement if UE supports per-FR gap. - When UE reports ‘no-gap’ and further indicates ‘no gap’ means ‘nogap-noncsg’ only, the interruption isn’t allowed for each of intra- and inter-frequency measurements for which UE reports ‘no-gap’. |
| Xiaomi | Generally fine with P1, we can further discuss the corresponding relations between UE indication and interruption in next meeting. |
| ZTE | Support P1. |
| Apple | We are fine with P1 in general. We assume the intention is to differentiate per-UE gap and per-FR gap. On P1-1 from E///, we think it is more about how to align UE capability reporting for NeedForGap and NCSG, which can be discussed under 1-3-1. |
| Qualcomm | We are fine with option1. However we first need to get clarification of the scenario and scope as we raised question in issue 1-1-1. |
| Intel | P1 is fine for us but the reporting indication can be FFS. |
| vivo | Support P1 |
| OPPO | Support option 1. |
| Huawei | Support P1, and the final wording can be depending on Issue 1-1-1. |

## Sub-topic 1-2: Measurement reporting delay requirements

[Moderator notes: it is better to differentiate the measurement without gap into the two scenarios below when considering the measurement reportint delay requirements as for the interruption requirements:

- Case 1: without gap and no interruption (e.g. ’nogap’ or ’nogap-nointerruption[TBD]’ indicated in NeedForGapInfoNR)

- Case 2: without gap but interruption allowed (e.g. ’nogap’ indicated in NeedForGapInfoNR)

]

### Issue 1-2-1: Requirement for intra-freq measurement without gap when no interruption (intra-f case 1)

< Way forward/Agreement >:

- Reuse requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) for the reporting delay requirements for intra-frequency measurement without gap and no interruption allowed

### Issue 1-2-2&Issue1-2-4 Requirement for intra-freq/inter-freq measurement without gap when interruption allowed (case 2)

< Way forward/Agreement >:

- Option 1:

  - Take requirements NCSG requirements as a starting point

  - The other aspects can be FFS. e.g.

    - The time slot alignment among the measurement objects and interruption location

- Option 2:

  - The deactivated SCell measurement requirement can be the start point.

  - The other aspects can be FFS, e.g.

    - The frequency layers in the band for which UE reports ‘no gap’ should be counted in CSSF outside gap

- Option 3:

  - Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point

| Company | Comments |
| --- | --- |
| Moderator | These open issues can be postponed to next meeting |

## Sub-topic 1-3: UE behaviour

### Issue 1-3-1: UE behaviours when UE supports both NeedForGap and NCSG capabilities

< Way forward/Agreement >:

  - FFS on:

    - Proposal 1: Error: Reference source not found

    - Proposal 2: Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2

| Company | Comments |
| --- | --- |
| Ericsson | Support P1 P1 should be updated as follow if option 3 in issue 1-1-1 is agreed. When UE further indicates interruption is allowed, - UE should report ‘no gap’ in the same band for NeedForGaps if reporting ‘nogap-noncsg’ or ‘ncsg’ in a band for NCSG - UE should report ‘gap’ in the same band for NeedForGaps if reporting ‘gap’ in a band for NCSG When UE further indicates no interruption is needed, - UE should report ‘no gap’ in the same band for NeedForGaps if reporting ‘nogap-noncsg’ in a band for NCSG - UE should report ‘gap’ in the same band for NeedForGaps if reporting ‘gap’ or ‘ncsg’ in a band for NCSG For P2, we have one question: How to understand no change for legacy behaviour since RAN4 didn’t define any UE behaviour for NeedForGaps? |
| CMCC | This issue pending on whether interruption is allowed for “NeedForGap”, can be further discussed. |
| Xiaomi | In our understanding, even if NeedForGap and NCSG are quite similar, they are still two independent UE capability. We still prefer not to bundle them. Open to further discuss. |
| ZTE | Wait for the outcome of whether interruption is allowed for “NeedForGap”. |
| Apple | We agree with P1 in principle. Otherwise NW may be confused if UE supports both NCSG and NeedForGap. Details can be FFS. |
| Qualcomm | We are okay waiting the outcomes of 1-1-1. |
| Intel | We agreed that there is overlapping among ‘needForGap’ and ‘needForNCSG’. But how to associate them or not can be FFS. |
| vivo | OK with P1. |
| OPPO | We are fine with option 1. |
| Huawei | Suggest FFS since the issue depends on outcome of issue 1-1-1. |

### Issue 1-3-2: UE behaviour mismatch between UE and NW

< Way forward/Agreement >:

  - Error: Reference source not found

    - Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps

    - Rel-16 UE which supports NeedForGaps in a Rel-17 NW which supports NCSG

    - Both UE and NW support NCSG and NeedForGaps

| Company | Comments |
| --- | --- |
| Ericsson | Fine with the recommended WF. |
| CMCC | Can be FFS. Furthermore, it is confusing about the wording “Rel-16 NW” and “Rel-17 NW”. |
| Xiaomi | FFS |
| ZTE | FFS |
| Apple | We are fine to further study. However, as commented in the 1st round, we are not clear what to study for the first two sub-bullets. For example, the first sub-bullet, a Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps. We assume the R16 NW can only understand R16 NeedForGap reporting, regardless of whether this R17 UE support NCSG or not. If the UE support R16 NeedForGap, it would report ‘gap’ or ‘no-gap’ via R16 NeedForGap. It may or may or cause interruption regardless of what RAN4 defines in R18, since corresponding requirements are missing in R16. Similar concern on the second sub-bullet. |
| Qualcomm | we first need to get clarification of the scenario and scope as we raised question in issue 1-1-1. |
| Intel | Can be FFS |
| vivo | FFS |
| OPPO | FFS |
| Huawei | Suggest FFS, and fine with the recommended WF. |

## Sub-topic 1-4: Scheduling availability

### Issue 1-4-1: Scheduling availability

< Way forward/Agreement >:

  - Take the similar requirements for NCSG (TS38.133 v17.6.0 9.3.10.3) as baseline to define scheduling availability

| Company | Comments |
| --- | --- |
| Ericsson | Fine with the recommended WF. |
| Xiaomi | Fine with the recommended WF. |
| ZTE | Fine with the recommended WF. |
| Apple | Agree with the recommended WF. |
| Qualcomm | we first need to get clarification of the scenario and scope as we raised question in issue 1-1-1. |
| Intel | Agree with the recommended WF. To Qualcomm: this scenario is when the interruption shall be allowed. These issues depended issue1-1-1 can be discussed in parallel. |
| vivo | Agree with the recommended WF. |
| OPPO | Fine with the recommended WF. |
| Huawei | Fine with the recommended WF. |

## Sub-topic 1-5: Requirements applicability

### Issue 1-5-1: General requirements applicability

< Way forward/Agreement >:

  - The requirement (e.g. the new UE capability to definite the interruption needed when UE report ‘nogap’ in Rel18) shall apply only for R18 UE who report no-gap. No impact on other release UE

| Company | Comments |
| --- | --- |
| Moderator | No further discussion needed for the technical perspective but some more exact wording is expected |
| Ericsson | From our understanding, the NeedForGaps requirement shall be applied from R18 UE regardless of UE’s reporting status. |
| Apple | Agree with the recommended WF. |
| Qualcomm | We raised question in issue1-1-1. Seems companies understands requirement regardless of interruption, no-interruption, or new capability is only applicable for R18 UE. In this case, we do not think any further work is needed for needforgap. NCSG is enough to cover which was our original condition. So we suggest to clarify the scenario and scope before moving forward. |
| Intel | Agree with the recommended WF. For the proposal itself, we assumed that we are discussing requirements for Rel18 UE who can support the necessary capability. |
| vivo | Agree with the recommended WF. |
| OPPO | Fine with the recommended WF. |
| Huawei | Fine with the recommended WF. |

# Reference

[1] RP-221018, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation