3GPP TSG-RAN WG4 Meeting # 104bis-e										R4-22xxxxx

Electronic Meeting, October 10 – October 19, 2022

Title: 	WF on inter-RAT measurement without gap

Agenda Item:	6.10.3.2

Source: 	Intel Corporation

Document for:	Approval

# Topic #2: inter-RAT measurement without gap(AI 6.10.3.2)

## Sub-topic 2-1: Using scenarios

[Moderator notes:

Up to this meeting, all proposed possible using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

We can use these index below to aviod any confustion.

]

### Issue 2-1-1: extend to MR-DC cases

< Way forward/Agreement >:

  - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-1 in SA only if case a-1 is agreed as one of inter-RAT NR target scenario (issue 2-1-3)

  - Define capability and/or requirements to support inter-RAT E-UTRAN measurement without gap case b-1 in SA only if case b-1 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4)

  - FFS on:

    - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-2 in in both SA and MR-DC if case a-2 shall be considered in as one of inter-RAT NR target scenario (issue 2-1-3).

    - Define capability and/or requirements to inter-RAT E-UTRAN measurement without gap case b-2 in both SA and MR-DC if case b-2 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4)

| Company | Comments |
| --- | --- |
| Ericsson | We suggest to update the FFS for MR-DC only: - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-2 in SA if case a-2 shall be considered in as one of inter-RAT NR target scenario (issue 2-1-3). - FFS on MR-DC - Define capability and/or requirements to inter-RAT E-UTRAN measurement without gap case b-2 in SA and if case b-2 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4) - FFS on MR-DC |

### Issue 2-1-2 Numerology

< Agreement >:

  - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, the mixed numerology need to be supported.

    - FFS on whether the additional UE capability is needed

### Issue 2-1-3: inter-RAT NR target scenario

< Way forward/Agreement >:

  - Case a-1 (Another spare RF chain is available for UE) can be considered as the inter-RAT NR target scenario.

  - FFS on Case a-2

| Company | Comments |
| --- | --- |
| Ericsson | Fine with the recommended WF. |
| CMCC | OK with the way forward. We support to consider case a-2, but we are also OK for FFS. |

### Issue 2-1-4: inter-RAT LTE target scenario

< Way forward/Agreement >:

  - Case b-2(The target CRS to be measured is with UE’s active BWP) shall be considered the inter-RAT LTE scenario under objective 2 of NR_MG_ehn2-core WI

  - FFS on Case b-1

| Company | Comments |
| --- | --- |
| Ericsson | From our understanding, Case b-1 has also agreed. We suggest companies to further check whether defined in Rel-17 or merged into Rel-18 since the issue such as ‘scheduling restriction’, ‘scaling factor’ and ‘measurement window’ are same as case b-2. |
| CMCC | OK with the way forward. We support to consider case b-1, but we are also OK for FFS. |

## Sub-topic 2-2: UE capabilities

### Issue 2-2-1: On top of which UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain avaiable (Case a-1)

< Way forward/Agreement >:

  - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1)

    - The exact value to be reported can be FFS.

  - FFS on top of NCSG to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1)

| Company | Comments |
| --- | --- |
| Ericsson | Fine with the recommended WF. |
| CMCC | Cannot agree only consider “interRAT-NeedForGapsNR-r16”. We propose to consider both NCSG and interRAT-NeedForGapsNR-r16. The reason we propose to consider NCSG is that according to TS 36.133 (the details are duplicated as following), NCSG can be used for inter-RAT measurement, but the requirements are missing. It is necessary to specify the requirements. TS36.133: UEs supporting network controlled small gap and which have signaled that they are capable of measurements without gap but requiring NCSG, can be configured with a network controlled small gap pattern in table 8.1.2.1.3-1 on all component carrier(s) to perform inter-frequency and inter-RAT measurement.. It is proposed to further check on NCSG. The WF is proposed as following (update part is highlighted in yellow): - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1) - The exact value to be reported can be FFS. - FFS on top of NCSG to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1) |

### Issue 2-2-2: On top of which UE capability to support the inter-RAT NR measurement without gap when the measurement reference signal can be contained within UE’s LTE channel bandwidth (Case a-2)

< Way forward/Agreement >:

  - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-2)

    - The exact value to be reported can be FFS.

| Company | Comments |
| --- | --- |
| Ericsson | We suggest to further check whether this is a valid case. |
| CMCC | Cannot agree on wayforwad. More discussion is needed. For case a-2, since the measurement reference signal is contained within UE’s LTE channel bandwidth, UE could mesurement it without gap even if there is no spare RF chain. In our view, this could be a per-UE capability. But “interRAT-NeedForGapsNR-r16” is reported per band per band combination. TS 36.306 #### 4.3.6.38 interRAT-NeedForGapsNR-r16 This field defines for each supported E-UTRA band or band combination whether measurement gaps are required to perform SSB based inter-RAT measurements on each supported NR band.. |

### Issue 2-2-3: On top of which UE capability to support the inter-RAT LTE measurement requirements when UE has vacant RF chain available(Case b-1)

< Way forward/Agreement >:

  - Reuse the existing capability.

    - UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement

    - FFS on: UE indicates via NeedForGap.

| Company | Comments |
| --- | --- |
| Ericsson | Fine with the recommended WF. |
| CMCC | OK with the recommended WF. We support to consider both NCSG and NeedForGap. But we are also fine to further study NeedForGap. |

### Issue 2-2-4: On top of which UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

< Way forward/Agreement >:

  - Option 1:

    - the capability of NeedForGaps for inter-frequency measurement wo gap in Rel16 can be taken as the baseline

  - Option 1a:

    - extend the capability of NeedForGaps

  - Option 2:

    - A new UE capability should be defined to indicate whether UE supports MG-less measurement in this scenario

| Company | Comments |
| --- | --- |
| Ericsson | Option 2. It should a static capability similar as ‘inter-frequency wo gap’ other than related to any RRC reporting mechanism by UE. |
| CMCC | Open for discussion, prefer option 2. For case b-2, measurement reference signal is contained within UE’s LTE channel bandwidth, no need to be coupled with NeedForGap. |

## Sub-topic 2-3: Measurement requirements

### Issue 2-3-1: General principle to define the requirements

< Way forward/Agreement >:

  - FFS on the general principle to define the requirements for inter-RAT NR measurements

    - Proposal 1:

      - For the inter-RAT NR measurements without gap, the  requirements frameworks can be based on NR inter-frequency measurement without gap

    - Others are not precluded

  - FFS on the general principle to define the requirements for inter-RAT LTE measurements

    - Proposal 2:

      - For the inter-RAT LTE gap-less measurement when LTE CRS to be measured is contained in UE’s active BWP, the requirements can be based on the measurement requirements for LTE inter-frequency measurement.

    - Proposal 3: Ericsson

      - RAN4 needs to further discuss where to perform the inter-RAT E-UTRAN measurement without gaps to reduce the performance degradation to legacy NR intra-frequency measurement without gap and L1-RSRP measurement.

      - Both NW and UE shall have the same understanding on the measurement occasions for Inter-RAT E-UTRAN measurement without gap.

    - Others are not precluded

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

### Issue 2-3-2: Scheduling restriction

< Way forward/Agreement >:

  - FFS on the scheduling restriction

    - Proposal 1:

      - The existing scheduling availability specified for intra-frequency measurements in TS 38.133 section 9.2.5.3 can also be applied to the inter-RAT measurement without measurement gaps as a start point.

    - Proposal 2:

      - Error: Reference source not found

      - Error: Reference source not found

      - Error: Reference source not found

    - Others are not precluded

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

### Issue 2-3-3: Searcher limitation

< Way forward/Agreement >:

  - FFS on the searcher limitation

    - Proposal 1:

      - Error: Reference source not found

    - Others are not precluded

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

### Issue 2-3-4: CCSF

< Way forward/Agreement >:

  - The updates of CSSF requirements when these inter-RAT measurements without gap introduced (e.g. CSSF_outside_gap) is needed

  - FFS on:

    - The scaling factor for inter-RAT E-UTRAN measurement without gap equals to the total number of frequency layers for E-UTRAN measurement without gap.

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

### Issue 2-3-5: Interruption requirements for inter-RAT measurement without gap

< Way forward/Agreement >:

  - FFS on whether interruption is allowed for inter-RAT NR measurements without MG

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

### Issue 2-3-6: Effective measurement window

< Way forward/Agreement >:

  - FFS on:

    - RAN4 needs to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap.

      - The effective measurement window can be defined based on measurement duration, measurement periodicity and offset.

    - Others are not precluded

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

### Issue 2-3-7: Release independent requirements

< Way forward/Agreement >:

  - FFS on whether to introduce the inter-RAT LTE measurements without gap case b-2 as release independent from Rel-17

  - Others are not precluded

| Company | Comments |
| --- | --- |
| Moderator | Postponed to next meeting. |

# Reference

[1] RP-222337, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation