3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-22xxxxx

Electronic Meeting, 10– 19 October 2022

Title: 	WF on L1/L2 inter-cell mobility

Agenda Item:	6.20.3

Source: 	MediaTek Inc.

Document for:	Approval

# L1/L2 inter-cell mobility

## Sub-topic 1-1 General and Scenarios

### Terminology

<Way forward/Agreement>: Issue 1-1-1 Terminology of L1/L2 based inter-cell mobility

Tentative agreement in the 1st round:

Wait and follow RAN2’s conclusion on the terminology of L1/L2 based inter-cell mobility.

Recommendations for 2nd round: No further discussion.

### Simultaneous data Rx/Tx?

<Way forward/Agreement>: Issue 1-1-2: Whether to consider simultaneous data Rx/Tx with both source cell and target cell

Tentative agreement in the 1st round:

For intra-frequency L1/L2 mobility, not consider simultaneous data Rx/Tx with both source cell and target cell during L1/L2 inter-cell mobility delay.

Please provide further comments on inter-frequency L1/L2 mobility case

  - Option 1 (QC, Huawei, Xiaomi, Intel, Ericsson, MTK, Apple, OPPO, CTC, CATT): For inter-frequency L1/L2 mobility, not consider simultaneous data Rx/Tx with both source cell and target cell during L1/L2 inter-cell mobility delay.

  - Option 2 (Nokia, vivo): FFS

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Support option 1. |
| MTK | Support Option 1. Regarding vivo’s comment in the 1st round “For UE capable of CA, there is no need for this restriction”, we would like to clarity that the intention is not to exclude such UE with higher capability. The benefit of higher UE capability may be reflexed in cell delay requirements or the interruption. In addition, supporting CA does not mean supporting simultaneous data Rx/Tx with both source cell and target cell during cell switch. Simultaneous data Rx/Tx with both source cell and target cell requires higher UE capability from higher layer to physical layer. |
| Huawei | Option 1. Inter-frequency DAPS requires high UE complexity, e.g., dual protocol stacks, enhanced RF/baseband capability. |

### Single panel & multiple panel

<Way forward/Agreement>: Issue 1-1-3: Whether to consider simultaneous multi-panel in FR2

No tentative agreements in 1st round.

Please provide further comments on the compromised proposal:

  - Compromised proposal: Focus on single active panel in FR2 in L1/L2 mobility

    - Further discuss whether to consider simultaneous multi-panel in FR2 after the discussions in multi-Rx WI converge.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Fine with the Compromised proposal. |
| MTK | Support the compromised proposal. Regarding “whether L1/L2 mobility can be applied to FR2 inter-cell mTRP”, it is another issue that needs further discussion in our understanding. |
| Huawei | Fine with the compromised proposal. |

Intra-frequency & inter-frequency

<Way forward/Agreement>: Issue 1-1-4: Definition of L1 intra-frequency/inter-frequency measurement

Tentative agreement in the 1st round:

  - For SSB L1-RSRP measurement, follow the definition of L3 measurement:

    - A measurement is defined as a SSB based intra-frequency L1 measurement provided the center frequency and SCS of the SSB of the neighbor cell is the same as SSB of the serving cell indicated in ServingCellConfigCommon

Note: RAN4 will revisit the definition based on RAN1/2 conclusion.

Recommendations for 2nd round: No further discussion.

<Way forward/Agreement>: Issue 1-1-5: Whether to cover inter-frequency L1-RSRP measurement

No tentative agreements in 1st round.

Note: Assuming the definition of “inter-frequency L1 measurement” here in the Issue is conceptually the same as “inter-frequency L3 measurement.”

RAN2 has reached the following agreement in the ongoing RAN2#119bis meeting and will send RAN4 a LS. Moderator suggests no further discussion in the 2nd round, waiting RAN2 LS and discussing the feasibility to support inter-frequency L1 measurement next meeting.

- RAN2 agreement - Inter-freq L1L2 mobility: R2 Confirms that For L1L2 mobility inter-freq scenarios in general should be supported (including mobility to inter-frequency cell that is not a current serving cell), including the support of inter-frequency L1 measurements, if feasible by R4 and R1.

Recommendations for 2nd round: No further discussion.

<Way forward/Agreement>: Issue 1-1-6: Whether to cover inter-frequency cell switch

No tentative agreements in 1st round.

The difference is 1-1-5 is from the point of measurement, 1-1-6 is from the point of cell switch. The definitions of intra-frequency and inter-frequency may be different from the point of measurement and the point of cell switch. Take the scenario “the target PCell is a current Scell” as an example, from the point of measurement, the SCC is still an intra-frequency. But from the point of cell switch, this is inter-frequency cell switch.

To make it clearer, moderator changes the title of the issue from “whether to cover inter-frequency L1/L2 inter-cell mobility” to “Whether to cover inter-frequency cell switch”.

Please provide further comments on the following options

  - Option 1 (Huawei, Xiaomi, MTK, vivo, Nokia, CTC): support inter-frequency L1/L2-based mobility, where the SSBs of active serving cell(s) and the corresponding candidate target cell(s) are on different frequency layers

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Fine with Option 1 if RAN2 has decide to support it. |
| MTK | Support Option 1. And understand the intention to discuss this issue: inter-frequency cell switch ≠ inter-frequency measurement |
| Huawei | Support inter-frequency cell switch after clarification from Moderator. |

### Synchronous & non-synchronous

<Way forward/Agreement>: Issue 1-1-7: Definition of synchronous and non-synchronous

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1 (Intel, MTK, Apple, OPPO, Huawei (from UE reception respective)): From the point of measurement, synchronous scenario will refer to timing offset smaller than CP between source cell and target cell.

  - Option 2 (CATT): take the following into consideration

- Whether the time offset between the serving cell and the adjacent cell under test is within CP?

- Whether the time offset between the serving cell and the adjacent cell under test is within MRTD/MTTD?

- Whether the UE needs to do RACH to obtain TA in the target cell?

- Whether some information is synchronized between the source cell and target cell in the interface?

  - Option 3 (vivo, Huawei (from network perspective)): From RAN4 perspective, non-synchronous scenario refers to the case when slot boundary between serving cell and neighbour cell is not aligned, i.e. larger than TAE, from gNB perspective, e.g. FDD. All other cases are called synchronous.

  - Option 4 (Ericsson, Nokia, CATT): reuse the legacy definition of sync and async for L3 HO

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Since RAN2 agreed that Rel-17 ICBM is not a prerequisite for using L1/L2 mobility in last meeting, we are fine to further discuss the synchronous definition according to RAN2 progress. |
| MTK | Our intention is defining sync from the point of L1 measurement for the convenience of further discussion. It is fine for us to define sync and async from the point of cell switch if needed. When discussing L1 measurement, we are fine to use RTD < CP or RTD≥CP instead of synchronous or non-synchronous. Option 4 is referring to DAPS? As far as I know, there is no legacy definition of sync and async for legacy L3 HO. |
| Huawei | This issue depends on UE capability (e.g., accommodate how long time difference with single FFT) and network deployment. Recalling the synchronization definition of DAPS (Table 6.1.3.2-1) which took sufficient analysis and discussion, this issue in this WI needs more discussion. |

<Way forward/Agreement>: Issue 1-1-8: Whether to cover non-synchronous scenarios

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1 (QC, Intel, vivo, Nokia, CATT): wait for RAN2’s progress

  - Option 2 (MTK, Apple, OPPO):

    - start from synchronous L1-RSRP measurement

    - async case: FFS

  - Option 3 (xiaomi, Ericsson, vivo, CATT): No need to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Support Option 1. |
| MTK | Option 2. We suggest start from the RTD between serving cell and neighbour cell within CP at first. And further discuss RTD larger than CP case based on RAN2 progress. |
| Huawei | Option1. |

### Relation of L1 measurement and L3 measurement

<Way forward/Agreement>: Issue 1-1-9: Relation between L3 measurement and L1 measurement

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1 (Huawei, MTK, QC, Apple, OPPO): Network shall configure L1 measurement on a neighbor cell after receiving L3 measurement report on that cell

    - FFS whether the spec has to define such a constraint explicitly.

  - Option 2 (Xiaomi, Nokia): L3 measurement report is not the prerequisite of L1 measurement configuration on a neighbor cell

  - Option 3 (Intel): Further discuss whether to support inter-cell L1-RSRP measurement when L3 measurement is not available recently and wait for RAN2 progress.

  - Option 4 (Ericsson, Apple, Nokia): Candidate cell L1-RSRP measurements can be measured within SMTC

  - Option 5 (vivo): wait for RAN1/2 progress

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Support Option 3 and Option 5. For Option 1, that’s the known cell condition defined for Rel-17 L1 measurement for cell with different PCI, and the requirement is only defined for known cell case. However, in current RAN2 discussion, the whole mobility timeline is under discussing. It’s FFS whether Rel-17 L1 measurement can replace L3 measurement before HO command. In that case, we are not sure whether there is any L3 measurement before the L1 measurement. therefore, we suggest to wait for RAN2 progress. |
| MTK | Support option 1. We wonder how NW knows which cell to measure L1 RS without L3 measurement report. RAN4 can make the conclusion but not wait for RAN1/2 as R17 ICBM. We don’t think L1 measurement would replace L3 measurement as cell search and SBI reading are both necessary. For option 4, we are open. |
| Huawei | Option 1. L3 measurement shall be always performed as the cell detection is always needed. The possible procedure is that the limited number of candidate cells selected through L3 measurement and then network configure L1/L2 related measurements related with the candidate cells and then UE report corresponding L1/L2 measurement report. |

### Single cells & multiple cells

<Way forward/Agreement>: Issue 1-1-10: Number of intra-frequency layers to measure per band

As the majority view is that it is premature to discuss this issue, moderator suggests postponing the discussion.

  - Option 1 (MTK, Apple, OPPO): For L1-RSRP measurement on neighbor cell, UE measures only one intra-frequency layer on each FR2-1 band in FR2-1 CA

  - Option 2 (QC, Huawei, Xiaomi, Ericsson, Nokia): Premature to discuss

  - Option 3 (vivo): discuss after concluding in issue 1-1-4 (definition of SSB L1-RSRP intra-frequency)

Recommendations for 2nd round: No further discussion

<Way forward/Agreement>: Issue 1-1-11: Number of cells to measure per intra-frequency layer

Tentative agreement in the 1st round:

Wait for RAN1/2 progress.

Recommendations for 2nd round: No further discussion.

### Others

<Way forward/Agreement>: Issue 1-1-12: L1-RSRP resources window

Tentative agreement in the 1st round:

Wait for RAN1/2 progress.

Recommendations for 2nd round: No further discussion.

## Sub-topic 1-2 L1-RSRP measurement requirements

<Way forward/Agreement>: Issue 1-2-1: L1-RSRP measurement delay requirements

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1 (MTK, Nokia): use the measurement delay requirements for L1 measurement on NSC in R17 as a baseline

  - Option 2 (QC, Xiaomi, Intel): Wait for RAN1/2 progress

  - Option 3 (Huawei, Ericsson, Apple, vivo): FFS

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Support Option 2. RAN2 agreed that Rel-17 ICBM is not a prerequisite for using L1/L2 mobility in last meeting. |
| MTK | We understand other companies concern. We are fine to discuss this later. |
| Huawei | Option 3. There are many open issues which impact measurement delay, e.g., the number of frequency layers, the number of non-serving cells, sync/non-sync scenarios, so it is premature to agree on option 1. |

<Way forward/Agreement>: Issue 1-2-2: Side condition in intra-frequency L1-RSRP measurement accuracy requirements

No tentative agreements in 1st round.

As we focus on side condition discussion.at this stage, moderator make some revision on the issue title.

Please provide further comments on the following options

  - Revised Option 1 (Huawei, MTK, Apple, vivo): Reuse legacy value SNR=-3dB

  - Revised Option 2 (vivo): SNR =-9dB (same as L3 measurement)

  - Revised Option 3 (QC, Intel, Ericsson, Nokia): FFS

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Intel | Support Option 3 to wait for RAN2 progress. |
| MTK | Support Revised Option 1. “-3dB” is already higher than the side condition for L3 HO. Double check with vivo: -9dB is side condition for L3 measurement? If I am not wrong, it should be -6dB. |
| Huawei | Revised Option 1. In R15, the side condition of L1-RSRP measurement is 3dB. One reason is that the sample number of L1-RSRP measurement is smaller than L3 measurement, i.e., 1 or 3. To guarantee measurement accuracy, higher SINR is set. 3dB side condition is set for R17 ICBM L1-RSRP as well. The reason is that the non-serving cell is selected by network and it is supposed to in a good condition. Therefore we don’t find strong view to use a lower SNR for R18 inter-cell mobility. We are open to further discuss. |

## Sub-topic 1-3 L1/L2 inter-cell mobility delay requirements

<Way forward/Agreement>: Issue 1-3-1: L1/L2 cell switch delay

No tentative agreements in 1st round.

As the majority (7/12) view is to wait for RAN1/2 progress, moderator suggest no further discussion in the 2nd round

  - Option 1 (Xiaomi, Intel): The timeline for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

  - Option 2 (Apple):

    - For RACH-less case (if supported), it is defined as the time UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell.

    - For RACH-based case (if supported), it is defined as the time UE receives the cell switch command to UE starts transmission of the new uplink PRACH channel to the target cell.

  - Option 3 (CMCC): taking RAN2 agreements on HO interruption time for L1/L2-based inter-cell mobility into account, and discuss following issues

    - For the RAN2 agreements that end point of HO interruption is when UE performs the first UL transmission on the indicated beam of the target cell, it is proposed to further discuss whether first UL transmission refer to PRACH transmission or UL data? If it refers to UL data, how to reflect this in RAN4 HO interruption requirements

    - For the RAN2 agreements that end point of HO interruption is when UE performs the first DL reception on the indicated beam of the target cell, it is proposed to further discuss how to reflect this in RAN4 HO interruption requirements

  - Option 4 (Huawei, MTK): For RACH-based case, the start point is UE receiving the cell switch command, the end point is UE transmitting PRACH to the target cell.

  - Option 5 (QC, Intel, Ericsson, vivo, OPPO, Nokia, CTC): wait for RAN2 progress.

Recommendations for 2nd round: No further discussion.

<Way forward/Agreement>: Issue 1-3-2: Components of L1/L2 inter-cell mobility delay

Background: To discuss whether to include other components in L1/L2 inter-cell mobility delay except the components in the following table.

| Components | Meaning |
| --- | --- |
| Tcmd | Time for processing L1/L2-command (HARQ and parsing) |
| Tprocessing,2 | Time for UE processing. This may include L2/3 reconfiguration, RF retuning, baseband retuning, security update if needed, etc. |
| Tsearch | Time required to search the target cell |
| TΔ | Time for fine tracking and acquiring full timing information |
| Tmargin | Time for SSB or CSI-RS post-processing |
| TIU | interruption uncertainty in acquiring the first available PRACH occasion in the new cell |

No tentative agreements in 1st round.

As the majority (6/10) view is to wait for RAN1/2 progress, moderator suggest no further discussion in the 2nd round.

  - Option 1 (Huawei, Intel, xiaomi): TCI state switching time is needed

  - Option 2 (Apple): L1/L2 inter-cell mobility execution time is needed.

  - Option 3 (MTK):

    - FFS to add TCI state switching time in L1/L2 mobility delay

    - Not add L1/L2 inter-cell mobility execution time in L1/L2 mobility delay

  - Option 4 (OPPO)

    - FFS to add TCI state switching time in L1/L2 mobility delay

    - FFS to add L1/L2 inter-cell mobility execution time in L1/L2 mobility delay

  - Option 5 (Nokia): Use “Tswitch-cmd processing” to replace “Tcmd and Tprocessing,2”

  - Option 6 (QC, Intel, Ericsson, vivo, OPPO, Nokia): wait for RAN1/2 progress

Recommendations for 2nd round: No further discussion

<Way forward/Agreement>: Issue 1-3-3: Components of L1/L2 cell switch interruption Tinterruption

No tentative agreements in 1st round.

As the majority (5/9) view is to wait for RAN1/2 progress, moderator suggest no further discussion in the 2nd round.

  - Modified Option 1 (Huawei, CMCC, Apple): further discussion

  - Option 2 (QC, Ericsson): all the other components in L1/L2 inter-cell mobility delay except Tcmd.

  - Option 3 (MTK, CTC): Focus on the delay requirement at first.

  - Option 4 (QC, Ericsson, Apple, vivo, Nokia): wait for RAN1/2 progress

Recommendations for 2nd round: No further discussion

<Way forward/Agreement>: Issue 1-3-4: On each component

No tentative agreements in 1st round.

As the majority (6/10) view is to wait for RAN1/2 progress, moderator suggest no further discussion in the 2nd round.

  - Option 1 (MTK): further consider the possibility of reducing Tprocessing,2, Tsearch and TΔ

  - Option 2 (Huawei): further analyze each component of L1/L2 inter-cell mobility delay:

    - Handover command processing delay: processing of L1 or L2 (MAC CE) is faster than RRC

    - Tsearch=0

    - FFS TCI state switching time

    - FFS reduction on Tprocessing,2

    - reuse legacy value for TIU for RACH-based L1/L2 mobility, FFS for RACH-less

  - Option 3 (Xiaomi): For L1/L2 based inter-cell mobility,

    - the MAC/DCI decoding delay instead of RRC processing delay should be defined in HO delay requirement;

    - the delay of cell search is not needed in HO delay requirement;

    - the UE processing time can be reduced in HO delay requirement;

    - fine timing tracking and RACH uncertainty delay need to be considered in HO delay requirement

  - Option 4 (Nokia):

    - LLM (low layer mobility) cell switch interruption time should be minimized, and upper limit should be agreed not to exceed the existing L3 HO interruption time

    - RAN4 is to review the delay components of the existing definition for L3 handover and discuss the adaptability of such definition in LLM

  - Option 5 (QC, Intel, Ericsson, Apple, vivo, OPPO): wait for RAN1/2 progress

Recommendations for 2nd round: No further discussion

## Sub-topic 1-4 Others

<Way forward/Agreement>: Issue 1-4-1: Transmit timing accuracy requirements

Tentative agreement in 1st round:

Transmit timing accuracy requirements for any uplink transmission should follow existing requirements as a starting point.

Recommendations for 2nd round: confirm the tentative agreement unless other reasonable technical reason is proposed.

Note: Please Nokia clarify whether there is addition impact on spec raised by some companies in the 1st round.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Huawei | Agree with the tentative agreements. |