3GPP TSG-RAN WG4 Meeting #104-bis-e										  R4-2217261

Electronic Meeting, 10– 19 October 2022

Title: 	WF on RRM requirements for Rel-17 MUSIM gaps

Agenda Item:	6.21.3

Source: 	vivo

Document for:	Approval

# Topic #1: RRM requirements for Rel-17 MUSIM gaps

## Sub-topic 1-1 On collision between MUSIM and legacy gaps

Issue 1-1-1: Clarification on the scope of Rel-17 legacy gap

- Proposals:

  - P1: The scope of legacy gap includes gap patterns defined in TS38.133 table 9.1.2-1 (legacy R15 gap), clause 9.1.1.3 (Pre-MG) and table 9.1.9.3-1 (NCSG).

  - P2: It is proposed that legacy measurement gaps include all gaps defined till Rel-17 except MUSIM gaps.

  - P3: Legacy MG” in Case 1 includes pre-MG, NCSG and ‘normal MG’ (MG configured via GapConfig, or via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17)

    - Re-use the conclusions from Rel-18 feMG WI on concurrency between MUSIM gaps and pre-MG/NCSG.

  - P4: At the phase 1 the gap collision between MUSIM gaps and Rel-17 legacy gaps excludes the scenarios where a MUSIM gap collides with a Pre-MG or NCSG. Also the agreement option 1 of the issue 2-3-2-2 in [4] needs be updated to exclude NCSG and Pre-MG from Rel-17 legacy gaps

    - The study on the collision between MUSIM gaps and NCSG/Pre-MG are considered after solutions for collision handling between Pre-MG or NCSG with other gaps are available, i.e., reuse solutions from Rel-18 “further Enhancements on NR and MR-DC Measurement Gaps” WI.

  - P5: RAN4 to focus on determining behavior in case of the collision between MUSIM gaps and Network A gaps in the following scenarios

      - Collision between MUSIM gaps and legacy gaps (gaps configured with GapConfig, not GapConfig-r17)

    - RAN4 to consider, in a later stage or later release, the behavior for collision of MUSIM gaps in the following scenarios:

      - Collision between MUSIM gaps and concurrent gaps

      - Collision between MUSIM gaps and pre-configured gaps

      - Collision between MUSIM gaps and NCSG

      - Collision between MUSIM gaps and positioning gaps

Agreements:

    - The scope of Rel-17 legacy gaps includes gaps configured via GapConfig or via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17, and Pre-MG and NCSG.

    - Focus on the collision between MUSIMG gaps and gaps configured via GapConfig or via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17 in the first stage.

    - Investigation on collision between MUSIM gaps and Pre-MG or NCSG will start after the study of Pre-MG/NCSG concurrent with legacy gaps in the Rel-18 feMG WI is stable; related conclusions from Rel-18 feMG WI should be re-checked for the collision handling between MUSIM gaps and pre-MG/NCSG.

Issue 1-1-2: Definition of the collision between MUSIM gaps and legacy gaps

- Proposals:

  - P1: Definition of gap collision and corresponding proximity condition specified under concurrent gaps can be reused for collision between MUSIM gap and legacy gaps

  - P2:

    - Definition and the gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between MUSIM gap with Rel-17 legacy gaps (excluding NCSG and Pre-MG)

    - For Rel-17 NCSG and Pre-MG, the gap proximity condition for the Rel-17 concurrent gap collision should be used as the baseline. Updates on collision definition for Pre-MG/NCSG from Rel-18 further enhancement on measurement gap WI should be reused in this WI. Agreements of issue 2-14 and 2-15 of [R4-2214346] should be adopted, i.e.,

      - (Pre-configured MG and multiple concurrent MGs), the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated.

      - When determining if a collision occurs between a NCSG instance and another gap instance, the baseline requirement considers the total NCSG duration, including both the VILs and the ML.

Agreements:

    - Definition of gap collision and corresponding proximity condition specified under concurrent gaps can be reused for collision between MUSIM gap and gaps configured via GapConfig or via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17.

    - For the collision definition between Pre-MG/NCSG and MUSIM gaps, related conclusions from Rel-18 feMG WI should be re-checked.

Issue 1-1-3: Priority of MUSIM against other legacy gaps

- Proposals:

  - P1: Up to network configuration

    - Up to NW A configuration if priority field is introduced to MUSIM, otherwise use default priority

  - P2: If an explicit priority level is not provided for MUSIM gaps via signalling, MUSIM gaps are assumed to have higher priority than all measurement gaps configured by the network.

  - P3: Aperiodic MUSIM gap is always prioritized over legacy MG in NW A.

  - P4: When MUSIM gaps collide with legacy MG

    - MUSIM paging and AGC occasions should have higher priority than NW-A MG

    - The priority between other MUSIM gaps and legacy MG can be indicated by NW

Agreement (GTW):

- RAN4 agrees on introduction of the priority for MUSIM gaps

Tentative Agreement (GTW):

- Send a LS to RAN2 about the outcome of RAN4 discussion

Tentative agreement:

If priority is introduced to each individual MUSIM gaps, priority of MUSIM gaps other than aperiodic and MUSIM gap for paging purpose are up to network configuration;

FFS on aperiodic MUSIM gap and MUSIM gap for paging purpose

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t support the tentative agreement nor sending an LS to RAN2 at this time. |

Issue 1-1-4: Solutions for collision between MUSIM gap and legacy measurement gap

- Proposals:

  - P1: Priority based solution is reused for gap collision handling between MUSIM gap and legacy gaps.

    - Option 1a: For priority-based solution, priorities can be allocated to each existing gap patterns and when two or more gaps collide, only the highest priority gap is kept and all other gaps are dropped

    - Option 1b: Further optimization can also be considered and it FFS at current stage.

  - P2: On top of priority-based solution, RAN4 shall also study the gap sharing based solution, at least for the scenario equal priority is assigned for different gap patterns.

  - P3: When MUSIM gaps collide with legacy MG, RAN4 to differentiate different usages of the MUSIM gaps, such as L3 measurement for cell reselection, paging monitoring etc;

    - The paging for NW-B cannot be dropped when the paging occasion is colliding with MG in NW-A.

    - The SSB for paging AGC retuning in NW-B cannot be dropped when the SSB occasion is colliding with MG in NW-A if the time distance between the SSB and paging occasion is less than 160ms

    - Whether priority rule or sharing rule will be applied for other MUSIM gaps is FFS

  - P4: RAN4 to study how mobility conditions can be taken into account for the MUSIM gap priorities

Agreements: No

## Sub-topic 1-2 On collision between different MUSIM gaps

Issue 1-2-1: Definition of the collision between different MUSIM gaps

- Proposals:

  - Option 1a: The gap proximity condition of concurrent gap collision could be reused for MUSIM gap collision

  - Option 1b: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps.

  - Option 2: No definition for collision between MUSIM gaps is needed

Agreements: No

Issue 1-2-2: Solutions for collision between different MUSIM gaps

- Proposals:

  - Option 1: Priority rule can be used as baseline for collision between different MUSIMs

    - Option 1a: Aperiodic gap should have higher priority than periodic gaps once collision happens within MUSIM gaps

  - Option 2: MUSIM gaps could be kept when different MUSIM gaps collide

    - Option 2a: MUSIM gaps are not dropped due to collision with another MUSIM gap

    - Option 2b:

      - When the time duration between the two closest gap occasions within the two measurement gap patterns is shorter than [4]ms and the second gap occasion is for paging, UE should keep both gap occasions instead of dropping any of them.

      - RAN4 to further identify the specific scenarios in which any MUSIM gap shall be dropped case by case

    - Option 2c: If multiple MUSIM gap instances overlap or occur back-to-back, they are merged into a single instance comprising the union of the individual gap instances

      - If the distance between two MUSIM gap instances is ≤ 4 ms, they are merged into a single instance comprising the union of the individual gap instances and the space between them

      - If the distance between two MUSIM gap instances is > 4 ms, both individual gap instances are kept separately.

  - Option 3: Solutions for collision between different MUSIM gaps is either down-selected from option 1 or option 2; or based on both option 1 and option 2

Agreements: No

## Sub-topic 1-3 On collision between MUSIM gaps and other signals

Issue 1-3-1: Definition of the collision between MUSIM gaps and other signals

- Proposals:

  - Option 1: A L1/L3 measurement resource is considered to be overlapped with a periodic MUSIM gap if it overlaps a MUSIM gap occasion, a L1/L3 measurement resource is considered to be overlapped with an aperiodic MUSIM gap if it overlaps that aperiodic MUSIM gap occasion

    - Option 1a: Condition “XXX is overlapping with MG” is used for defining MUSIM gap collision with SMTC and L1 measurement resources in NW A.

  - Option 2: RAN4 to use the proximity condition to define the collision between MUSIM gaps with SMTC and L1 measurement resources

Agreements: No

Issue 1-3-2: Priority of MUSIM against SMTC, and other L3/ L1 measurement resources

- Proposals:

  - Option 1a: Collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and measurement gaps

  - Option 1b: MUSIM gaps should have high priority against SMTC and L1 measurement resources

  - Option 1c: UE is in general not expected to transmit or receive signals for NW A (including SMTC and L1 measurement resources) during MUSIM gaps, except for signals used for random access procedure

  - Option 2: RAN4 follows NTN to define the proximity between SMTC/L1 measurement resources with MUSIM gaps

    - Apply priority rule between MUSIM gaps and SMTC/L1 measurement resources for NW-A based on NW-A’s priority indication, or

    - Apply sharing rule between MUSIM gaps and SMTC/L1 measurement resources for NW-A

  - Option 3: RAN4 to discuss how to handle overlap between MUSIM gaps and SMTC in network A for RRC connected procedures like e.g., mobility procedures in Network A

Agreements: No

Issue 1-3-3: Priority of MUSIM against uplink signals, such as PRACH, CSI-RS reporting

- Proposals:

  - P1: The UE is not required to conduct any transmission towards network A, including PRACH, during MUSIM gaps

  - P2: When MUSIM gaps collide with DL RS or UL signals, RAN4 to differentiate different usages of the DL RSs and UL signals in NW-A

  - P3: For the Priority of MUSIM against uplink signals such as PRACH, CSI-RS reporting, suggest to reuse rules defined at 5.14 of TS38.321 (copied below for reference)

  - P4: Reuse the rules for the legacy MGs specified in current specs as listed in P3 as the solution for issue 1-3-3. FFS on other DL/UL signals which are not covered by rules in P3.

Agreements: No

## Sub-topic 1-4 Other aspects on priority for MUSIM gaps

Issue 1-4-1: Priority assignment for MUSIM gaps

- Proposals:

  - P1: Priority of MUSIM gaps, including both periodic and aperiodic gaps, should be up to NW configuration (Apple CMCC Huawei oppo Charter)

  - P2: Whether UE could request priority should be discussed in RAN2 (oppo)

  - P3-a: UE should be allowed to request appropriate priorities for different MUSIM gaps from NW A (Apple Qualcomm Charter xiaomi MTK vivo);

    - Request RAN2 to introduce optional signalling so that the UE can request the priority level of MUSIM gaps ()

  - P3-b: Regarding priority assignment for MUSIM gaps, network A can fulfil this task with the facilitation from UE side when UE requesting MUSIM gaps. A LS should be sent to RAN2 after RAN4’s solution is stable. (Apple vivo xiaomi)

  - P4: Define gap priority for MUSIM gaps that depend on the gap purpose; Network A should be able to configure MUSIM gap priorities for each purpose; RAN4 to study how mobility conditions can be taken into account for the MUSIM gap priorities.  Send LS to RAN2 asking how priority can be specified for MUSIM gaps and legacy gaps. (Apple Nokia)

Agreements: No

Issue 1-4-2: Other Priority aspects

- Proposals:

  - P1: If (assuming) priority rules is used for the collision between MUSIM gaps and legacy MGs of NW A, then MUSIM gaps and normal gaps of NW A should have different priorities

Agreements: No

Issue 1-4-3: Paging issue for MUSIM gap and legacy measurement gap

- Proposals:

  - P1: Both NW-A and UE should have the same understanding on which MUSIM gap is used for paging monitoring. RAN4 to further discuss how to identify this paging gap within MUSIM gaps

  - P2: UE should request an exclusive MUSIM gap for paging instead of monitoring paging in several MUSIM gaps. (Ericsson)

  - P3: FFS

Agreements: No

## Sub-topic 1-5 On network A requirements

Issue 1-5-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

- Proposals:

  - P1: RAN4 can start from outcome of concurrent gap design, i.e. counting Navailable and Ntotal when defining L1 and L3 measurement requirements.

  - P2: The following parameters need to be updated to account for collisions with MUSIM gaps

    - Kp for intra-frequency and inter-frequency measurements without gaps

    - Kgap for intra-frequency and inter-frequency measurements with gaps

    - Kgap_EUTRA for inter-RAT measurements

    - Kp_CSI-RS for CSI-RS L3 measurements

    - Kp,PRS,i for NR positioning measurements

    - CSSFintra for intra-frequency measurements

    - CSSFinter for intra-frequency measurements

    - CSSFinterRAT for intra-RAT measurements

    - P scaling factor for L1-RSRP and L1-SINR measurements

  - P3: Considering MUSIM gap impact on L3 measurements, define Kp and Kgap as follows

- Intra-frequency measurements (without gap):

  - Kp = Ntotal / Navailable, where

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

- W is the largest periodicity among MGs, MUSIM gaps and SMTC.

- Inter-frequency measurements:

  - Kgap = Ntotal / Navailable, where

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- W is the largest periodicity among MGs, MUSIM gaps and SMTC.

Considering MUSIM gap impact on L1 measurements, define P as follows:

  - Ntotal / Noutside_MG in FR1

  - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

  - Ntotal / Navailable in FR2 with Navailable > 0

Where,

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with MGs, MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

- W is the largest periodicity among MGs, MUSIM gaps and SSB periodicity.

  - P4: Reuse principles used in Rel-17 when concurrent gaps are configured to define network A L1/L3 measurement requirements when MUSIM gaps are configured for issue 2-2-3 and 2-2-4, i.e., option 1, a scaling factor should be introduced for network A requirements when MUSIM gaps are configured.

  - P5: The issue can still be discussed parallelly with gap collision handling issue

Agreements: FFS

Issue 1-5-2: On the time window W for aperiodic gap

- Proposals:

  - Option 1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (oppo)

  - Option 2: Take aperiodic gap into account when determining the time window W by adding a time margin. (oppo MTK)

    - Option 2a: W for aperiodic gap can be defined as: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic gap. (MTK)

Agreements: No and postpone the discussion on this issue until when requirements are discussed

## Sub-topic 1-6 On network B requirements

Issue 1-6-1: Whether to define network B requirements

- Proposals:

  - Option 1: Deprioritize NW B requirement in R18

  - Option 2: No measurement requirements in network B will be defined by RAN4

  - Option 3: RAN4 does not define new UE idle/inactive measurement requirements for measurements on Network B for a UE configured with MUSIM gaps (exiting UE measurement requirement for Idle/Inactive mode applies)

  - Option 4: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B

Agreements: No

Issue 1-6-2: Network B requirements if it will be defined

- Proposals:

  - P1: RAN4 has to define some requirement for NW B in R18, RAN4 shall focus on cell reselection

  - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP)

  - P3: UE measurement requirements for idle/inactive mode in network B shall be the same as existing Idle/Inactive measurement requirements.

    - UE measurement requirement for Idle/Inactive mode on network B, when assigned with requested MUSIM gaps, are the same as for network A. Hence, same as legacy requirements

Agreements: Postpone discussion until there is outcome of issue 1-6-1

## Sub-topic 1-7 Other aspects

Issue 1-7-1: MUSIM overhead

- Proposals:

  - Option 1: Use the overhead cap principle on multiple concurrent gaps in Rel-17 as the baseline for MUSIM gaps, and discuss further enhancements considering :

    - Up to 3 periodic MUSIM gaps and one aperiodic MUSIM gap

    - Longer MGRP

  - Option 2: Regarding the overhead cap on all configured gaps for a UE, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR

  - Option 3: RAN4 to define MUSIM gap overhead for MUSIM gap(s)

  - Option 4: RAN4 does not to define overhead cap for MUSIM gaps.

Agreements: No

Issue 1-7-2: Order for applying the priority when number of colliding MGs is larger than 2

- Proposals:

  - P1: RAN4 to discuss the order for applying the priority when number of colliding gaps is larger than 2

  - P2: For issue 2-3-2-4, the order for applying priority rules when multiple gaps are overlapping, investigate one solution by considering the following two cases:

    - 1. Within a particular time window, each gap collides with all other gaps.

    - 2. Within a particular time window, each gap collides with one or few particular gaps and does not collide with one or few particular gaps.

  - P3: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority

Agreements: No

Issue 1-7-3: Total number of gaps when MUSIM gaps are configured

- Proposals:

  - P1: RAN4 to discuss total number of gaps when MUSIM gaps are configured

Agreements: No

Issue 1-7-4: On MUSIM operation conditions

- Proposals:

  - P1: RAN4 needs to define the conditions in which the UE is considered to be in MUSIM operation mode.

Agreements: Do not need consider this issue in this WI.