3GPP TSG-RAN WG4 Meeting # 104-bis-e										 R4-22xxxxx

Electronic Meeting, October 10 – October 19, 2022

Title: 	WF on MRTD/MTTD requirement for multi-TRPs with 2 TAs

Agenda Item:	8.1.1

Source: 	Apple

Document for:	Approval

# Topic #1: MRTD/MTTD requirement for multi-TRPs with 2 TAs

## Sub-topic 1-1: MRTD/MTTD requirement applicability in 7.5 and 7.6 of 38.133

Issue 1-1-1: Applicability of MRTD/MTTD requirements in 7.5 and 7.6 of 38.133

Agreements:

- Existing MTTD requirements in 7.5 and 7.6 of 38.133 only define the timing difference limitation between different CCs for CA or DC operations, not define the timing difference limitation between different TRPs on the same CC for MIMO case.

Issue 1-1-2: whether clarifications are needed for the following note in 7.6.1 of 38.133

Way forward:

- Companies have different interpretation of existing applicability of MRTD/MTTD requirements. RAN4 will further discuss this issue in RAN4#105 under TEI since it is a legacy issue.

## Sub-topic 1-2: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR1

Issue 1-2: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR1

Agreements:

- For both intra-cell and inter-cell multi-TRP, the maximum uplink transmit timing difference between multiple TRPs can be assumed within a CP length as baseline.

- For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 33/34.6 µs.

## Sub-topic 1-3: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR2

Issue 1-3: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR2

Agreements:

- For both intra-cell and inter-cell multi-TRP, the maximum uplink transmit timing difference between multiple TRPs can be assumed within a CP length as baseline.

- For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 8/8.5 µs.

## Sub-topic 1-4: TAE for multi-TRP

Issue 1-4: TAE for multi-TRP

Agreements:

- Do not change the TAE requirement due to the LS.