3GPP TSG-RAN WG4 Meeting # 104-bis-e										 R4-22xxxxx

Electronic Meeting, October 10 – October 19, 2022

Title: 	WF on MRTD/MTTD requirement for multi-TRPs with 2 TAs

Agenda Item:	8.1.1

Source: 	Apple

Document for:	Approval

# Topic #1: MRTD/MTTD requirement for multi-TRPs with 2 TAs

## Sub-topic 1-1: MRTD/MTTD requirement applicability in 7.5 and 7.6 of 38.133

Issue 1-1-1: Applicability of MTTD requirements in 7.5 and 7.6 of 38.133

Agreements:

- Existing MTTD requirements 7.6 of 38.133 only define the timing difference limitation between different CCs for CA or DC operations.

Issue 1-1-2: whether clarifications are needed for the following note in 7.6.1 of 38.133

Way forward:

- Companies have different interpretation of existing applicability of MRTD/MTTD requirements.

## Sub-topic 1-2: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR1

Issue 1-2: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR1

Agreements:

- For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP+M1 µs for FR1. Where M1 is FFS.

  - M1 jointly accounts for UE Tx timing error and timing advance uncertainty.

  - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered

- For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 33/34.6 µs.

## Sub-topic 1-3: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR2

Issue 1-3: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR2

Agreements:

- For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP + M2 µs for FR2. Where M2 is FFS.

  - M2 jointly accounts for UE Tx timing error and timing advance uncertainty.

  - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered

- For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 8/8.5 µs.

## Sub-topic 1-4: TAE for multi-TRP

Issue 1-4: TAE for multi-TRP

Agreements:

- Do not change the TAE requirement due to the LS.

- MRTD/MTTD for multi-TRP with RTD>CP is based on the current inter-band CA TAE requirement