3GPP TSG-RAN WG4 Meeting # 106	R4-2300192

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	9.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.877: DPD in FR2 multiband BS

Document for:	Approval

# 1.	Introduction

The revised SID on NR BS RF requirement evolution was approved at TSG RAN#97-e [1]. The objective of this study item is to study certain aspects for FR2 multi-band BS, including the following aspects relating to FR1 multi-band BS:

    - Investigate the feasibility and performance of wideband RF and antenna architectures covering multiple FR2 bands

This topic was discussed at TSG RAN4#104-e, RAN4#104bis-e and RAN4#105 and the latest WF on the feasibility and performance of wideband RF and antenna architectures of FR2-1 multi-band BS was agreed [2]. This contribution provides a text proposal into TR 38.877 [3] to capture on the aspect regarding to the DPD in FR2-1 multi-band BS according to the agreements in [2].

# 2.	Text proposal

<Start of change>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[3]	3GPP R4-2212622: “Multi-band BS in mm wave”, Ericsson

[4]	Rusek, Fredrik, et al. "Scaling up MIMO: Opportunities and challenges with very large arrays." IEEE signal processing magazine 30.1 (2012): 40-60.

[5]	Fager, Christian, et al. "Linearity and efficiency in 5G transmitters: New techniques for analyzing efficiency, linearity, and linearization in a 5G active antenna transmitter context." IEEE Microwave Magazine 20.5 (2019): 35-49.

[6]	Tervo, Nuutti, et al. "Analyzing the effects of PA variations on the performance of phased array digital predistortion." 2018 IEEE 29th Annual International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC). IEEE, 2018.

[7]	Jalili, Feridoon, et al. "Linearization trade-offs in a 5G mmWave active phased array OTA setup." Ieee Access 8 (2020): 110669-110677.

[8]	Khan, Bilal, et al. "Statistical Digital Predistortion of 5G Millimeter-Wave RF Beamforming Transmitter Under Random Amplitude Variations." IEEE Transactions on Microwave Theory and Techniques 70.9 (2022): 4284-4296.

[9]	Ng, Eric, et al. "Digital predistortion of millimeter-wave RF beamforming arrays using low number of steering angle-dependent coefficient sets." IEEE Transactions on Microwave Theory and Techniques 67.11 (2019): 4479-4492.

[10]	Larsson, Erik G., and Liesbet Van der Perre. "Out-of-band radiation from antenna arrays clarified." IEEE Wireless Communications Letters 7.4 (2018): 610-613.

[11]	Haider, Muhammad Furqan, et al. "Predistortion-Based Linearization for 5G and Beyond Millimeter-Wave Transceiver Systems: A Comprehensive Survey." IEEE Communications Surveys & Tutorials (2022).

[12]	ADI, Technical article, “Why Millimeter Wave Requires a Different Approach to DPD and How to Quantify Its Value”

[13]	Abdelaziz, Mahmoud, et al. "Digital predistortion for hybrid MIMO transmitters." IEEE Journal of Selected Topics in Signal Processing 12.3 (2018): 445-454.

[14]	Wu, Qian, et al. "Digital Predistortion for Concurrent Dual-Band Millimeter Wave Analog Multibeam Transmitters." IEEE Transactions on Circuits and Systems II: Express Briefs 69.3 (2021): 1747-1751.

[15]	Li, Yue, Xiaoyu Wang, and Anding Zhu. "Sampling rate reduction for digital predistortion of broadband RF power amplifiers." IEEE Transactions on Microwave Theory and Techniques 68.3 (2019): 1054-1064.

[16]	Yu, Chao, et al. "Linear-decomposition digital predistortion of power amplifiers for 5G ultrabroadband applications." IEEE Transactions on Microwave Theory and Techniques 68.7 (2020): 2833-2844.

[17]	3GPP R4-2219154: “Further discussion on phase shifters”, Huawei, HiSilicon.

<Next change>

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

ADC	Analog-Digital Converter

BB	Base Band

BS	Base Station

BW	Bandwidth

DPD	Digital Pre-Distortion

EIRP	Effective Isotropic Radiated Power

EVM	Error Vector Magnitude

FBW	Fractional Bandwidth

FR	Frequency Range

IM	Inter-Modulation

LNA	Low Noise Amplifier

MCS	Modulation and Coding Scheme

NR	New Radio

OBUE	Operating Band Unwanted Emissions

OOB	Out-of-band

OTA	Over-The-Air

PA	Power amplifier

RB	Resource Block

RDN	Radio Distribution Network

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RX	Receiver

SCS	Sub-Carrier Spacing

TX	Transmitter

TRP	Total Radiated Power

ZF	Zero Forcing

<Next change>

# 5	Feasibility study

## 5.1	General

## 5.2	Wideband RF architectures

## 5.2.2	Digital Pre-distortion

For single-band FR2 radios, the application of DPD may not be critical compared to that of the FR1 counterpart. On one hand, the contribution of PAs to the total DC power consumption in a FR2 BS is much reduced. This is due to the fact that more antenna elements are added to increase the directivity of the antenna array to combat against the high path loss occurred at FR2, which in turn requires smaller power to feed each antenna element, and thus small-power PA per Tx is sufficient. On the other hand, the requirement on ACLR for FR2 BSs has been much relaxed, i.e., 24 - 28dBc (by means of OTA measurement), compared to that applied to FR1 BSs, i.e., 45dBc [2]. This somewhat alleviates the essential need of high-linearity PAs on meeting required ACLR. Therefore, the DPD in single-band FR2 BSs is expected to provide a little gain in terms of improving power efficiency and meeting the ACLR requirement. Furthermore, the analog and hybrid beamforming, which are predominantly used in FR2 radios, also pose challenges for DPD implementation. With hundreds to thousands of PAs and higher operating bandwidths for FR2 radio, simply utilizing a similar DPD architecture as in FR1 would cost extra for RF hardware design and power consumption, while it may be infeasible to deploy single DPD for every PA in the analog/hybrid beamforming phased array since several or all analog chains essentially share one digital path. The DPD algorithms would also have more demands on the bandwidth of feedback receiver/ADC and BB signal processing resources, which are scaled with the size of bandwidth to be linearized [3]. These limited-gain and implementation-challenge factors make the DPD implementation less attractive to the FR2 single-band BS.

Nevertheless, DPD may still bring benefit for FR2 BSs in terms boosting the overall system performance (e.g., throughput). It is worth highlighting that common FR2 transmitter architecture requires to have tight integration between RF components to reduce hardware costs, sizes, and power loss in which isolator between an antenna element and a PA is preferably avoided [5], e.g., as illustrated in Figure 5.5.2-1. In such architecture, PAs directly interact with the antenna array due to low path isolation between them. As a result, mutual coupling and antenna mismatch between antenna paths have strong impact to the PAs’ output matching impedance which changes the PA’s efficiency and nonlinearity behaviors [5]. The array steering angle, which also alters the antenna matching impedance, shows strong dependence on the nonlinearity to the PAs too. In addition, input of PAs in different branches may be driven with different power as a result of the beamforming techniques applied (i.e., tapering, ZF, etc.), or gain error of the phase shifters and gain imbalance of power division network [6]. These mentioned factors have the detrimental effect to efficiency and linearity behaviors of PAs which may degrade the ACLR while increasing the OOB emission and beam distortions [5]. For example, several studies have demonstrated the impact of steering angle to ACLR and OOB emission and how DPD can help to improve the beamforming performance [5, 6, 7, 8, 9].

Figure 5.2.2-1: A typical single-band FR2 antenna array architecture

Compared to FR2 single band BS, FR2 multi-band BS equipped with multi-band RIBs could have a stronger need for the DPD. As abovementioned, antenna array and TRXs are desired to be tightly integrated in which RF filter is preferably omitted after the PA. Since a multi-band RIB essentially needs to transmit multiple-band signals concurrently, the nonlinearity of PAs will likely cause intermodulation (IM) distortion. It would be highly challenging to manage the PAs in multiband RIB not to operate in the nonlinearity power region. Particularly, the varying nonlinearity behaviors of the PAs, which causes by the nonlinear interaction between antenna array and the PAs as discussed above, also inherit to the FR2 multi-band RIB. Such issues would be expected to be more complicated in the multiband use cases than in single-band ones due to higher requirements on matching load impedance of PAs covering multiband/wideband. Therefore, unwanted emissions due to IM distortions likely exist and may possibly fall into the operating bands or inter-RF bandwidth. Note that the latter case occurs if there is multicarrier transmission in one band. Figures 5.5.2-2 and 5.5.2-3 illustrate some examples. Assume that operation bandwidths of Band A and B are 24.25-26.5GHz (n258) and 27-29.5 GHz (n257), and there is transmission taking place at 26GHz in Band A and 27.5GHz in band B. Then IM3 components occur at 24.5GHz and 29GHz, which obviously fall into operating bandwidth of both bands as seen in Figure 5.5.2-2. Now assume that band A transmits two carrier frequencies at 24.75 and 25.75GHz. Then one IM3 component occurs at 26.75GHz which falls into the inter-RF bandwidth of the multiband BS as seen in Figure 5.5.2-3.

Figure 5.2.2-2: Example for possible unwanted emission due to concurrent multiband transmission

Figure 5.2.2-3: Example for possible unwanted emission due to multi-carrier transmission in one band

The IM distortion is also beamformed [10]. In some case, the IM distortions could be seen strong at the slightly different direction or same direction with the intended UEs, e.g., when UEs are nearby each other. Thus, it needs to be managed to ensure that RF requirements are still be met for the FR2 multiband RIB. One may want to have a RF filter/diplexer after every PAs. However, this solution would be very expensive since beamforming phased array in FR2 could have thousands of antenna paths. Filter per path may also generate significantly phase error between antenna paths and increase power loss. Thus, this may not be feasible in terms of cost, size, and performance of FR2 radios.

Alternatively, DPD may be a cheaper solution for RF hardware architectures to this issue. For FR2 multi-band DPD, it essentially inherits the abovementioned advantages as well as challenges for single-band FR2 BS. Investigating practically feasible DPD solutions for beamforming phased array in FR2 has been an active research topic [11]. To address the concerns on the DPD implementation in FR2, DPD architectures and efficient DPD training model/algorithms have been intensively studies. Note that since one digital path will be shared between some or all analog chains in the analog/hybrid beamforming phased array, an architecture in which DPD linearization is applied for a set of PAs has been proposed and then demonstrated to be able to improve ACLR and EVM, e.g., [12, 13]. For dual-band DPD, one can either deploy a common DPD for all bands or a dedicated DPD for each band [14]. The former may have less demand on DPD architecture but requires much higher bandwidth for feedback receiver/ADC as DPD captured, training and correcting samples are wideband; since the correction is wide-band, band-specific linearization may not be achievable as such. The latter has lower demand on DPD hardware bandwidth and can achieve per-band linearization but may require more complex DPD architectures and algorithms, i.e., to decompose the captured multiband signals to single-band one and vice versa for the corrections, as well as jointly optimize linearized coefficients for all bands. On the other hand, DPD training algorithms which minimize the demand on DPD hardware bandwidth and BB resources have also been proposed [15, 16]. It should be highlighted that the implementation of phase shifters will not significantly impact the DPD solutions as linearization are applied to signals seen at the output of the PA, as mentioned in [17].

<End of change>

# References

[1]	RP-222501, “SID revision: NR BS RF requirement evolution”, Huawei, HiSilicon.

[2]	R4-2220249, “WF for the feasibility study on FR2 multi-band RIB”, Huawei, HiSilicon.

[3]	R4-2219148, “TR 38.877 v0.1.0”, Huawei, HiSilicon.