3GPP TSG-RAN WG4 Meeting #106	R4-2300200

# Athens, Greece, February 27 – March 3, 2023

Source:			Nokia, Nokia Shanghai Bell

Title:			TP to TR 38.844 on Comparison Between Different Schemes

Agenda item:	9.1.3

Document for:	Approval

# Introduction

RAN#96 encouraged RAN4 to focus the discussion about this SI in August 2022 on SIB1 signalling issues. RAN#97e agreed to continue to sort out all open issues before completing the study of all proposed solutions and SI completion is postponed to March 2023 [1]. RAN#98e concluded to follow the current SI schedule [2], although RAN4#105 did not reach consensus on SIB1 signalling of CBW configuration and channel raster issues [3].

As there has been neither a common understanding on the legacy UE behaviors nor on possible modifications (or additions) in Rel-18 regarding the above contentious issues, we have proposed a way-forward such that each company provides its understanding by filling in the questionnaire [4]. Once answers to the questionnaire are provided in RAN4#106, we can have a common ground to identify what clarifications are needed in legacy releases and which methods can be feasible in a future 3GPP release.

Since RAN4#106 is the last WG meeting before the currently scheduled completion of the SI, we propose the TR revision to reflect the discussion we had about the above open issues. The text proposal can be further updated during the RAN4#106 meeting depending on the consensus on the legacy UE behaviors and the possible modifications in a future 3GPP release.

In this contribution, we propose to revise the comparison table of TR 38.844 Section 7 according to our companion papers [5,6].

# Discussion

We have discussed some issues regarding the existing descriptions found in TR 38.844 v0.0.9 and provide text proposals to TR Section 6 [5,6] to revise the outdated contents. The comparison table in TR 38.844 Section 7 needs to be revised accordingly as proposed in this contribution. The text proposal is provided in the following.

# Reference

[1] RP-222618 Revised SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths, RAN#97e

[2] RP-223452 Moderator's summary of discussion [98e-12-RAN4-Irregular-BWs], RAN#98e

[3] RP-223273 Status report for SI Study on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths; rapporteur: Ericsson, RAN#98e

[4] R4-2220502 WF on irregular channel bandwidth, Nokia, RAN4#105

[5] R4-2300201 TP to TR 38.844 on Larger Channel BW than licensed BW, Nokia, RAN4#106

[6] R4-2300202 TP to TR 38.844 on Overlapping UE CBWs from Network Perspective, Nokia, RAN4#106

# Text Proposal to 38.844 v0.0.9

<Start of Changes>

Table 7.2.1. Comparison of different schemes

| Comparison Criteria | Overlapping CA (two cells) – described in Section 6.2.1 | Combined UE CBW (One cell) – described in Section 6.2.2 | Overlapping UE CBW – described in Section 6.2.3 | Wider CBW (one cell) – described in Section 6.2.X |
| --- | --- | --- | --- | --- |
| Regulatory requirement | No issue | No issue | No issue | Potential issue on the BS side, gNB filters will be needed depending on BS implementation. The UL carrierBandwidth in SIB1 plus guard bands must fit into the licensed spectrum. |
| UE performance degradation relative to minimum requirements | Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. | Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. | No issue | UE ACS and in-band blocking degradation unless the current limits are extended to irregular BWs. Possible impact on Rx sensitivity vs regular CBW, if single carrier Tx-Rx separation is not maintained. |
| gNB complexity | gNB has to support CA and schedule the data without collision in the two CC’s overlapping PRBs | gNB has to support splitting the signal into 2 RF carriers with a predefined phase relationship | gNB has to support the irregular channel BW (can also be implemented through RF combining of 2 channels) | No changes needed if the BS can meet regulatory requirements with the RF front end of the wider channel BW. Otherwise, gNB has to support the irregular channel BW |
| UE complexity | UE has to support intra-band NC CA. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | UE has to support RF architecture as in intra-band NC CA. Needs new capability to aggregate 2 RF channels in baseband. Complexity higher than CA because the baseband will need a new "combiner" module. 1 less CC can be supported when irregular BW is used in combination from other bands, or more total CCs needed. | No changes needed, supported by legacy UEs | New UEs are required to support asymmetric BW, UE specific channel BWs not on the 100 kHz raster and larger than the resource grid indicated in SIB1, as well as new combinations of guard band size and transmission BW configuration. |
| UE throughput | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UEs supporting the feature can use the entire spectrum allocation, legacy UEs can use an already supported channel BW | UE throughput based on existing channel BWs (5MHz for holdings <10MHz, 10MHz for holdings <15MHz, etc) | UE throughput based on how many RBs can be used |
| Spectral utilization | Channel edge guardband based on the aggregated channel BW (5MHz for <10MHz, 10MHz for <15MHz, etc), 2 SSBs are needed | Channel edge guardband based on the aggregated RF carrier BW (5MHz for <10MHz, 10MHz for <15MHz, etc), single SSB needed | Channel edge guardband based on the actual holding (can be same as Overlapping CA), 2 SSBs are needed in most cases | Depends on the usable number of RBs, single SSB needed |
| Cell Spectral utilization | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding can be used even only with legacy UEs for some scenarios depending on whether a single SSB can be used to configure legacy channels at both edges of the spectrum holding. Otherwise, entire spectrum can be used only by new UEs, all legacy UEs have to use the same regular channel BW part of the spectrum holding. | Entire spectrum holding can be used even only with legacy UEs | Entire spectrum holding, but with wider guard bands than in the other methods |
| Network capacity | Entire spectrum can be used by multiplexing different UEs(even legacy UEs) | Entire spectrum can only be used for new UEs, whether legacy UEs can be multiplexed to cover entire channel depends on the configuration and bandwidth | Entire spectrum can be used by multiplexing different UEs in the frequency domain | Entire spectrum can be used by new UE |
| Legacy UE support | Legacy UEs supported, can use one of the aggregated CCs | Legacy UEs can use part of the spectrum that contains the SSB | Legacy UEs supported | Legacy UEs supported with the next smaller channel bandwidth |
| RAN1/2/4 Specification impact | RAN1/2 – new UE capabilities needed, RAN4 – new band combinations, changes to channel spacing definition, Overlapping CA reqs applicability, new demod requirements for UEs | RAN2 – impact on new capability RAN4 –core requirements equivalent to new channel BW for BS, new demod requirements for UEs | RAN4 – BS requirements for new channel BW | Transmission bandwidth configurations and minimum guard band sizes for the irregular BWs RAN4 – BS requirements for new channel BW, possibly restrictions of the suitable scenarios. Extension of today's UE RX requirements to the irregular BWs New asymmetric bandwidth combinations for UE are needed. However these combinations would be “regular” BW combinations, so existing process. Extension of the resource grid signalled in SIB1 to support a larger UE specific channel BW UE specific CBWs that need not be on the 100 kHz raster |

<End of Changes>