3GPP TSG-RAN4 Meeting #106	R4-2300258

# Athens, Greece, February 27 – March 3, 2023

Agenda item:	6.2.4

Source:	Apple

Title:   	Draft LS to RAN1 on L3-RSSI measurement for NR up to 71GHz

Document for:	Approval

# Introduction

RAN4 received the LS from RAN1 LS on L3-RSSI measurement for NR up to 71GHz [1]:

In RAN1 #110, RAN4 sent an LS response with clarification of the following:

“When a UE has no serving cell in FR2-2, it is not clear if the explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI?”

In RAN1 #110b-e, RAN1 responded in R1-2201590 to the above question as follows

- When a UE has no serving cell in FR2-2, the UE does not expect that a TCI-state is provided in RMTC-Config for inter-frequency RSSI measurement on FR2-2.

- For a UE that has no serving cell in FR2-2 and configured with inter-frequency RSSI measurement in FR2-2, it is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

In RAN1 #111, RAN1 further agreed the following

- For intra-frequency and inter-frequency RSSI measurements for FR2-2, when a UE has a serving cell in FR2-2, the UE does not expect to be configured with an explicit TCI-state in RMTC-Config with a reference serving cell in FR1 or FR2-1.

To TSG RAN4 and RAN2

ACTION: 	RAN1 respectfully asks RAN4 and RAN2 to consider the above agreement and capture it in RAN4 and RAN2 spec.

In this contribution, we propose how to capture the RAN1 agreement in 38.133 and provide a draft reply LS in the Annex. Note that a formal CR to 38.133 is provided in a companion contribution.

# Discussion

For intra-freq. case, we can capture the RAN1 agreement to the existing specification text as shown below (highlighted in yellow)

“For performing intra-frequency RSSI measurement in FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. UE does not expect to be configured with an explicit TCI-state in RMTC-Config with a reference serving cell in FR1 or FR2-1. If no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier in FR2-2.”

For inter-freq. case, we can capture the RAN1 agreement to the existing specification text as shown below (highlighted in yellow):

“For performing inter-frequency RSSI measurement in FR2-2,

-	If a UE has serving cell in FR2-2,

-	If the TCI state is provided in RMTC configuration of FR2-2, the UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. The UE does not expect to be configured with an explicit TCI-state in RMTC-Config with a reference serving cell in FR1 or FR2-1. If no TCI state is provided in the RMTC configuration, the UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of a serving carrier in FR2-2.

-	If a UE has no serving cell in FR2-2,

-	The UE is not expected that a TCI state is provided in RMTC configuration. It is up to UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2, and no requirements are defined.”

Proposal 1: It is proposed to capture the RAN1 agreement with the above highlighted text in 38.133.

Proposal 2: RAN4 send the draft LS as provided in Annex.

# Conclusion

In this contribution, we make the following proposals.

Proposal 1: It is proposed to capture the RAN1 agreement with the above highlighted text in 38.133.

Proposal 2: RAN4 send the draft LS as provided in Annex.

# References

- R1-2212830, “LS to RAN4 and RAN2 on L3-RSSI measurement for NR up to 71GHz,” RAN1

Annex:

Title:	 LS reply on L3-RSSI measurement for NR up to 71GHz

Response to:	R1-2212830

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name:

E-mail:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN1 on the LS and agree to capture the RAN1 agreement in TS 38.133 as in the attached CR.

2. Actions:

To RAN1:

ACTION: 	No action is expected.

3. Dates of next TSG-RAN WG4 meetings:

RAN4#106-bis-e	17 – 26 Apr. 2023			e-Meeting

RAN4#107			22 – 26 May 2023			Incheon, Korea