3GPP TSG-RAN WG4 Meeting # 106	R4-2300275

# Athens, Greece, February 27 – March 3, 2023

Agenda item:	5.2.5.2

Source:	Apple

Title:	On PTW window limitation for RRM relaxation

Document for:	Discussion

# Introduction

In RAN4 #105 meeting, the WF[1] for RedCap RRM requirements was agreed, but there are still some open issues as followings,

Issue 1-4-1: Mismatch between PTW length when measurement relaxation is configured and the maximum PTW length defined by RAN2 - Proposals - Observation: a mismatch between the values of PTW length in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 and RAN2 agreements on maximum PTW length (Nokia) - WF - Discuss this issue at the next meeting

In this contribution, we discuss the above remaining and propose the revision to the spec.

# Discussion

It was agreed in their RAN2 #115e meeting that (R2-2109301),

However, in RAN4 requirement for RRM relaxation, it was specified that:

The lowest bound of PTW length is derived based on the cell evaluation delay, and the evaluation delay is defined as 2.56xN1xK3, where the N1 is the beam sweeping factor and K3 is the measurement relaxation factor for UE fulfilling the stationaryMobilityEvaluation criterion.

We think the evaluation shall be still contained in the same PTW window since the PHY samples for cross PTW window can be separated by eDRX cycle>=20.48s. Regarding the beam sweeping factor for DRX =1.28s and 2.56s, it was already reduced to 4 and 3 to consider the mobility performance, but it still has small room to further reduce. Another possible way in our view is to reduce the K3 factor to fit the PTW window length limitation, since eDRX mode already provide UE much power saving gain and it shall be fine to reduce RRM relaxation factor.

The rules to determine the PTW window for DRX=1.28s and 2.56s cases are:

- The lowest bound of PTW length is derived based on the cell evaluation delay

- The PTW length cannot be greater than 40.96s

- The N1 and K3 factors can be further adjusted

- The PTW length shall be not in descending order when DRX cycle increases

The new evaluation delay requirement with adjusted K3 and N1 can be calculated as followings:

| DRX=1.28s | N1=4 | N1=3 | N1=2 |
| --- | --- | --- | --- |
| K3=4 | Evaluation delay= 2.56*N1*K3=40.96s | 30.72s (cannot use, since it’s smaller than DRX =0.64s case) | 20.48s(cannot use, since it’s smaller than DRX =0.64s case) |
| K3=3 | 30.72s(cannot use, since it’s smaller than DRX =0.64s case) |  |  |
| K3=2 | 20.48s(cannot use, since it’s smaller than DRX =0.64s case) |  |  |

| DRX=2.56s | N1=3 | N1=2 |
| --- | --- | --- |
| K3=4 | Evaluation delay= 5.12*N1*K3=61.44s (cannot use, since it’s greater than 40.96) | 40.96s |
| K3=3 | 46.08s (greater than 40.96) | 30.72s(cannot use, since it’s smaller than DRX =0.64s case) |
| K3=2 | 30.72s(cannot use, since it’s smaller than DRX =0.64s case) |  |

Based on the above calculation,

- for DRX=1.28s, the K3 shall be reduced to 4 and N1=4, the PTW length can be revised to  40.96s

- for DRX=2.56s, the K3 shall be reduced to 4 and N1 reduced to 2, the PTW length can be revised to  40.96s.

Proposal:

For IDLE intra-frequency and inter-frequency relaxed measurement requirement with eDRX in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 of TS38.133, following parameters and requirement shall be applied:

- Regarding DRX=1.28s, K3=4 and N1=4, and the PTW length shall be revised to  40.96s

- Regarding DRX=2.56s, K3=4 and N1=2, and the PTW length shall be revised to  40.96s

A corresponding CR[2] is proposed in this meeting as well.

# Conclusion

In this contribution, we discuss the above remaining and propose the revision to the spec.

Proposal:

For IDLE intra-frequency and inter-frequency relaxed measurement requirement with eDRX in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 of TS38.133, following parameters and requirement shall be applied:

- Regarding DRX=1.28s, K3=4 and N1=4, and the PTW length shall be revised to  40.96s

- Regarding DRX=2.56s, K3=4 and N1=2, and the PTW length shall be revised to  40.96s

A corresponding CR[2] is proposed in this meeting as well.

# References

[1] R4-2220714, WF on Rel-17 NR RedCap RRM Core requirement maintenance, vivo, RAN4 #105.

[2] R4-2300276, CR on Rel-17 NR RedCap measurement requirement in IDLE mode with eDRX, Apple, RAN4 #106.