3GPP RAN WG4 Meeting #106		R4-2300367

Athens, Greece, February 27th – March 3rd, 2023

Title:	LS on signaling for FR2 FBG5 CA BW classes

Response to:

Release:	Release 17

Work Item:	NR_RF_FR2_req_enh2-Core

Source:	Apple

To:	RAN2

Cc:

Contact Person:

- Name:	James Wang

- E-mail Address:	 fucheng_wang@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

None

1. Overall Description:

The newly introduced FR2 CA BW classes in FBG5 though with the merits of providing the most flexibility in combining the 100MHz and 200MHz cells as well as having the prescience for future aggregated BW extension up to 2400 MHz, it also has a potential issue when UE’s maximum aggregated BW is less than the CA BW class upper BW limit where UE’s full capability in the fallback BW classes can only be indicated by the rather complicated FeatureSet signalling, and that would result in substantial signalling overhead.

To mitigate the potential signaling overhead issue for FR2 FBG5 CA BW classes, RAN4 has agreed to repurpose the existing IE defined for intra-band non-contiguous CA frequency separation classes to also be applicable to indicate UE’s maximum aggregated BW capability for intra-band contiguous CA which would be independent from UE’s per CC BW capability. By reusing the frequency separation classes to indicate UE’s maximum aggregated BW capability for contiguous CA, there can be two potential signalling solutions to avoid the complicated FeatureSet signalling for FBG5 CA BW classes (using R12 in FBG5 and maximum aggregated BW = 1600 MHz as an example) which can be further down-selected to one.

Alternative 1: 	The following parameters are signalled,

CA BW Class: R12

intraBandFreqSeparationDL = V (1600 MHz)

Note: The default assumption is that the intraBandFreqSeparationDL is also supported by the UE for all the lower order CA BW classes (R11, R10, R9, R8) where their aggregated BW upper limit is higher than or equal to intraBandFreqSeparationDL.

Alternative 2: 	The following parameters are signalled,

CA BW Class: R12

intraBandFreqSeparationDL = V (1600 MHz)

CA BW Class: U

Note: From the above signalling, network recognizes the UE can support up to 12 CCs and at least 8 out of 12 CCs can support 200 MHz.

2. Actions:

To: 3GPP TSG RAN WG2

RAN4 respectfully asks RAN2 to take RAN4’s agreement above into consideration for FR2 FBG5 CA BW classes signaling.

3. Date of Next TSG-RAN WG4 Meetings:

- 3GPP RAN4 #106bis-e 					April 17th – 26th, 2023		        		   Online

- 3GPP RAN4 #107							May 22nd – 26th, 2023					   South Korea