3GPP TSG-RAN WG4 Meeting#106      	R4-2300426

Athens Greece, 27 Feb – 3 Mar, 2023

Title:	Intra-band non-collocated EN-DC/NR-CA type 4 UE

Agenda item:	8.11.2

Source:	Nokia

Document for:	Approval

# 1	Background

WI [1] objectives are

- Phase I:

  - Study the feasibility to support non-co-located scenarios for both FR1 inter-band EN-DC with overlapping and FR1 intra-band NR-CA, except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17.

    - Investigate the tolerable power imbalance between carriers

    - Investigate the required arrival time difference between CCs

    - Investigate the additional impacts of contiguous case, if time units are available.

    - Evaluate the UE performance under the power imbalance and arrival time difference

      - Discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA

        - NOTE 1: For UE capable of supporting 2-layer MIMO, only assuming power imbalance 25dB

        - NOTE 2: For UE capable of supporting 4-layer MIMO, 2-layer MIMO power imbalance assumption can be considered as a base line but another value or the same power imbalance with different throughput performance requirement is not precluded.

        - NOTE 3: RAN4 is recommended to start the work on 2-layer first and after that start 4-layer work based on the conclusion of 2-layer work.

    - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78

    - Investigate whether the power imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the power imbalance based on the outcome of the investigation.

    - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify MRTD and MTTD requirements in non-collocated deployment

  - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

In RAN4#104-bis meeting under [104-bis-e][134] WF[2] was approved where it was decided that handheld UE type3a/3b is to be prioritised.

- Prioritize Type 3a/3b discussion for 4MIMO layer in Rel-18.

4MIMO layer UE architecture choices are the ones listed as 4Rx in Table 1. From [3] we can see some rough UE architectures but mapping to table one is difficult.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

Table 1: UE type details

# 2 	Discussion

In our opinion even though Type 3a/3b is prioritized in discussion after that discussion is concluded or parallel if discussion is not converging RAN4 should define requirements for FWA UE type 4a/4b.

This is important as FWA volumes are all the time increasing globally. For example, according to GSA [4]

“5G FWA customer premises equipment shipments more than doubled to 3.6 million in 2021 – a year-on-year increase of over 162%. FWA CPE shipments are forecast to double again to 7.6 million in 2022”

Some of the FWA devices currently produced [5] support  LTE or NR SA and NSA with 4x4 DL MIMO. There is a need for FWA UE type 4a/4b in future. Therefore, we propose

Proposal 1: RAN4 will also develop requirements for UE type 4a/4b.

From requirement perspective UE type 4a/4b is easier that type 3a/3b as all signal paths have dedicated antenna and LNA thus UE type 2 power imbalance requirements can be re-used.

Proposal 2: UE type 4a/4b power imbalance requirements are same as UE type 2 i.e. 25 dB over full dynamic range.

Proposal 3: WI is not considered to be completed before the requirements for UE type 4a/4b are finalized.

# 3	Conclusion

In this contribution we have done two proposals

Proposal 1: RAN4 will also develop requirements for UE type 4a/4b.

Proposal 2: UE type 4a/4b power imbalance requirements are same as UE type 2 i.e. 25 dB over full dynamic range.

Proposal 3: WI is not considered to be completed before the requirements for UE type 4a/4b are finalized.

# 4	References

[1] R4-2218192, Intra-band non-collocated EN-DC/NR-CA type 4 UE

[2] R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, RAN4#104bis, KDDI

[3] R4-2215736, Views on UE RF aspect for non-collocated EN-DC, NR-CA deployment, Samsung

[4] GSA Fixed Wireless Access vendor survey reveals 5G FWA CPE shipments forecast to more than double 2022 - GSA (gsacom.com)

[5] https://www.nokia.com/fastmile/