3GPP TSG-RAN4 Meeting #106	R4-2300510

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2838 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR correction of SDT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarification ont eh description of the test cases for SDT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification ont eh definitions for the test points |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear test cases for SDT with missing information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2, A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

## A.6.2	SA: RRC_INACTIVE state mobility

### A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

#### A.6.2.1.1	Test purpose and Environment

The purpose of this test is to partly verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. In the test, UE is configured two CG-SDT configurations configured when (re-)entering RRC Inactive state. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 repectively. There is one cell, which is the active NR cell in FR1.  Figure A.6.2.1.1-1 shows the variation of the RSRP over the duration of the entire test, both sub-tests included.

Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1), be registered to the cell and have entered RRC connected mode.

Before starting the test at time point TA, test equipment configures RSRP to P0. P0 must be chosen to be above the cg-SDT-RSRP-ThresholdSSB.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, UE expect to receive a test mode command with T_timer_modeB defined to expire just after a CG-SDT resource at time point TF and an RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P2. TE must be W2 before TF. W2 is the RSRP2 window defined by TS 38.133, section 5.5.3. W1 is defined as half the RSRP1 window.

P2 is defined to be within the maximum threshold allowed compared to P1.

TF, configured by the test equipment at time point TC, shall be aligned to be just after a CG-SDT resource. TF is defined as the timeout time, T_delay_modeB, as given in the test mode command of time point TC.

The timeout triggers UL data arrival at UE lower layer at time point TF.

The repetition period of CG-SDT resources is set at maximum value, 640ms. Therefore, TG will be close to 640ms after TF.

At time point TH, UE expect to receive a test mode command with timer T_delay_modeB defined to expire just after a CG-SDT resource at time point TJ and an RRC release with CG-SDT configurations. TJ must be just after a CG-SDT resource.

At time point TI, RSRP is changed from P2 to P3. TI is T4 after TE, which is TE+W2+640ms+W1+delay between TG and TH, which is not defined.

When the timeout value given at point TH expires, the UL data arrival at UE lower layer at time point TJ.

Figure A.6.2.1.1-1: RSRP variation model for CG-SDT testing

#### A.6.2.1.2	Test Parameters

There is one cells in the test, the FR1 PCell. The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz |  | 10: NRB,c = 52 |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR1 |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |
| DRX configuration |  | 1,2,3 | DRX.8 |  |
| T1 | s | 1,2,3 | [1] |  |
| T2 | s | 1,2,3 | [1.28] |  |
| T3 | s | 1,2,3 | [2.36<=T3<=3.36] |  |
| T4 | s | 1,2,3 | [>W2+640ms+W1] |  |
| T5 | s | 1,2,3 | [>timeout value + 2] |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 | [8] |  |
| cg-SDT-RSRP-ThresholdSSB | dBm | 1,2,3 | P0 – 1dB |  |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 | infinity |  |
| CG-SDT resource period | ms |  | 640 | configuredGrantConfig Peridocity at 38.331 Config 1,2 : sym640x14, config 3 : sym1024x14 |
| Propagation condition |  | 1,2,3 | AWGN |  |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
|  | 1,2,3 | dBm/15kHz | -100 |  |  |  |  |
|  | 1,2 | dBm/SSB SCS | -100 |  |  |  |  |
|  | 3 | dBm/SSB SCS | -103 |  |  |  |  |
|  | 1,2,3 | dB | [0] | [10.35] | [0] | [15.25] | [6] |
| SS RSRP | 1,2,3 | dBm/SSB SCS | [-100] | [-89.65] | [-100] | [-84.75] | [-96] |
| Io | 1,2 | dBm/9.36MHz | [-69.04] | [-61.31] | [-69.04] | [-56.67] | [-65.07] |
|  | 3 | dBm/38.16MHz | [-69.04] | [-61.31] | [-69.04] | [-56.67] | [-65.07] |
|  | 1,2,3 | dB | [0] | [10.35] | [0] | [15.25] | [6] |

#### A.6.2.1.3	Test requirements

The UE behaviour in each test during time durations shall be as follows:

During T4, UE shall transmit PUSCH at CG-SDT resource within 640ms + Z after time point TF.

During T5, UE shall not transmit PUSCH at CG-SDT resources after TJ until the end of the test at time point TK.

The rate of correct events observed during repeated tests shall be at least 90%.

### <End of Change 1>

### <Start of Change 2>

## A.7.2	SA: RRC_INACTIVE state mobility

### A.7.2.1	Small Data Transmission

#### A.7.2.1.1	TA validation for CG-SDT in FR2

##### A.7.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly performs TA validation for CG-SDT. This test will partly verify the TA validation requirements in clause 5.5.3.

Supported test configurations are shown in table A.7.2.1.1.1-1. The test parameters are given in Tables A.7.2.1.1.1-2 and A.7.2.1.1.1-3.

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of 5 successive time periods, with time duration of  T1 to T5 respectively. Figure A.7.2.1.1.1-1 shows the variation of the RSRP level in the active cell during the test.

Time duration T1 starts at time point TA and ends at time point TB. During T1 test equipment sets the transmit power to P0.

Time duration T2 starts at time point TB and ends at time point TD. At time point TC which is W1 after time point TB, test equipment sends RRCRelease message with CG-SDT configuration to UE and UE enters RRC_INACTIVE. During T2 test equipment sets the transmit power to P1.

Time duration T3 starts at time point TD and ends at time point TE. During T3 test equipment sets the transmit power to P0.

Time duration T4 starts at time point TE and ends at time point TI. Test equipment triggers UL data arrival at UE lower layer at time point TF which is W2 after time point TE, and the details to configure UL data arrival is left to RAN5. After time point TF, test equipment observes whether UE transmits with CG-SDT within 640ms + Z after TF. The time point when UE transmits with CG-SDT is denoted as TG. At time point TH (FFS how TH is determined), the second RRCRelease is sent to UE. During T4 test equipment sets the transmit power to P2.

Time duration T5 starts at time point TJ and ends at time point FFS. During T5 test equipment sets the transmit power to P3. Test equipment triggers UL data arrival at UE lower layer at time point TJ, and the details to configure UL data arrival is left to RAN5.

Table A.7.2.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.2.1.1.1-2: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 DD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 kHz |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  | s | 1.28 |
| cg-SDT-RSRP-ThresholdSSB |  | dB | [TBD] |
| cg-SDT-RSRP-ChangeThreshold |  | dB | [8] |
| cg-SDT-TimeAlignmentTime |  |  | infinity |
| CG-SDT resource period |  | ms | 640 |
| T1 |  | s | [TBD] |
| T2 |  | s | [TBD] |
| T3 |  | s | [TBD] |
| T4 |  | s | [TBD] |
| T5 |  | s | [TBD] |

Table A.7.2.1.1.1-3: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
|  | Config 1 | dBm/15kHz | -100 |  |  |  |  |
|  | Config 1 | dBm/SCS | -100 |  |  |  |  |
|  | Config 1 | dB | [0] | [10.35] | [0] | [15.25] | [6] |
|  | Config 1 | dB | [0] | [10.35] | [0] | [15.25] | [6] |
| SS-RSRP | Config 1 | dBm/SCS | [-100] | [-89.65] | [-100] | [-84.75] | [-96] |
| Io | Config 1 | dBm/95.04 MHz | [-69.04] | [-61.31] | [-69.04] | [-56.67] | [-65.07] |
| Propagation condition |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

Figure A.7.2.1.1.1-1: RSRP variation for TA validation for CG-SDT

##### A.7.2.1.1.2	Test Requirements

The UE behaviour in each test during time durations shall be as follows:

During T4, UE shall transmit UL data with CG-SDT within 640ms + Z after time point TF.

During T5, UE shall not transmit UL data with CG-SDT.

The rate of correct events observed during repeated tests shall be at least 90%.

### <End of Change 2>