3GPP TSG-RAN WG4 Meeting#106                	R4-2300613

Athens, Greece, February 27 – March 3, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for 8Rx UE demodulation and CSI requirements

Agenda item: 	9.6.6.1

Document for: 	Discussion

Introduction

During RAN4#105 meeting, WF on 8 RX and CSI requirements is agreed [1].

In this contribution, we provide our initial simulation results for 8 Rx UE demodulation requirements.

PDSCH simulation results

Test parameters

We provide in this section the initial simulation results for PDSCH based on simulation assumption showed in Table 1.

Table 1: Simulation assumption for PDSCH

| Simulation parameters |  |
| --- | --- |
| Test metric | 70% and 30% of maximum TP |
| Duplex Mode | TDD (pattern: 7D1S2U with S:6D+4G+4U) |
| MIMO layers | Rank2, Rank 4, Rank 6, Rank 8 |
| Propagation channels | TDLA30-10 for Rank >2 |
|  | TDLA 30-10 and TDC300-100 for Rank =2 |
| Antenna configuration | - Rank 2: 2T8R |
|  | - Rank 4: 4T8R |
|  | - Rank 6 and Rank 8: 8T8R |
| MCS | MCS4 MCS13 MCS17 MCS23 (MCS table 4) |
| SCS/Bandwidth | 30kHz/20MHz |
| DMRS config | - Rank 6 and Rank 8: DMRS Type 1 with 2+2 with symbol# 2,3,10,11 - Rank<=4: DMRS Type 1 with 1+1 with symbol# 2,10 |
| Start symbol and Length for PDSCH | 14 symbols and S=2, L=12 |

Simulation results: SNR at 30% and 70% throughput

The Table 2 gives the SNR at 30% and 70% throughput using the simulation parameters presented in the previous section.

Table 2: SNR at 30% and 70 for different number of layers and MCS (MCS table 4).

| Rank | MCS | Channel | SNR (dB) at 30% throughput | SNR(dB) at 70% throughput |
| --- | --- | --- | --- | --- |
| Rank2 | MCS4 | TDLA30-10 | 1,22 | 2,18 |
| Rank2 | MCS13 | TDLA30-10 | 12,50 | 13,55 |
| Rank2 | MCS17 | TDLA30-10 | 15,91 | 16,95 |
| Rank2 | MCS19 | TDLA30-10 | 18,15 | 19,28 |
| Rank2 | MCS23 | TDLA30-10 | 23,49 | 24,70 |
| Rank2 | MCS4 | TDLA300-100 | 1,93 | 2,49 |
| Rank2 | MCS13 | TDLA300-100 | 13,69 | 14,33 |
| Rank2 | MCS17 | TDLA300-100 | 17,09 | 17,82 |
| Rank2 | MCS19 | TDLA300-100 | 19,83 | 20,63 |
| Rank2 | MCS23 | TDLA300-100 | 26,61 | 28,30 |
| Rank4 | MCS4 | TDLA30-10 | 6,20 | 6,93 |
| Rank4 | MCS13 | TDLA30-10 | 18,45 | 19,42 |
| Rank4 | MCS17 | TDLA30-10 | 21,69 | 22,66 |
| Rank4 | MCS19 | TDLA30-10 | 24,26 | 25,39 |
| Rank4 | MCS23 | TDLA30-10 | 29,93 | 31,21 |
| Rank6 | MCS4 | TDLA30-10 | 8,87 | 10,06 |
| Rank6 | MCS13 | TDLA30-10 | 22,60 | 24,45 |
| Rank6 | MCS17 | TDLA30-10 | 25,68 | 27,37 |
| Rank6 | MCS19 | TDLA30-10 | 28,41 | 30,18 |
| Rank6 | MCS23 | TDLA30-10 | 35,52 | 39,53 |
| Rank8 | MCS4 | TDLA30-10 | 13,07 | 14,36 |
| Rank8 | MCS13 | TDLA30-10 | - | - |
| Rank8 | MCS17 | TDLA30-10 | - | - |
| Rank8 | MCS19 | TDLA30-10 | - | - |
| Rank8 | MCS23 | TDLA30-10 | - | - |

The Figure 1and Figure 2 simulate the throughput in case of Rank 8 for MCS4 and MCS13 respectively. The two figures show that Rank 8 is not testable with high MCS with high modulation order. Our discussion in paper includes a recommendation accordingly.

Figure 1: Throughput of Rank 8 for TDD (30K), MCS (4), Type I DMRS (2+2), TDLA30-10

Figure 2: Throughput of Rank 8 for TDD (30K), MCS (13), Type I DMRS (2+2), TDLA30-10

Conclusion

In this contribution we provided the results of our initial simulation campaign on 8 Rx for different number of layers and MCS to define UE demodulation performance requirements.

References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2300612, “Discussion on 8Rx UE demodulation and CSI requirements”, RAN WG4 #106, Athens, Feb. 2023