3GPP TSG-RAN WG4 Meeting # 106

R4-2200704

Athens, Greece, February 27 – March 3, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Absolute physical layer throughput simulation results

Agenda item: 	9.18.2

Document for: 	Discussion

Introduction

This paper presents Nokia’s simulation results for Rel. 18’s Application Throughput Layer requirement definition. As stated in [1], RAN4 has agreed to keep the same scope of the ATP SI captured in clause 5.10.3 of TR37.901-5 [2], where the following three tests are considered:

- Test 1: FR1 FDD, SCS/CBW=15kHz/10MHz, 2Tx, 2Rx/4Rx - Test 2: FR1 TDD, SCS/CBW=30kHz/40MHz, 2Tx, 2Rx/4Rx - Test 3: FR2-1 TDD, SCS/CBW=120kHz/100MHz, 2Tx, 2Rx

The test parameters and simulation assumptions were discussed in RAN4#105, and the following assumptions were agreed upon and collected in [1]:

RAN4 use the same assumption in the following aspects as in the Rel-17 SI on feasibility of introducing requirements for physical layer TP requirements with link adaptation for application layer throughput captured in clause 5.10.3 in TR37.901-5: - Maximum rank and CSI-RS port number i.e., maximum rank 2 with 2 CSI-RS ports Note) It’s not precluded to further discuss the possibility of extension the study with rank 4 and number of CSI-RS ports to 4 or 8 in future release. - “Disabling OLLA” for physical layer TP requirements as a baseline - Interested companies can bring further analysis on the OLLA impact - Reuse channel models in SI phase - FR1: Table 5.10.3-1 in TR 37.901-5, i.e., TDLA30-5 - FR2: Table 5.10.3-1 in TR 37.901-5, i.e., TDLA30-35 Note) For FR1, other options such as high-doppler not precluded pending on further evaluation - Maximum number of HARQ transmission: - ‘Set the maximum number of HARQ transmission to 1’ is a baseline assumption - Reuse the same test parameters as defined in Table 5.10.3-1 in TR 37.901-5 including (not precluding parameters in 1.2 above) - MMSE-IRC receiver type - CQI Table and Codebook related configurations - Configure CQI table 2 for FR1 and CQI table 1 for FR2 - Configure Type I Single-Panel Codebook - Not configure CodebookSubsetRestriction - CSI delay - 6 ms for FR1 FDD, 5.5 ms for FR1 TDD, 1.375 ms for FR2-1 TDD

Table 1 shows the simulation assumptions used in the Rel.17 study item (SI) with one correction to the used MCS table for FR2 which was incorrect in the study (see also above agreements from RAN4#105).

Table 1 Simulation Assumptions used in Rel.17 SI

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

Simulation Results

The following sections present Nokia’s simulation results under the agreed simulation assumptions detailed above.  Each simulation test is characterized by its absolute physical layer throughput, the overall block error rate (BLER), median rank indicator (RI) and median channel quality indicator (CQI). Additional simulation results are included to show the impact of enabling Outer Loop Link Adaptation (OLLA) and HARQ re-transmission (re-Tx), described as three separate configuration scenarios. These follow the same simulation assumptions as the legacy SI configuration from Rel. 17 described above except for the parameters summarized in Table 2.

Table 2 - Simulation configuration scenarios

| Configuration scenarios | OLLA | Number of HARQ transmissions |
| --- | --- | --- |
| Legacy SI | disabled | 1 |
| OLLA enabled | enabled | 1 |
| HARQ re-TX enabled | disabled | 4 |
| HARQ re-TX with OLLA enabled | enabled | 4 |

Legacy physical layer throughput results

These results follow the legacy configuration from the SI (no OLLA, no HARQ reTx).

Table 1: ATP Link Adaptation results for Test 1 – FR1 FDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 4.12 | 0.10 | 1 | 3 | 6.40 | 0.04 | 1 | 4 |
| 2 | 5.65 | 0.12 | 1 | 4 | 8.38 | 0.05 | 1 | 6 |
| 4 | 7.39 | 0.12 | 1 | 5 | 10.44 | 0.15 | 2 | 4 |
| 6 | 9.27 | 0.04 | 1 | 6 | 14.20 | 0.02 | 2 | 5 |
| 8 | 11.25 | 0.04 | 1 | 7 | 17.81 | 0.03 | 2 | 6 |
| 10 | 13.36 | 0.04 | 1 | 8 | 21.46 | 0.04 | 2 | 7 |
| 12 | 15.41 | 0.04 | 1 | 9 | 25.64 | 0.03 | 2 | 8 |
| 14 | 16.98 | 0.02 | 1 | 10 | 29.95 | 0.02 | 2 | 9 |
| 16 | 19.79 | 0.09 | 2 | 6 | 32.52 | 0.06 | 2 | 10 |
| 18 | 23.06 | 0.09 | 2 | 8 | 36.38 | 0.04 | 2 | 10 |
| 20 | 26.00 | 0.04 | 2 | 8 | 40.71 | 0.06 | 2 | 12 |

Table 2: ATP Link Adaptation results for Test 2 – FR1 TDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 13.08 | 0.09 | 1 | 3 | 20.77 | 0.03 | 1 | 4 |
| 2 | 17.97 | 0.11 | 1 | 4 | 27.26 | 0.04 | 1 | 6 |
| 4 | 23.34 | 0.12 | 1 | 5 | 34.93 | 0.12 | 2 | 4 |
| 6 | 29.40 | 0.10 | 1 | 6 | 45.77 | 0.02 | 2 | 5 |
| 8 | 35.97 | 0.08 | 1 | 7 | 57.27 | 0.03 | 2 | 6 |
| 10 | 42.49 | 0.08 | 1 | 8 | 69.78 | 0.03 | 2 | 7 |
| 12 | 48.81 | 0.04 | 1 | 9 | 83.05 | 0.06 | 2 | 8 |
| 14 | 54.27 | 0.06 | 2 | 5 | 91.92 | 0.10 | 2 | 9 |
| 16 | 64.53 | 0.07 | 2 | 6 | 103.36 | 0.01 | 2 | 9 |
| 18 | 76.12 | 0.07 | 2 | 8 | 116.28 | 0.03 | 2 | 10 |
| 20 | 85.54 | 0.08 | 2 | 8 | 127.97 | 0.07 | 2 | 12 |

Table 3: ATP Link Adaptation results for Test 3 – FR2-1 TDD

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 20.16 | 0.07 | 1 | 5 |
| 2 | 26.73 | 0.15 | 1 | 7 |
| 4 | 35.17 | 0.09 | 1 | 8 |
| 6 | 43.10 | 0.11 | 1 | 9 |
| 8 | 52.12 | 0.04 | 1 | 10 |
| 10 | 63.27 | 0.10 | 1 | 11 |
| 12 | 75.43 | 0.07 | 1 | 12 |
| 14 | 87.00 | 0.06 | 1 | 13 |
| 16 | 101.67 | 0.04 | 2 | 9 |

The throughput results for the legacy SI configuration are graphically represented in Figure 1. We see good alignment with previously provided company results from the Rel.17 SI [2].

Figure  1 –Throughput (Mbps) for legacy SI configuration.

OLLA-enabled physical layer throughput results

This section details the impact of enabling OLLA for physical layer throughput requirements. The results are obtained for a proposed OLLA model applied on CQI with numerical values for Stepup and Stepdown of 0.45 [CQI units] and 0.05 [CQI units], respectively for a target BLER of 10%. Refer to our contribution [3] for further details on the proposed OLLA modelling and configuration.

Table 1: ATP Link Adaptation results for Test 1 – FR1 FDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.05 | 0.10 | 1 | 3 | 5.18 | 0.10 | 1 | 5 |
| 2 | 4.47 | 0.10 | 1 | 4 | 7.13 | 0.10 | 1 | 6 |
| 4 | 6.24 | 0.10 | 1 | 5 | 8.92 | 0.10 | 1 | 7 |
| 6 | 8.01 | 0.10 | 1 | 6 | 11.82 | 0.10 | 2 | 5 |
| 8 | 9.84 | 0.10 | 1 | 7 | 15.43 | 0.10 | 2 | 6 |
| 10 | 11.73 | 0.10 | 1 | 8 | 18.85 | 0.10 | 2 | 7 |
| 12 | 13.65 | 0.10 | 1 | 9 | 22.39 | 0.10 | 2 | 8 |
| 14 | 15.54 | 0.10 | 1 | 11 | 26.15 | 0.10 | 2 | 9 |
| 16 | 17.55 | 0.10 | 1 | 12 | 29.90 | 0.10 | 2 | 10 |
| 18 | 20.37 | 0.10 | 2 | 8 | 33.57 | 0.10 | 2 | 11 |
| 20 | 23.49 | 0.10 | 2 | 9 | 37.88 | 0.10 | 2 | 12 |

Table 2: ATP Link Adaptation results for Test 2 – FR1 TDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 7.17 | 0.10 | 1 | 3 | 18.40 | 0.10 | 2 | 2 |
| 2 | 10.75 | 0.10 | 1 | 4 | 25.06 | 0.10 | 2 | 3 |
| 4 | 15.75 | 0.10 | 1 | 5 | 34.53 | 0.10 | 2 | 4 |
| 6 | 21.07 | 0.10 | 1 | 6 | 45.17 | 0.10 | 2 | 4 |
| 8 | 26.77 | 0.10 | 1 | 7 | 55.86 | 0.10 | 2 | 5 |
| 10 | 32.49 | 0.10 | 1 | 8 | 67.33 | 0.10 | 2 | 7 |
| 12 | 38.34 | 0.10 | 1 | 9 | 79.94 | 0.10 | 2 | 8 |
| 14 | 44.14 | 0.10 | 1 | 10 | 92.38 | 0.10 | 2 | 9 |
| 16 | 50.48 | 0.10 | 1 | 12 | 104.27 | 0.10 | 2 | 9 |
| 18 | 59.96 | 0.11 | 2 | 8 | 115.04 | 0.10 | 2 | 11 |
| 20 | 69.45 | 0.10 | 2 | 9 | 127.51 | 0.10 | 2 | 12 |

Table 3: ATP Link Adaptation results for Test 3 – FR2-1 TDD

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 14.40 | 0.10 | 1 | 6 |
| 2 | 19.45 | 0.10 | 1 | 7 |
| 4 | 26.28 | 0.10 | 1 | 8 |
| 6 | 33.27 | 0.10 | 1 | 9 |
| 8 | 41.51 | 0.10 | 1 | 10 |
| 10 | 50.99 | 0.10 | 1 | 11 |
| 12 | 60.57 | 0.10 | 1 | 12 |
| 14 | 71.32 | 0.10 | 1 | 13 |
| 16 | 83.32 | 0.10 | 2 | 10 |

Figure 2 – Throughput (Mbps) for proposed OLLA configuration.

Impact of OLLA configuration

Two additional OLLA configurations (Config2 and Config3) have been studied as less aggressive Stepup / Stepdown alternatives to the one proposed in [3]. Results indicate that, for the tests considered in these studies, a less aggressive configuration leads to a throughput improvement, as exemplified in Figure 1 for FR2-1, most likely due to the use of slow fading channels for testing (TDLA30-5 in FR1and TDLA30-35 in FR2-1), which makes it easier for the solution to converge.

| OLLA configurations | Stepup [CQI units] | Stepdown [CQI units] |
| --- | --- | --- |
| OLLA Conf1 | 0.45 | 0.050 |
| OLLA Conf2 | 0.25 | 0.028 |
| OLLA Conf3 | 0.10 | 0.011 |

Figure 3  -  Throughput (Mbps) for different OLLA configurations.

Less aggressive OLLA configurations with smaller Stepup / Stepdown limits on CQI appear to significantly improve the throughput performance in slow fading channels without mobility or changes in the environment.

HARQ re-Tx-enabled physical layer throughput results

This set of results takes the maximum number of HARQ transmission to be 4 with the redundancy-version-sequence="0 2 3 1".

Table 1: ATP Link Adaptation results for Test 1 – FR1 FDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.81 | 0.03 | 1 | 3 | 6.52 | 0.03 | 1 | 4 |
| 2 | 5.72 | 0.06 | 1 | 4 | 8.55 | 0.04 | 1 | 6 |
| 4 | 7.66 | 0.08 | 1 | 5 | 11.66 | 0.06 | 2 | 4 |
| 6 | 9.63 | 0.07 | 1 | 6 | 14.75 | 0.10 | 2 | 5 |
| 8 | 11.65 | 0.07 | 1 | 7 | 18.23 | 0.09 | 2 | 6 |
| 10 | 13.61 | 0.07 | 1 | 8 | 22.14 | 0.06 | 2 | 7 |
| 12 | 15.85 | 0.08 | 1 | 10 | 25.96 | 0.02 | 2 | 8 |
| 14 | 18.34 | 0.11 | 2 | 6 | 29.72 | 0.02 | 2 | 9 |
| 16 | 21.05 | 0.11 | 2 | 7 | 33.66 | 0.04 | 2 | 10 |
| 18 | 24.00 | 0.06 | 2 | 8 | 37.39 | 0.03 | 2 | 10 |
| 20 | 27.23 | 0.07 | 2 | 8 | 41.68 | 0.04 | 2 | 12 |

Table 2: ATP Link Adaptation results for Test 2 – FR1 TDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 11.70 | 0.03 | 1 | 3 | 21.08 | 0.02 | 1 | 4 |
| 2 | 17.89 | 0.06 | 1 | 4 | 27.77 | 0.03 | 1 | 6 |
| 4 | 24.24 | 0.08 | 1 | 5 | 37.75 | 0.04 | 2 | 4 |
| 6 | 30.66 | 0.07 | 1 | 6 | 47.90 | 0.08 | 2 | 5 |
| 8 | 37.09 | 0.06 | 1 | 7 | 59.45 | 0.08 | 2 | 6 |
| 10 | 43.48 | 0.06 | 1 | 8 | 72.00 | 0.04 | 2 | 7 |
| 12 | 50.70 | 0.08 | 1 | 9 | 84.51 | 0.03 | 2 | 8 |
| 14 | 58.91 | 0.11 | 2 | 6 | 96.45 | 0.06 | 2 | 9 |
| 16 | 67.85 | 0.11 | 2 | 7 | 108.54 | 0.02 | 2 | 10 |
| 18 | 77.73 | 0.05 | 2 | 8 | 119.98 | 0.02 | 2 | 10 |
| 20 | 88.50 | 0.05 | 2 | 8 | 132.26 | 0.04 | 2 | 12 |

Table 3: ATP Link Adaptation results for Test 3 – FR2-1 TDD

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 21.77 | 0.15 | 1 | 5 |
| 2 | 28.87 | 0.14 | 1 | 7 |
| 4 | 37.11 | 0.13 | 1 | 8 |
| 6 | 45.79 | 0.12 | 1 | 9 |
| 8 | 55.30 | 0.11 | 1 | 10 |
| 10 | 65.92 | 0.07 | 1 | 11 |
| 12 | 77.56 | 0.05 | 1 | 12 |
| 14 | 90.84 | 0.11 | 1 | 13 |
| 16 | 107.18 | 0.10 | 2 | 9 |

Figure 4 - Throughput (Mbps) for HARQ configuration with 4 HARQ-reTx.

OLLA + HARQ re-Tx

Table 1: ATP Link Adaptation results for Test 1 – FR1 FDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.65 | 0.10 | 1 | 3 | 5.84 | 0.10 | 1 | 5 |
| 2 | 5.12 | 0.10 | 1 | 4 | 7.78 | 0.10 | 1 | 6 |
| 4 | 6.87 | 0.10 | 1 | 5 | 9.82 | 0.10 | 2 | 4 |
| 6 | 8.68 | 0.10 | 1 | 6 | 13.23 | 0.10 | 2 | 5 |
| 8 | 10.59 | 0.10 | 1 | 7 | 16.69 | 0.10 | 2 | 6 |
| 10 | 12.54 | 0.10 | 1 | 8 | 20.22 | 0.10 | 2 | 7 |
| 12 | 14.55 | 0.10 | 1 | 9 | 23.92 | 0.10 | 2 | 8 |
| 14 | 16.39 | 0.10 | 1 | 10 | 27.71 | 0.10 | 2 | 9 |
| 16 | 19.08 | 0.10 | 2 | 7 | 31.58 | 0.10 | 2 | 10 |
| 18 | 22.24 | 0.10 | 2 | 8 | 35.12 | 0.10 | 2 | 11 |
| 20 | 25.38 | 0.10 | 2 | 9 | 39.58 | 0.10 | 2 | 12 |

Table 2: ATP Link Adaptation results for Test 2 – FR1 TDD

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 8.96 | 0.10 | 1 | 3 | 15.48 | 0.10 | 1 | 5 |
| 2 | 12.64 | 0.10 | 1 | 4 | 21.51 | 0.10 | 1 | 6 |
| 4 | 17.90 | 0.10 | 1 | 5 | 27.44 | 0.10 | 1 | 7 |
| 6 | 23.21 | 0.10 | 1 | 6 | 35.87 | 0.10 | 2 | 5 |
| 8 | 29.40 | 0.10 | 1 | 7 | 46.90 | 0.10 | 2 | 6 |
| 10 | 35.49 | 0.10 | 1 | 8 | 58.85 | 0.10 | 2 | 7 |
| 12 | 41.42 | 0.10 | 1 | 9 | 70.33 | 0.10 | 2 | 8 |
| 14 | 47.33 | 0.10 | 1 | 10 | 81.56 | 0.10 | 2 | 9 |
| 16 | 55.11 | 0.10 | 2 | 7 | 92.28 | 0.10 | 2 | 10 |
| 18 | 65.38 | 0.10 | 2 | 8 | 103.75 | 0.10 | 2 | 11 |
| 20 | 75.95 | 0.10 | 2 | 9 | 117.07 | 0.10 | 2 | 12 |

Table 3: ATP Link Adaptation results for Test 3 – FR2-1 TDD

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 16.70 | 0.10 | 1 | 6 |
| 2 | 22.74 | 0.10 | 1 | 7 |
| 4 | 30.23 | 0.10 | 1 | 8 |
| 6 | 37.92 | 0.10 | 1 | 9 |
| 8 | 47.08 | 0.10 | 1 | 10 |
| 10 | 56.72 | 0.10 | 1 | 11 |
| 12 | 66.99 | 0.10 | 1 | 12 |
| 14 | 77.89 | 0.10 | 2 | 9 |
| 16 | 94.11 | 0.10 | 2 | 10 |

Figure 5 - Throughput (Mbps) for both OLLA (Conf1) and HARQ re-Tx enabled.

Conclusion

This paper has presented Nokia’s simulation results based on the agreed configuration from the discussion of feasibility to define requirements for Application Layer Throughput with link adaptation.

The legacy results show a good alignment with company results provided in the Rel.17 SI. In addition, we have included simulation results for OLLA and HARQ reTx.

- Less aggressive OLLA configurations with smaller Stepup / Stepdown limits on CQI appear to significantly improve the throughput performance in slow fading channels without mobility or changes in the environment.

References

- R4-2220276 - “WF on physical layer throughput requirements”, Intel, RAN4 #105, November 2022.

- 3GPP TR 37.901-5 “Study on 5G NR User Equipment (UE) application layer data throughput performance (Release 16)”, V16.8.0.

- R4-2300703 - “On Absolute physical layer throughput requirements with link adaptation”, Nokia