3GPP TSG-RAN WG4 Meeting #106	R4- 2300717

Athens, Greece, 27th Feb – 3rd Mar, 2023

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for band n28

Agenda Item:	8.23.2

Document for:	Approval

# 1	Introduction

This paper presents the PC2 A-MPR simulation results for band n28 as per the WF in [1].  Accordingly, the A-MPR requirements to be specified are proposed.

# 2	Discussion

### 2.1 Additional Spectrum Emission Requirements for Band n28

The additional spectrum emission requirements specified in TS 38.101-1 are duplicated below.

##### 6.5.3.3.2	Requirement for network signalling value "NS_17"

When "NS_17" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-1: Additional requirements for "NS_17"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10 |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |

##### 6.5.3.3.3	Requirement for network signalling value "NS_18"

When "NS_18" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3. 3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3-1: Additional requirements for "NS_18"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20, 30 |  |  |
| 692-698 | -26.2 | 6 MHz |  |

Observation 1: BW=25MHz is missing from the requirements for NS_18 in Table 6.5.3.3-1 of TS 38.101-1.

Observation 2: In Clause 5.3.5 UE channel bandwidth per operating band, NOTE 7 of Table 5.3.5-1 applies for CBW=20/25/30, which is duplicated below.

For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 25 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 715.5-720.5 MHz or 730.5-735.5 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz.

### 2.2 A-MPR Simulation Results

The simulation results are obtained for NS_18 for PC2 1Tx, which are shown in Table 1 below.

Table 1: A-MPR simulation results for NS_18 with 1Tx

| BW | PC2 A-MPR |
| --- | --- |
| 5MHz |  |
| 20MHz |  |
| 30MHz |  |

### 2.1 A-MPR for Band n28

The existing PC3 A-MPR requirements in TS 38.101-1 are duplicated below.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.13-1, A2 |
|  |  |  | 25, 30 |  | Table 6.2.3.13-1, A3, A4, A5 |

#### 6.2.3.13	A-MPR for NS_18

Table 6.2.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |

Table 6.2.3.13-1: A-MPR for NS_18

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Based on the simulation results for BW=5MHz, it’s proposed to modify A1 for PC2 as follows:

Table 2: NS_18 PC2 A-MPR for BW=5MHz

| Modulation/Waveform |  | A1 (dB) |
| --- | --- | --- |
|  |  | Outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [2+1] |
|  | QPSK | ≤ [2+1] |
|  | 16 QAM | ≤ [3+1] |
|  | 64 QAM | ≤ [4+0.5] |
|  | 256 QAM | ≤ [6] |
| CP-OFDM | QPSK | ≤ [5] |
|  | 16 QAM | ≤ [5] |
|  | 64 QAM | ≤ [5.5] |
|  | 256 QAM | ≤ [8.5] |

Based on the simulation results for BW=20MHz, it’s proposed to modify A2 for PC2 as follows:

Table 3: NS_18 PC2 A-MPR for BW=10, 15, 20MHz

| Modulation/Waveform |  | A2 (dB) |
| --- | --- | --- |
|  |  | Inner/Outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [5+3] |
|  | QPSK | ≤ [5+3] |
|  | 16 QAM | ≤ [6+3] |
|  | 64 QAM | ≤ [7+3] |
|  | 256 QAM | ≤ [9+2] |
| CP-OFDM | QPSK | ≤ [6.5+3] |
|  | 16 QAM | ≤ [7+3] |
|  | 64 QAM | ≤ [8.5+3] |
|  | 256 QAM | ≤ [11.5] |

Based on the simulation results for BW=30MHz, it’s proposed to modify A3/A4/A5 as follows:

Table 4: NS_18 PC2 A-MPR for BW=25, 30MHz

| Modulation/Waveform |  | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | [3+0.5] | [8+2] | [3+1] |
|  | QPSK | [3+0.5] | [8+2] | [3+1] |
|  | 16 QAM | [3+0.5] | [8+3] | [3+1] |
|  | 64 QAM | [3+0.5] | [8+3] | [4.5+0.5] |
|  | 256 QAM | [3+0.5] | [8+3] | [5.5] |
| CP-OFDM | QPSK | [4.5] | [9.5+2] | [5+1] |
|  | 16 QAM | [4.5] | [9.5+2] | [5+1] |
|  | 64 QAM | [4.5] | [9.5+2] | [5.5] |
|  | 256 QAM | [4.5] | [9.5+2] | [7.5] |

The figure 1 below is an illustration of the A-MPR regions for A3/A4/A5.

A3

A4

A5

RB start

LCRB

Figure 1: Illustration of the A-MPR regions for A3/A4/A5.

# 3	Conclusion

In this paper, we have presented our initial simulation results for NS_18 for PC2 1Tx. Based on the existing PC3 A-MPR requirements, the following modifications for PC2 1Tx are proposed:

Proposal 1: For NS_18 PC2 1Tx BW=5MHz, modify the A-MPR values for A1 as follows:

Table 2: NS_18 PC2 A-MPR for BW=5MHz

| Modulation/Waveform |  | A1 (dB) |
| --- | --- | --- |
|  |  | Outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [2+1] |
|  | QPSK | ≤ [2+1] |
|  | 16 QAM | ≤ [3+1] |
|  | 64 QAM | ≤ [4+0.5] |
|  | 256 QAM | ≤ [6] |
| CP-OFDM | QPSK | ≤ [5] |
|  | 16 QAM | ≤ [5] |
|  | 64 QAM | ≤ [5.5] |
|  | 256 QAM | ≤ [8.5] |

Proposal 2: For NS_18 PC2 1Tx BW=10/15/20MHz, modify the A-MPR values for A2 as follows:

Table 3: NS_18 PC2 A-MPR for BW=10, 15, 20MHz

| Modulation/Waveform |  | A2 (dB) |
| --- | --- | --- |
|  |  | Inner/Outer |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [5+3] |
|  | QPSK | ≤ [5+3] |
|  | 16 QAM | ≤ [6+3] |
|  | 64 QAM | ≤ [7+3] |
|  | 256 QAM | ≤ [9+2] |
| CP-OFDM | QPSK | ≤ [6.5+3] |
|  | 16 QAM | ≤ [7+3] |
|  | 64 QAM | ≤ [8.5+3] |
|  | 256 QAM | ≤ [11.5] |

Proposal 3: For NS_18 PC2 1Tx BW=25/30MHz, modify the A-MPR values for A3/A4/A5 as follows:

Table 4: NS_18 PC2 A-MPR for BW=25, 30MHz

| Modulation/Waveform |  | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | [3+0.5] | [8+2] | [3+1] |
|  | QPSK | [3+0.5] | [8+2] | [3+1] |
|  | 16 QAM | [3+0.5] | [8+3] | [3+1] |
|  | 64 QAM | [3+0.5] | [8+3] | [4.5+0.5] |
|  | 256 QAM | [3+0.5] | [8+3] | [5.5] |
| CP-OFDM | QPSK | [4.5] | [9.5+2] | [5+1] |
|  | 16 QAM | [4.5] | [9.5+2] | [5+1] |
|  | 64 QAM | [4.5] | [9.5+2] | [5.5] |
|  | 256 QAM | [4.5] | [9.5+2] | [7.5] |

Based on observation 1, it’s also proposed to add the missing CBW=25MHz to NS_18.

Proposal 4: Add BW=25MHz to the requirements for NS_18 in Table 6.5.3.3-1 of TS 38.101-1.

# References

- R4-2217121 WF on requirements for HPUE_Basket_FDD, China Unicom, RAN4#104bis-e