3GPP TSG-RAN WG4 Meeting#106	R4-2300740

# Athens, Greece, 27 February – 3 March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1333 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to configured maximum power for inter-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. For inter-band UL CA band combinations (BC), the PHR for serving cells are not correct in case - the power class of the band combination is lower than the NR-band power class - the power class for a band is modified (derated) by the per-band-per-BC power class ue-PowerClassPerBandPerBC-r17 2. The PCMAX is not consistent with the definition of PCMAX in 38.213, the former more related to the PUMAX requirement rather than the threshold to be used for power prioritization when the UE is power limited. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2A.4: 1. the Pcmax,f,c for each serving cell of the band combination is modified with the limits appliable when the UE is configured with inter-band UL CA. 2. PCMAX is modified to be consistent with the definition in 38.213 and evaluated for a transmission occasion only accounting for other transmissions overlapping with the said occasion. Table 6.2A.4.1.3-0 of the evaluation window is removed for inter-band UL CA. 3. The measured output power PUMAX is verified similar to that for EN-DC with due account for power prioritization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PHR is overestimated when the power class of the band combination is lower that of the NR-band or when the power class for a band is modified (derated) by the per-band-per-BC ue-PowerClassPerBandPerBC-r17. The PCMAX is not consistent with the definition of PCMAX in 38.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This version corresponds to changes in Rel-16 but also includes additional cases. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

##### 6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

For a UE configured with uplink inter-band CA, the configured maximum output power PCMAX,c for each transmission occasion on serving cell c shall be set as specified in clause 6.2.4 modified by PEMAX,CA, PPowerClass,CA and ΔPPowerClass,CA as follows,

PCMAX_L,f,c = MIN {MIN(PEMAX,c, PEMAX,CA) – ∆TC,c,  MIN(PPowerClass – ΔPPowerClass, PPowerClass,CA – ΔPPowerClass,CA) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass, PEMAX,CA, PPowerClass,CA – ΔPPowerClass,CA}

where

-	PPowerClass is the maximum UE power for serving cell c as specified in Table 6.2.1-1 without taking into account the tolerance and as indicated by ue-PowerClassPerBandPerBC-r17, if present, ue-PowerClass otherwise;

-	PEMAX,CA is the value indicated by p-NR-FR1 or by p-UE-FR1 whichever is the smallest if both are present;

-	PPowerClass,CA is the maximum UE power specified in Table 6.2A.1.3-1 for the band combination without taking into account the tolerance specified in the Table 6.2A.1.3-1;

-	ΔPPowerClass,CA = 3 dB for a power class 2 capable UE when the requirements of default power class of the band combination are applied as specified in sub-clause 6.2A.1.3; otherwise ΔPPowerClass, CA = 0 dB.

The MPRc and A-MPRc apply per serving cell c and transmission occasion as specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,f,c  is calculated under the assumption that the transmit power is increased independently on all serving cells c. If the UE indicates higherPowerLimit-r17 as specified in Table 6.2A.1.3-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c, where pPowerClass,c is the maximum UE power for serving cell c as specified in sub-clause 6.2.4 and given in linear scale.

For uplink inter-band carrier aggregation with one serving cell c per operating band, the total configured maximum output power on serving cells in a respective transmission occasion l of any numerology is

PCMAX (l) = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA – ΔPPowerClass, CA}

with PEMAX,CA, PPowerClass,CA and ΔPPowerClass, CA as specified above and

pEMAX,c the linear value of PEMAX, c given by IE P-Max [7] if present for serving cell c.

If the UE indicates higherPowerLimit-r17 as specified in Table 6.2A.1.3-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.

If for the case of one uplink serving cell per band a transmission occasion l within a slot p(q) of numerology or symbol pattern i(j) on a serving cell is overlapping in time with one or more transmission occasions m = mk within one or more slots q(p) of slot numerology or symbol pattern j(i) on the other serving cell overlapping with slot p(q) in time, the UE allocates power to transmissions such that the total UE transmit power for transmissions on serving cells is smaller than or equal to PCMAX (l) in every symbol of transmission occasion l as specified in [8].

For combinations of intra-band and inter-band carrier aggregation with UE configured for transmission on three serving cells with up to two contiguously aggregated carriers per operating band and PUSCH/PUCCH/SRS transmissions of the same numerology on all serving cells in each operating band the following applies for a respective transmission occasion l:

PCMAX (l) = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA – ΔPPowerClass, CA}

with parameters as specified above. The total configured maximum output power PCMAX for the two contiguously aggregated cells shall be set as specified in sub-clause 6.2A.4.1.1 with PPowerClass,CA for the intra-band part replaced by the UE power class for the operating band. For the two contiguously aggegated serving cells c, the PCMAX,f,c  is set with MPRc = MPR and A-MPRc = A-MPR and P-MPR c = P-MPR for the operating band as specified in sub-clause 6.2A.4.1.1, while PCMAX,f,c  for the remaining serving cell c in the other operating band is calculated under the assumption that the transmit power is increased independently for the two operating bands.

The UE power class for the serving cells in the operating band including intra-band carrier aggregation is indicated by the parameter ue-PowerClassPerBandPerBC-r17 if present, ue-PowerClass otherwise.

If for the case of three uplink serving cells the said transmission occasion l within a slot p of numerology or symbol pattern i on a serving cell c1(i) in one operating band is overlapping in time with one or more transmission occasions m = mk within a slot of the same numerology in the same operating band and/or with one or more transmission occasions n = nk in one or more slots q of slot numerology or symbol pattern j on any serving cell c2(j) in the other operating band overlapping with slot p in time, the UE allocates power to transmissions such that the total UE transmit power for transmissions on all serving cells is smaller than or equal to PCMAX (l) in every symbol of transmission occasion l as specified in [8].

If the UE is configured with multiple TAGs and transmissions of the UE on slot i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on slot i +1 for a different serving cell in another TAG, the configuration of PCMAX (l) for a transmission occasion l also includes transmission occasions in the overlapping portion of slots i and i + 1.

For the case of one uplink serving cell per band, the measured maximum output power PUMAX over all serving cells during a transmission occasion l within a slot p of numerology or symbol pattern i on serving cell c(i) overlapping in time with one or more transmission occasions m = mk within one or more slot(s) q of slot numerology or symbol pattern j on serving cell c(j) overlapping with slot p in time shall be within the following range

P’CMAX_L (l)   – MAX{TL, TLOW(P’CMAX_L (l)) }  ≤  PUMAX  ≤  P’CMAX_H (l)   +  THIGH(P’CMAX_H (l))

with

PUMAX = 10 log10 ∑ pUMAX,f,c

P'CMAX_L (l) = MINk{10 log10[pCMAX_L,f,c(i)(l) + pCMAX_L,f,c(j)(m)]} of all m = mk

P'CMAX_H (l) = MIN{MAXk{10 log10[pCMAX_H,f,c(i)(l) + pCMAX_H,f,c(j)(m)], PCMAX (l)} of all m = mk

when pCMAX,f,c(i)(l) + pCMAX,f,c(j)(m) ≤ pCMAX (l) for all transmission occasions l and m = mk with pUMAX,f,c and pCMAX the respective measured maximum output power for serving cell c and total configured maximum power expressed in linear scale, and within the following range when the UE prioritizes power of transmissions

P’CMAX_L (l) – MAX{TL, TLOW(P’CMAX_L (l)) }  ≤  PUMAX  ≤  PCMAX (l)  + THIGH(PCMAX (l))

with the lower limit modified to

P'CMAX_L (l) = PCMAX_L,f,c(i)(l)

and pCMAX,f,c(i)(l) + pCMAX,f,c(j)(m) > pCMAX (l) for at least one m = mk and with the priority of each transmission occasion l higher than those of all transmission occasions m = mk according [8].

The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2A.4.1.3-1. The tolerance TL is the absolute value of the lower tolerance for applicable NR CA configuration as specified in Table 6.2A.1.3-1-2 for inter-band carrier aggregation. The period of measurement shall be at least one sub-frame.

For combinations of intra-band and inter-band carrier aggregation with UE configured for transmission on three serving cells with up to two contiguously aggregated carriers per operating band and PUSCH/PUCCH/SRS transmissions of the same numerology on all serving cells in each operating band, the measured maximum output power PUMAX over all serving cells during a transmission occasion l is determined as

P’CMAX_L (l)   – MAX{TL, TLOW(P’CMAX_L (l)) }  ≤  PUMAX  ≤  P’CMAX_H (l)   +  THIGH(P’CMAX_H (l))

with

PUMAX = 10 log10 ∑ pUMAX,f,c

P'CMAX_L (l) = MIN{10 log10[pCMAX_L,f,c(i)(l) + ∑ pCMAX_L,f,c]}

P'CMAX_H (l) = MIN{MAX{10 log10[pCMAX_H,f,c(i)(l) + ∑ pCMAX_H,f,c], PCMAX (l)}

when pCMAX,f,c(i)(l) + ∑ pCMAX,f,c ≤ pCMAX (l) for all transmission occasions l and any overlapping transmission occasions of the same or different numerology on the two other serving cells c, and within the following range when the UE prioritizes power of transmissions

P’CMAX_L (l) – MAX{TL, TLOW(P’CMAX_L (l)) }  ≤  PUMAX  ≤  PCMAX (l)  + THIGH(PCMAX (l))

with the lower limit modified to

P'CMAX_L (l) = PCMAX_L,f,c(i)(l)

and pCMAX,f,c(i)(l) + ∑ pCMAX,f,c > pCMAX (l) for transmission occasion l or any other overlapping transmission occasion on the other serving cells c and with the priority of the each transmission occasion l higher than those of the said overlapping transmission occasions.

Table 6.2A.4.1.3-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 28 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

##### 6.2A.4.1.4	Void

< end of changes >