3GPP TSG-RAN WG4 Meeting # 106							              R4-2300807

Athens, Greece, February 27 – March 3, 2023

Agenda item:	9.13.2.3

Source: 	CMCC

Title: 	Discussion on ATG UE Rx requirements

Document for:	Discussion

# 1. Introduction

In RAN4#105, the ATG UE RF requirements had been discussed and the WF was approved in [1]. In this meeting, we provide our views on the remaining issues.

# 2. Discussion

Figure 1. ATG network deployment

last meeting had an agreement about ATG network deployment. In the ATG network aircraft altitude to derive cell coverage is 3km. However, when calculating the link budget the minimum distance between ATG BS and UE is not 3km. Because the beam of the ATG BS in Figure 1 will not transmit vertically upwards and the ATG BS will not serve the aircraft directly above. Considering the following table 1, the minimum distance for the non sub-array model is 6.7 km, and the minimum distance for the sub-array model is 15.1 km.

Table 1. link budget for 256 QAM

|  | Non Sub-array model | Sub-array model |
| --- | --- | --- |
| Angle of aircraft at minimum altitude and maximum distance from ATG BS | 1.5° | 1.5° |
| Base station vertical coverage range | 25° | 10° |
| Mechanical uptilt | 14° | 6.5° |
| The minimum distance | 3/sin(25°+1.5°) = 6.7 (km) | 3/sin(10°+1.5°) = 15.1 (km) |

The minimum distance is 6.7 km for non sub-array and 15.1 km for sub-array model according to current deployment example.

The following Table 2 shows the link budget to calculate the maximum input level for ATG UE.

Table 2. Maximum input levek

|  | OmniOmni-direction | Beam forming | Beam forming (non sub-array) |
| --- | --- | --- | --- |
| Carrier frequency (GHz) | 2 | 4 | 4 |
| Distance (km) | 3 Considering that the possibility of ATG serving the aircraft directly above in the future still exists, we could use 3km as the minimum distance between ATG UE from BS | 3 Considering that the possibility of ATG serving the aircraft directly above in the future still exists, we could use 3km as the minimum distance between ATG UE from BS | 6.7 which is calculated as in table 1 |
| Path Loss (dB) | 108 | 114 | 121 |
| BS output power (dBm) | 46 | 53 | 53 |
| BS antenna gain can assume (dBi) | 25.1 | 25.1 | 25.1 |
| ATG UE antenna gain (dBi) | 0 | 17 | 17 |
| Maximum input level (dBm) | -37 | -19 | -26 |

Observation 1: max input level is -37dBm for 2GHz and -19dBm for 4GHz with min 3km distance assumption. If we assume min 6.7km min distance, max input level is -26dBm 4GHz.

Table 3. 2GHz Parameter Comparison

| 2GHz | ATG UE | MR BS | Legacy UE |
| --- | --- | --- | --- |
| Max input power/blocking | -37 | -38 | -25 |
| Max output power | 40 for 200km cell range 33 for 100km cell range | 38 | 26 |
| NF | [10] | 10 | 9 |

Observation 2: ATG UE for 2GHz is very similar to MR BS from perspective of output power, blocking requirement.

Table 3 compared some requirements between ATG UE, legacy UE and MR BS for 2GHz. Considering the max output power and blocking requirement, ATG UE is very similar to MR BS. In our view, ATG UE could use the same noise figure requirement as legacy UE.

Proposal 1: ATG UE could use 10dB as the noise figure for omni-direction assumption, the same as MR BS.

Table 4. 4GHz Parameter Comparison

| 4GHz | ATG UE | Legacy UE | MR BS |
| --- | --- | --- | --- |
| Max input power/blocking | -19 | -25 | -38 |
| Max output power | 26 | 26 | 38 |
| NF | larger than 9dB | 9 | 10 |

ATG UE for 4GHz is similar to legacy UE from the perspective of output power, but ATG UE has a higher requirement for max input power. Considering this difference, the noise figure could be relaxed from 9dB if we assume ATG UE at 4GHz is like handhold UE adding large antenna array.

Proposal 2: Noise figure of ATG UE for 4GHz could be relaxed compared with legacy UE 9dB NF if we assume ATG UE at 4GHz is like handhold UE adding large antenna array.

We notice that there are two NF requirements for ATG UE. Considering the calculation, two factors, frequency and antenna types lead to this difference. Different antenna types cause different ATG UE output power and finally cause different RF requirements. So we suggest defining at least two kinds of power classes based on the antenna types

Proposal 3: Define at least two kinds of power classes based on the antenna types. One for omni-direction and the other for antenna array.

# 3. Conclusions

In this contribution, ATG UE requirements are discussed with the following proposals:

Observation 1: max input level is -37dBm for 2GHz and -19dBm for 4GHz with min 3km distance assumption. If we assume min 6.7km min distance, max input level is -26dBm 4GHz.

Observation 2: ATG UE for 2GHz is very similar to MR BS from perspective of output power, blocking requirement.

Observation 2: ATG UE for 2GHz is very similar to MR BS from perspective of output power, blocking requirement.

Proposal 1: ATG UE could use 10dB as the noise figure for omni-direction assumption, the same as MR BS.

Proposal 2: Noise figure of ATG UE for 4GHz could be relaxed compared with legacy UE 9dB NF if we assume ATG UE at 4GHz is like handhold UE adding large antenna array.

Proposal 3: Define at least two kinds of power classes based on the antenna types. One for omni-direction and the other for antenna array.

# 4. References

[1] R4-2220543, WF on ATG UE RF requirements, Huawei, HiSilicon