3GPP TSG-RAN WG4 Meeting #106	R4-2301088

Athens, Greece, 27th February – 3rd March 2023

Source:	Ericsson, BT plc

Title:	TP for TR 37.718-11-21: Including band combination DC_(n)3-n28

Agenda item:			8.6.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-11-21 to add DC_(n)3-n28.

# 2. Text Proposal

---Start of changes---

## 6.x	DC_(n)3-n28

### 6.x.1	Configuration for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

Table 6.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_(n)3AA-n28A | DC_(n)3AA DC_3A_n28A |

### 6.x.2 	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n28A |  |  | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  |  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

UL DC_3_n28 does not give IMD into DL band n3.

### 6.x.4		∆TIB and ∆RIB values

For DC_(n)3-n28, the TIB,c and RIB,c values are reused from DC_3_n28 and are given in the tables below.

Table 6.x.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n28 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n28 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.x.5	MSD requirements

MSD value does not need to be defined.

---End of changes---

# Reference

[1]		R4-2218027,	Revised WID on Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics Deutschland

[2]		R4-2218026, TR 37.718-11-21 v0.3.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18, LG Electronics Deutschland