# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301225

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on band definition for 900 MHz LTE Band

# Agenda Item:	10.4.2

# Document for:	Approval

- Introduction

During RAN#98-e meeting, a new Work Item [1] has been approved on introduction of 900 MHz LTE Band in the US. In this contribution, we want to share some initial views on band definition.

- Specify a new E-UTRA/LTE FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 5 MHz, 3 MHz, 1.4 MHz and NB-IoT channel bandwidths with 15 kHz SCS and 3.75kHz SCS.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Discussion

  - Operating band and band numbering

As indicated in the objective, a new E-UTRA/LTE FDD operating band should be defined.

Table 2.1-1: E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [106] | 896 MHz | – | 901 MHz | 935 MHz | – | 940 MHz | FDD |

Proposal 1: To define the new LTE band as in table 2.1-1.

  - Channel bandwidth and transmission bandwidth configuration

For the new E-UTRA/LTE FDD operating band, as stated in the Work Item, it supports 5 MHz, 3 MHz, 1.4 MHz and NB-IoT channel bandwidths with 15 kHz SCS and 3.75kHz SCS. The transmission bandwidth configuration should follow TS 36.101.

Table 2.2-1: E-UTRA channel bandwidth

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 106 | Yes | Yes | Yes |  |  |  |

Table 2.2-2: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 | 3 | 5 |
| --- | --- | --- | --- |
| Transmission bandwidth configuration NRB | 6 | 15 | 25 |

Proposal 2: For the new LTE band, it supports 1.4MHz, 3MHz and 5MHz channel bandwidth.

Proposal 3: For the new LTE band, to reuse the transmission bandwidth configuration of TS 36.101.

2.2.1 Channel bandwidth for NB-IoT

It’s also proposed to reuse the transmission bandwidth configuration of TS 36.101 for the new LTE band which supports NB-IoT.

Table 2.2-3: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB-IoT channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Proposal 4: For the new LTE band which supports NB-IoT, to reuse the transmission bandwidth configuration of TS 36.101.

  - Carrier frequency and EARFCN

For E-UTRA channel numbers, we propose to use the similar descending manner to define as following:

Table 2.3-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low(MHz) | NOffs-UL | Range of NUL |
| [106] | 935 | 70656 | 70656 – 70705 | 896 | 134292 | 134292 – 134341 |

Proposal 5: For E-UTRA channel numbers, it is proposed to define as Table 2.3-1.

  - TX–RX frequency separation

For TX–RX frequency separation, we propose to define as following:

Table 2.4-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [106] | 39 MHz |

Proposal 6: For TX–RX frequency separation, it is proposed to define as Table 2.4-1.

  - Summary of corrections on TS 36.101

Based on the above, the summary of the corrections on TS 36.101 are given in Table 2.5-1.

Table 2.5-1: The summary of the corrections on TS 36.101

| General part |  |
| --- | --- |
| Operating bands | A new FDD operating band is needed. |
| UE channel bandwidth |  |
| - Channel bandwidths per operating band | The new FDD band supports 1.4 MHz, 3 MHz and 5 MHz channel bandwidth. |
| - transmission bandwidth configuration | To reuse the transmission bandwidth configuration of TS 36.101. |
| - Channel bandwidth for NB-IoT | To follow the existing requirements in TS 36.101. |
| Channel arrangement |  |
| - Channel spacing | To follow the existing requirements in TS 36.101. |
| - Channel raster | To reuse 100 kHz channel raster. |
| - Carrier frequency and EARFCN | The EARFCN channel numbers should be needed for the new added FDD band. |
| - TX–RX frequency separation | To define as 39 MHz. |

- Conclusion

In this contribution, we give some initial discussion on band definition for 900 MHz LTE band and the proposals are made as following:

Proposal 1: To define the new LTE band as in table 2.1-1.

Proposal 2: For the new LTE band, it supports 1.4MHz, 3MHz and 5MHz channel bandwidth.

Proposal 3: For the new LTE band, to reuse the transmission bandwidth configuration of TS 36.101.

Proposal 4: for the new LTE band which support NB-IoT, to reuse the transmission bandwidth configuration of TS 36.101.

Proposal 5: For E-UTRA channel numbers, it is proposed to define as Table 2.3-1.

Proposal 6: For TX–RX frequency separation, it is proposed to define as Table 2.4-1.

In addition, the summary of the corrections on TS 36.101 are given in Table 2.5-1.

- Reference

RP-223530, New WID on Introduction of 900 MHz LTE Band in the US, Anterix