3GPP TSG-RAN WG4 Meeting # 106 	R4-2301423

Athens, Greece, February 27 – March 3, 2023

Source: 	Huawei, HiSilicon

Title: 	On general section 5.1

Agenda Item:	9.4.2

Document for:	Discussion

1 Introduction

The feasibility part of the SI focusses on wideband RF architectures and devices and wide band antenna array configurations. The context of these studies is outlined in the general section of the same clause.

In previous meetings some contributions have been submitted with extensive “general discussion” in the general clause however this information overlaps with the technical sub-clauses and we believe that such discussion should be placed in those technical sub-clauses, the general clause should be used for issues which are applicable to the whole system.

The TP below offers text for the general section using this approach, which is for discussion.

Summarizing the feasibility study art of the SI and forming conclusions may be difficult due to the nature of the many variables, however potentially such summaries could be placed in the general section or in a summary section at the end, however they should perhaps be agreed prior to capturing them in a TP.

2 TP for TR 38.877

5	Feasibility study

5.1	General

The feasibility study focuses on the 2 core RF parts of the FR2-1 BS the RF architecture and the antenna architecture. It is feasible that not all parts of the BS are wideband but that some advantage can be gained by implementing some parts as multi-band. For example one potential advantage of a multiband PA is that power can be distributed between the 2 bands offering more flexibility to optimise the power available for each, this requires only a multi-band PA.

In the RF architecture here is a focus on a number of key components:

- RF front end including PA

- DPD

- Phase shifter

In addition the antennas array can be implemented as either a wide band (multi-band) antenna or as stacked patches.

When discussing the feasibility of wideband components/systems the multi-band bandwidth as a percentage should be considered.

Note this is generally called fraction BW but in RAN4 specifications the term fractional BW is already defined to mean something specific so is best avoided to prevent confusion.

Table 5.1-1 shows the percentage bandwidth for the FR2-1 bands and various potential multi-band groups.

Table 5.1-1:Percentage band width of FR2-1 operating band (s)

| NR operating band | Band | Percentage bandwidth |
| --- | --- | --- |
| n258 | 24250 MHz – 27500 MHz | 12.6% |
| n257 | 26500 MHz – 29500 MHz | 10.7% |
| n261 | 27500 MHz – 28350 MHz | 3% |
| n260 | 37000 MHz – 40000 MHz | 7.8% |
| n259 | 39500 MHz – 43500 MHz | 9.6% |
| n262 | 47200 MHz – 48200 MHz | 2.1% |
| 24-29 GHz (includes n257/n258/n261) | 24250 MHz –29500 MHz | 19.5% |
| 37-48 GHz (includes n260/n259/n262) | 37000 MHz – 48200 MHz | 26.3% |
| 26+28 GHz (includes n258 + n261) | 24250 MHz – 29500 MHz | 19.5% |
| 28+39 GHz (includes n257/n261 + n260) | 26500 MHz – 40000 MHz | 40.6% |
| 26+40 GHz (includes n258 + n259/n262) | 24250 MHz – 48200 MHz | 66.1% |
| 28+40 GHz (includes n257/n261 + n259/n262) | 26500 MHz – 48200 MHz | 58.1% |

5.2	Wideband RF architectures

5.2.1	RF Front end

5.2.2	DPD

5.2.3	Phase shifter

5.3	Wideband antenna architectures