3GPP TSG-RAN WG4 Meeting # 106	R4-2301538

# Athens, Greece, Feb. 27th – Mar. 3rd, 2023

Source: 	vivo

Title: 	Discussion on 8Rx UE RF requirements

Agenda Item:	9.6.3

Document for:	Discussion

Introduction

The 8Rx in FR1 enh has been discussed for several meetings, the remaining issues has been captured in the WF [1] and the Topic summary is in [2].

In this paper, some views were provided for the remaining issues.

Discussion

PDCCH aggregation level andΔRIB

This value has been discussed for some time. If the PDCCH aggregation level is changed, slight better performance compared to LTE may be expected. If PDCCH aggregation level is kept, reusing LTE value of 4.0dB might be more reliable. We tend to prefer what is proposed in [3].

Proposal 1: Setting the delta_Rib according to the PDCCH aggregation level.

- If PDCCH aggregation kept then the same -4.0dB with LTE is expected.

- If PDCCH aggregation level = 8, the requirement could be further improved by 0~0.5dB.

ΔTRxSRS Reporting and main branch value

There are a lot of discussion on the reporting of this value to network. Theoretically, any new information could be beneficial in certain scenario. However, we tend to agree with what [4] has proposed, that it seems quite limited value is perceived for this reporting.

Proposal 2: It is prefer not to introduce the indication of ΔTRxSRS Reporting to the network, unless more clear benefit can be seen.

For the insertion loss in main branch value. 0dB seems a more reasonable design target, since the main branch UL performance is important.

Proposal 3: 0dB is preferred for ΔTRxSRS in main branch.

Conclusion

In this paper, some views are provided for 8Rx for FR1 enh.

Proposal 1: Setting the delta_Rib according to the PDCCH aggregation level.

- If PDCCH aggregation kept then the same -4.0dB with LTE is expected.

- If PDCCH aggregation level = 8, the requirement could be further improved by 0~0.5dB.

Proposal 2: It is prefer not to introduce the indication of ΔTRxSRS Reporting to the network, unless more clear benefit can be seen.

Proposal 3: 0dB is preferred for ΔTRxSRS in main branch.

- References

[1] R4-2220516, WF on 8Rx RF requirements, NTT DOCOMO, RAN4#105

[2] R4-2220108, Topic summary for [105][128] FR1_enh2_part3, RAN4#105

[3] R4-2218209, Discussion on FR1 8RX UE RF requirements, MediaTek, RAN4#105

[4] R4-2220033, Discussion on UE RF requirements for 8Rx in FR1, Ericsson, RAN4#105