3GPP TSG-RAN WG4 Meeting # 106										  R4-2301617

Athens, Greece, February 27 – March 3, 2022

Title: 	Discussion on UL Tx switching with multiple TAG

Source: 	Xiaomi

Agenda item:	9.22.2.2

Document for:	Approval

- Introduction

In RAN4#105 meeting, one WF was approved on UL Tx switching with multiple TAG [1]. However, switching time mask for dual TAG with timing difference still needs further discussion. This contribution will provide our view on the switching time mask.

- Discussion

The candidate options for switching time mask for dual TAG are captured in the WF [1] as below:

#### Issue 2-1-1: Need of new time mask requirements for 2-TAG case Option 1: Time mask with an UL time difference covering all possible time delays (R4-2218827), example for 2 bands but also relevant for 3-4 bands for both SUL and CA configurations with dual TAG. Option 2: Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction. Example below (R4-2219410) for two bands: General understanding: only full OS are “blanked” (not scheduled or configured for transmission) WF: TBD

RAN4 agreed the switching time for UL Tx switching is the same for single TAG and multiple TAG cases but the switching time mask for multiple TAG is FFS. Two options are proposed during the previous discussion and the difference is whether to include TA difference in the time mask.

We understand the technical point of option 1 but it would not be a good solution. TA difference varies from the distance between UE to BS. It is not a constant value. In our understanding option 1 will potentially cause more challenge and burden for RAN5 to do the test with a lot of uncertainty. For example, whether to test all possible cases with different TA differences or choose several or a proper TA difference, and how to decide a proper TA difference is not clear or needs to be decided.

The switching time mask should capture time period and location for switching on UE side that are used to verify hardware performance. The timing difference related issue can be left up to network scheduling. The similar issue was discussed in Rel-17 NR intra-band V2X con-current operation. Uu and SL are operated in the same licensed band with different TA. Finally RAN4 agreed the switching time mask should not capture the TA difference. The hardware performance can be verified in the test with such a time mask. The same principle can be followed for UL Tx switching with multiple TAG.

TS 38.101-1 #### 6.3E.3.4 Transmit ON/OFF time mask for V2X con-current operation For intra-band V2X con-current operation band specified in subclause 5.3.E.2, the switching time mask in Figure 6.3E.3.4-2 shall apply for the different carrier case. The switching time shall be located on the RAT of low priority when NR Uu and NR SL have different priorities based on priority information specified in TS 38.321 and TS38.213. It is up to UE implementation when NR Uu and NR SL have the same priority based on priority information specified in TS 38.213. Figure 6.3E.3.4-2: Time mask for switching between Uu and SL for different carrier case In the real field, there is a timing advance difference, i.e. NTA∙Tc between NR Uu slot and NR SL slot due to different timing advance of NR Uu and NR SL. The switching time masks do not include timing advance difference but the timing advance difference should be considered with the switching time for same carrier case and different carrier case.

Proposal 1: The switching time mask for single TAG case can be used for multiple TAG case.

- Conclusion

This contribution gives our view on UL Tx switching with multiple TAG. The following proposal are derived:

Proposal 1: The switching time mask for single TAG case can be used for multiple TAG case.

- Reference

- R4-2220547, WF on UE Tx switching requirement for multiple TAG, Ericsson, RAN4#105