3GPP TSG-RAN WG4 Meeting # 106	R4-2301645

# Athens, Greece, February 27 – March 03, 2023

Agenda Item:	9.24.2.4

Source: 	OPPO

Title: 	Discussion on network B requirements

Document for:	Approval

1	Introduction

MUSIM gaps can be configured to the UE for the measurement, paging reception, on-demand SI request and etc in NW-B. Last meeting discussed some open issues and summarized in the WF [1]. In this contribution, our views on these issues will be provided.

Issue 4-1-1: Whether to define network B requirements - Proposals - Option 1: Deprioritize NW B requirement at R18. (Apple xiaomi) - Option 1a: If necessary, only idle/inactive mode measurement/cell reselection requirements need to be considered. (Apple oppo) - Option 2: No measurement requirements in network B will be defined by RAN4 at R18 (vivo oppo Huawei MTK Qualcomm) - Option 3: RAN4 need to define specific UE requirements for Network B when UE is allocated MUSIM gaps (Nokia) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode when no MUSIM gap collision happens. (Ericsson) Issue 4-1-2: Network B requirements if it will be defined - Proposals - P1: Framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example: (Apple) - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP) (oppo Huawei) - P3: The UE requirements for Network B are the same as/re-use the existing UE requirements (Nokia)

2	Discussion

For NW-B, the measurement period will also be impacted by MUSIM gaps. When MUSIM gap is dropped due collision, the measurement period will be extended. However, it may be hard for NW-B to get the full information of gap collision and the associated priority without new signalling introduced in RAN2, especially for the collision between MUSIM gap and legacy gap in NW-A. In this scenario, no requirement is also acceptable for us.

Proposal-1: No measurement requirements in NW-B will be defined by RAN4.

If NW-B requirements will be defined, we agree with P1 to focus on cell reselection since there is no RRM requirement for paging reception or SIB reading.

Proposal-2: If NW-B requirements will be defined, only cell reselection in RRC idle/inactive mode should be considered.

3	Conclusion

This contribution gave our views on NW-B requirements with Rel-17 MUSIM gaps and the following proposals:

Proposal-1: No measurement requirements in NW-B will be defined by RAN4.

Proposal-2: If NW-B requirements will be defined, only cell reselection in RRC idle/inactive mode should be considered.

4	Reference

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo