3GPP TSG-RAN WG4 Meeting # 106	R4-2301694

Meeting, February 27th – March 3rd , 2023

Title:	TP to TR 38.718-02-01 Addition of CA_n78-n102 and DC_n78-n102

Source:	Nokia, BT

Agenda item:	8.10.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.718-02-01 to add CA_78A-n102A with UL interband CA and variants of non-contiguous and contiguous intra-band carrier aggregation in downlink including UL interband CA with intra-band uplink CA support of this band combination. It also includes the NR DC combinations of the same. Some UL configurations in this TP is dependent in fallbacks in R4-2301692.

************************************* Start of TP*****************************************

## 5.x		CA_n78-n102

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1 Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n78+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n78+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n78A-n102A | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78A-n102(2A) | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n102B | CA_n78A-n102A CA_n78A-n102B | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78A-n102C | CA_n78A-n102A CA_n78A-n102C | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78A-n102D | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78A-n102E | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102A | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78(2A)-n102B | CA_n78(2A) CA_n78A-n102A CA_n78A-n102B | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78(2A)-n102C | CA_n78(2A) CA_n78A-n102A CA_n78A-n102C | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78(2A)-n102D | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78(2A)-n102E | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102(2A) | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |

#### 5.x.1.3	Co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n78-n102.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no uplink harmonic issues.

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no harmonic mixing issues.

#### 5.x.1.4	∆TIB and ∆RIB values

For CA_n78-n102, the DTIB,c is based on CA_n78-n79 having n79 as the highest frequency range among similar CA’s. Since this case may have simultaneous RX/TX the values with Note 8 have been used, For CA_n78-n102, the DRIB values are also based on CA_n46-n78.

Table 5.x.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n78-n102 | 1.5 | 1.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n78-n102 | 0.5 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENs requirements

#### There are no specific REFSENS requirements for 1 band UL

#### 5.x.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n102A | 23 | +2/-3 |

#### 5.x.2.2		UE co-existence

Table 5.x.2.2-1 gives IMD interference analysis for CA_ n78-n102 with uplink CA.

Table 5.x.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 3800 | 5925 | 6425 |
| DL Frequency [MHz] | 3300 | 3800 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2125 | 3125 | 9225 | 10225 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 175 | 1675 | 8050 | 9550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12525 | 14025 | 15150 | 16650 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3475 | 5475 | 13975 | 15975 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6250 | 4250 | 18450 | 20450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 15825 | 17825 | 21075 | 23075 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19900 | 9275 | 6775 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12675 | 10175 | 450 | 2950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27000 | 29500 | 19125 | 21625 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 24375 | 26875 | 21750 | 24250 |

Based on the table 5.x.2.2-1, there are no IMD interference issues of this band combination

Table 5.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n78-n102 | E-UTRA Band 1, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 30, 39, 40, 44, 45, 50, 51, 53, 65, 66, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 2, 7, 25, 41, 34, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.x.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

### 5.x.3		Specific for 2 bands with 1 non-contiguous UL CA

#### 5.x.3.1		UE co-existence

Table 5.x.3.1-1 gives IMD interference analysis for CA_n78-n102 with non-contiguous uplink CA in n78.

Table 5.x.3.1-1: IMD analysis non-contiguous uplink CA

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

The analysis shows that the 4th and the 6th order IMD products of n78 will fall inside the RX band of n102 as the band combinations is defined with simultaneous RX/TX.

#### 5.x.3.2		REFSENS requirements

The TDD-TDD combination is assumed simultaneous RX/TX operation. There’s a non-contiguous UL CA case of n77 hitting n5 RX as IMD4. This MSD value is based on isolation between antennas of different operating frequency range and we propose similar value is applied for this case.

Table 5.x.3.2-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n78-n102 | n7812 | 3350 | 10 | 1 (RBstart=0) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBstart=10) | 3750 |  |  |  |
|  | n102 | N/A | 20 | N/A | 6300 | 8.6 | TDD | IMD4/6 |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

# 7	Dual Connectivity: Specific Band Combination Part

## 7.x	DC_n78-n102

#### 7.x.1	Configurations for DC_n78-n102

Table 7.x.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n78A-n102A | DC_n78A-n102A |
| DC_n78A-n102D |  |
| DC_n78A-n102E |  |
| DC_n78A-n102B | DC_n78A-n102A |
|  | DC_n78A-n102B |
| DC_n78A-n102C | DC_n78A-n102A |
|  | DC_n78A-n102C |
| DC_n78A-n102(2A) | DC_n78A-n102A |
| DC_n78(2A)-n102A | DC_n78A-n102A |
| DC_n78(2A)-n102D |  |
| DC_n78(2A)-n102E |  |
| DC_n78(2A)-n102B | DC_n78A-n102A |
|  | DC_n78A-n102B |
| DC_n78(2A)-n102C | DC_n78A-n102A |
|  | DC_n78A-n102C |
| DC_n78(2A)-n102(2A) | DC_n78A-n102A |

#### 7.x.2	Maximum output power for NR-DC

Table 7.x.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n78A-n102A | 23 | +2/-3 |

************************************* End of TP*****************************************