3GPP TSG RAN WG4 Meeting #106		 R4-2301710

# 27 February - 3 March 2023, in Athens, Greece

Agenda Item:	8.27.2

Source: 	MediaTek Inc.

Title: 	Discussion simultaneous Rx/Tx inter-band combinations for CA_n40-n41

Document for:	Discussion

1. Introduction

In RAN4 #105, simultaneous Rx/Tx operation for CA_n40-n41 were not widely discussed and no consensus reached. The contribution provides continue discussion and calculation on the requirements for simultaneous Rx/Tx operation

2. Discussion

The CA_n40-n41 has been discussed in last RAN4 meeting. It was found there may be MSD due to cross band isolation if simultaneous Rx/Tx operation is required. One of the WI[1] scope is to harmonize the requirements for NR CA, EN-DC and NR SUL corresponding combos. We provide further discussion on MSD for CA_n40-n41. In last RAN4 meeting we proposed re-use the MSD due to mandatory simultaneous Rx/Tx operation for SUN_n41-n97 specified in existing specs as starting point. We found that the assumption was not proper for some of MSD test point configuration. So this time we re-calculate MSD on n40/n41 with the components data in our vendor pool and calculation assumption listed below

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| PA ACP2 | -50 | dBc |
| Front-end loss | 4 | dB |
| n41 filter rejection on n40 range | 30 | dB |
| n40 filter rejection on n41 range | 36 | dB |
| PA RXBN noise | -117 | dBm/Hz |
| Thermal noise at n41 RX ANT port | -165.1 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

Regarding DL BW=100MHz, since uplink ACP2 is the dominant noise source, for aggressor UL=80MHz, its ACP2 range falls in n41 would be 2500~2560MHz. The MSD test point are calculated as table below:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2505 | 10 | 21.5 | ACLR2 |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=0) | 2550 | 100 | 15.7 | ACLR2 |

Table 2. Proposed MSD due to cross band isolation on n41

From calculation, we found that uplink in n40 would act like OOB blocker so the MSD would be quite large due to n41 filter insufficient rejection on n40 range.

As for the uplink band in n41 and downlink band in n40, considering the similarity of the two TDD bands, the above assumption stands. So we propose aggressor uplink BW to be same as 80MHz that the victim can still maintain in the ACLR2 region. For DL=100MHz, if aggressor fc is at 2536MHz, there will be 64MHz overlap due to aggressor ACP2 fall in n40 DL 100MHz channel. The calculated MSD is proposed below table. The ACP2 of n41 aggressor fall in n40 range due to poor n41 filter rejection is dominant factor.

Proposal 1: MSD due to cross band isolation of CA_n40-n41 when enabling simultaneous Rx/Tx operation is proposed as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2505 | 10 | 21.5 | ACLR2 |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=0) | 2550 | 100 | 15.7 | ACLR2 |
| n41 | n40 | 2536 | 80 | 30 | 216 (RBstart=1) | 2395 | 10 | 22.5 | ACLR2 |
| n41 | n40 | 2536 | 80 | 30 | 216 (RBstart=0) | 2350 | 100 | 19.1 | ACLR2 |

Proposal 2: To harmonize requirements as instructed in the WID, the MSD of SUL_n41-n97 is proposed to align CA_n40-n41 as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2505 | 10 | 21.5 | ACLR2 |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=0) | 2550 | 100 | 15.7 | ACLR2 |

Let’s further consider antenna design for smartphone. It is usually required for one antenna to support multiple band frequency range. However, due to limited space for antenna design, there’s trade off that not all antennas can support full frequency range of all supported bands. In general, there’s less margin on transmit TRP requirement then receiving TIS requirements. There may also be some other features such as UL MIMO/TxD, SRS antenna switching and Tx switching need to be supported simultaneously. In our initial estimation, we found it is difficult to have all antennas to meet design requirements on both transmission and receiving of all the supporting bands and MRDC/NRDC combinations fulfil multiple UE capability at the same time.

Proposal 3: For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

3. Conclusion

Proposal 1: MSD due to cross band isolation of CA_n40-n41 when enabling simultaneous Rx/Tx operation is proposed as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2505 | 10 | 21.5 | ACLR2 |
| n40 | n41 | 2360 | 80 | 30 | 216 (RBstart=0) | 2550 | 100 | 15.7 | ACLR2 |
| n41 | n40 | 2536 | 80 | 30 | 216 (RBstart=1) | 2395 | 10 | 22.5 | ACLR2 |
| n41 | n40 | 2536 | 80 | 30 | 216 (RBstart=0) | 2350 | 100 | 19.1 | ACLR2 |

Proposal 2: To harmonize requirements as instructed in the WID, the MSD of SUL_n41-n97 is proposed to align CA_n40-n41 as below

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=1) | 2505 | 10 | 21.5 | ACLR2 |
| n97 | n41 | 2360 | 80 | 30 | 216 (RBstart=0) | 2550 | 100 | 15.7 | ACLR2 |

Proposal 3: For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

Reference:

- RP-223354 WID revision: Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 RAN-P#98-e