3GPP TSG-RAN WG4 Meeting#106                               R4-2301744

Athens, Greece,27th Feb – 3rd Mar,2023

Agenda item:	9.25.1.1

Source: 	ZTE Corporation

Title: 	Further discussion on system parameter for NTN in Ka band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In this contribution, we want to share further views for system parameters for NTN in Ka-band.

Agreement:

- FSS spectrum should be included in 3GPP band definition for the purpose of enabling ESIM/moving VSAT moveable NTN terminal with priority.  [Fixed VSAT in FSS allocation – Companies to check RAN4 ToR to see if there is a restriction on fixed device in fixed spectrum]

- Define several NTN Ka-bands to address the diversity of spectrum allocation to ESIM by national regulators, RAN4 should consider specifying several “NTN Ka-bands”

Discussion

## 2.1. Channel bandwidth

In Last RAN4 meeting, we agree to define 400MHz as optional for NTN UE in Rel-18. Regarding higher channel bandwidth beyond 400MHz, we don’t see its necessity for it. In Rel-17 FR2-2, we have defined larger channel bandwidth for it, however this is not applicable for FR2-1. Ka-band is somehow overlapping with band n257 which is still part of FR2-1. For MSS-band, maximum available spectrum should be 500MHz which is slightly larger than 400MHz, however if we want to define 500MHz channel bandwidth for NTN UE in Ka-band in Rel-18, then it could cause quite lots of additional standardization work and cannot reuse the existing FR2-1 implementation as much as possible. Based on above analysis, we made the following proposal:

Proposal 1: not to define channel bandwidth beyond 400MHz for Ka-band MSS-band.

In the last RAN4 meeting, there were some initial discussions for Asymmetric channel bandwidths between DL and UL in Ka-band, we are also open for further discussion. In general, from system parameter perspective, for asymmetric channel bandwidth, we need to define the following Tx-Rx carrier center frequency separation due to the constraints of duplex design of FDD band, however as for NTN UE filtering assumption in last RAN4 meeting, due to the large frequency gap up to 7GHz in Ka-band MSS-band, then no such of filtering or duplex are needed for Ka-band MSS-band, in other words, the following restriction for Tx-Rx carrier center frequency separation are not needed.

5.3.6	Asymmetric channel bandwidths

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 5.4.4-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Observation 1: the legacy restriction for Tx-Rx carrier center frequency separation might be not needed if there are no filtering or duplex between MSS-band DL and UL.

## 2.2. Spectral utilization

For the spectral utilization for NTN Ka-band, technically speaking, the spectral utilization should be more or less dependent on the waveform, ACLR, out of band emission, ACS and EVM requirements etc. However based on the experience of Rel-17 NR over NTN discussion, it’s most likely that SAN RF requirement for NTN Ka-band would be relaxed compared with the RF requirement of the terrestrial BS, therefore we don’t see the problem to follow the spectral utilization of terrestrial NR system.

Proposal 2: to use the same spectral utilization requirement of FR2 TN system for NTN Ka-band as starting point.

## 2.3. Channel raster and sync raster

The channel raster for NTN Ka-band could be be the same as band n257, i.e. 60/120kHz to the simplify the VSAT /ESIM/MES implementation complexity for initial access. In addition, sync raster could use similar approach as band n257. It should be noted that range of NREF should fall into 3GHz to 24.25GHz instead of 24.25-52.6GHz. Therefore global raster should be still 15kHz while with step size as 4 and 8 for 60kHz and 120kHz ΔFRaster.

Proposal 3: to use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band.

Table 2.3-1: NR-ARFCN parameters for the global frequency raster [TS 38.101-5]

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 17700 – 20200 | 15 | 17700 | 1580000 | 1580000 – 1746664 |

Table 2.4.2. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| [n512] | 60 | 1580000 – <4> – 1746664 |
|  | 120 | 1580000 – <8> – 1746664 |

Similar as NR-ARFCN, it should be noted that range of GSCN should fall into 3GHz to 24.25GHz instead of 24.25-52.6GHz. Therefore sync raster should be still 1.44MHz while with step size as 12 (17.28MHz) and 24 (34.56MHz) for 120kHz and 240kHz SSB.

Table 2.3-3: GSCN parameters for the global frequency raster

| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

| 17700 – 20200 MHz | 3000MHz MHz + N *1.44 MHz N = 10209:11944 | 7499 + N | 17708 – 19443 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Table 2.3.4. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n512] | 120 kHz | Case D | 17724- <12> - 19428 |
|  | 240 kHz | Case E | 17736 - <24> - 19416 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: Step size 12 equal to 17.28MHz by 1.44MHz*12 and step size 24 equals to 34.56MHz by 1.44MHz*24. |  |  |  |

Proposal 4: to use following GSCN in the Table 2.3-3/2.3-4 for NTN Ka-band MSS-band.

In addition, it should be noted that NTN Ka-band is not belonging to any FR1 or FR2 definition. From our understanding, if we want to apply SSB block pattern D and SSB pattern E for NTN Ka-band, some updates in TS 38.213 is needed at least.

In addition,

-	Case D - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 4,8,16,20+28⋅n. For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

-	Case E - 240 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 8,12,16,20,32,36,40,44+56⋅n. For carrier frequencies within FR2-1, n=0, 1, 2, 3, 5, 6, 7, 8.

Proposal 5: to send LS to RAN1 to indicate the NTN Ka-band support the Case D and Case E..

Conclusions

In this contribution, we want to share some further views from system parameter perspective for NTN Ka-band.and proposals are made as following:

Proposal 1: not to define channel bandwidth beyond 400MHz for Ka-band MSS-band.

Observation 1: the legacy restriction for Tx-Rx carrier center frequency separation might be not needed if there are no filtering or duplex between MSS-band DL and UL.

Proposal 2: to use the same spectral utilization requirement of FR2 TN system for NTN Ka-band as starting point.

Proposal 3: to use following NR-ARFCN in the Table 2.3-1/2.3-2 for NTN Ka-band.

Table 2.3-1: NR-ARFCN parameters for the global frequency raster [TS 38.101-5]

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 17700 – 20200 | 15 | 17700 | 1580000 | 1580000 – 1746664 |

Table 2.4.2. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| [n512] | 60 | 1580000 – <4> – 1746664 |
|  | 120 | 1580000 – <8> – 1746664 |

Proposal 4: to use following GSCN in the Table 2.3-3/2.3-4 for NTN Ka-band MSS-band.

| 17700 – 20200 MHz | 3000MHz MHz + N *1.44 MHz N = 10209:11944 | 7499 + N | 17708 – 19443 |
| --- | --- | --- | --- |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

Table 2.3.4. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n512] | 120 kHz | Case D | 17724- <12> - 19428 |
|  | 240 kHz | Case E | 17736 - <24> - 19416 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: Step size 12 equal to 17.28MHz by 1.44MHz*12 and step size 24 equals to 34.56MHz by 1.44MHz*24. |  |  |  |

Proposal 5: to send LS to RAN1 to indicate the NTN Ka-band MSS-band support the Case D and Case E..

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2217496, Email discussion summary for [104-bis-e][312] NR_NTN_enh_Part1

- R4-2217467, WF on NR NTN Enhancements Part1, Thales, approved.

- R4-1709181,WF on NR band Numbering, approved.

- R4-2220224 Ad-hoc minutes for NTN above 10GHz band introduction

- R4-2220239 WF for above 10GHz band definition and system parameters,Thales, Approved.