3GPP TSG- RAN4 Meeting # 106	R4-2301839

# Athens, Greece, February 27 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2931 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding relaxation factor on measurement relaxation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2023-2-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In R16 power saving, when UE met low mobility criterion or not-at-cell edge criterion, measurement relaxation factor K1 shall be applied. However in Table 4.2.2.10.2-1 and Table 4.2.2.10.3-1, K1 is lost. |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding relaxation factor K1 in Table 4.2.2.10.2-1 and Table 4.2.2.10.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation for measurement relaxation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.10.2 and 4.2.2.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with lowMobilityEvaluation [2] criterion and UE has fulfilled, or

-	UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the lowMobilityEvaluation [2] criterion.

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements are defined as follows:

-	Tdetect,NR_Inter_Relax as specified in Table 4.2.2.10.2-1.

-	Tmeasure,NR_Inter_Relax as specified in Table 4.2.2.10.2-1.

-	Tevaluate,NR_Inter_Relax as specified in Table 4.2.2.10.2-1.

-	The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax  * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax  * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax *Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax  * Tevaluate,NR_Inter.

-	When T331 is running,

-	The parameter Ncarrier_Relax is the total number of NR inter-frequency carriers not configured for idle mode CA/DC measurements.

-	The parameter Ncarrier_Non_relax is the total number of NR inter-frequency carriers configured for idle mode CA/DC measurements.

-	When T331 is not running,

-	The parameter Ncarrier_Relax is the total number of inter-frequency carriers configured for mobility measurements only and the number of inter-frequency carriers configured for both mobility measurement and idle mode CA/DC measurements.

-	The parameter Ncarrier_Non_relax =0.When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.10.2-1: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x K1 x 1.5 (36 x N1 x K1 x 1.5) | 1.28 x N1 x K1x 1.5 (4 x N1 x K1 x 1.5) | 5.12 x N1 x K1x 1.5 (16 x N1 x K1 x 1.5) |
| 0.64 |  | 5 | 8 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 6 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |  |

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with cellEdgeEvaluation [2] criterion, and UE has fulfilled or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] not configured, and

-	UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The UE shall not relax measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements (defined in clause 4.4) while T331 is running.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2.2.4 apply for this clause except that:

-	Tdetect,NR_Inter_Relax as specified in Table 4.2.2.10.3-1.

-	Tmeasure,NR_Inter_Relax as specified in Table 4.2.2.10.3-1.

-	Tevaluate,NR_Inter_Relax as specified in Table 4.2.2.10.3-1.

-	The UE shall be able to evaluate whether a newly detectable inter-frequency NR cell meets the reselection criteria defined in TS38.304 [1] within Ncarrier_Relax * Tdetect,NR_Inter_Relax + Ncarrier_Non_relax  * Tdetect,NR_Inter. Cells which have been detected shall be measured at least every Ncarrier_Relax * Tmeasure,NR_Inter_Relax + Ncarrier_Non_relax  * Tmeasure,NR_Inter. The UE shall be able to evaluate that an already identified inter-frequency NR cell has met reselection criterion defined in TS 38.304 [1] within Ncarrier_Relax * Tevaluate,NR_Inter_Relax + Ncarrier_Non_relax  * Tevaluate,NR_Inter.

-	When T331 is running,

-	The parameter Ncarrier_Relax is the total number of NR inter-frequency carriers not configured for idle mode CA/DC measurements.

-	The parameter Ncarrier_Non_relax is the total number of NR inter-frequency carriers configured for idle mode CA/DC measurements.

-	When T331 is not running,

-	The parameter Ncarrier_Relax is the total number of inter-frequency carriers configured for mobility measurements only and the number of inter-frequency carriers configured for both mobility measurement and idle mode CA/DC measurements. -	The parameter Ncarrier_Non_relax =0.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7

Table 4.2.2.10.3-1: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x K1 x 1.5 (36 x N1 x K1 x 1.5) | 1.28 x N1 x K1 x 1.5 (4 x N1 x K1 x 1.5) | 5.12 x N1 x K1 x 1.5 (16 x N1 x K1 x 1.5) |
| 0.64 |  | 5 | 8 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 6 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |  |

<End of Change 1>