3GPP TSG-RAN WG4 Meeting #106	R4-2301893

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.6.6.1

Source:	Ericsson

Title:	UE demodulation and CSI reporting requirements for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #105 has been the kickoff of the performance WI for the enhanced 8Rx in FR1. As a first meeting, the discussions mainly focused on the physical channels’ definition and the CSI reporting to be considered [1]. In this paper, we continue providing our view on the remaining issues. In the following, we remind the agreements.

Duplex model - Focus on TDD as first priority - FFS on FDD pending on the progress in RF core requirements. TDD UL-DL pattern - 7D1S2U with S:6D+4G+4U Tx EVM value to be considered - QPSK/16QAM/64QAM: 6% - 256QAM: [3%] - 1024QAM: 2.5% Test scenario - Focus on single CC for PDSCH and SDR requirements as 1st priority - FFS on the CA requirements Whether to define demodulation requirements for PDCCH - FFS whether PDCCH requirements shall be specified for 8Rx - PDSCH shall be prioritized over PDCCH for initial simulation work. Whether to define demodulation requirements for PBCH Don’t introduce PBCH requirements

# 2	General aspects for UE demodulation and CSI reporting requirements

As general aspects to be considered [1], and to be more efficient and focus the efforts on relevant parameters, we propose to extend the test cases in FR1 for 8 Rx UE as follow.

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR1 with:

- TDD mode as a priority

- Number of receive antennas: 8Rx

- Modulation order: Up to 1024QAM

- Rank: Up to 8 DL MIMO layers.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- FR1 TDD n77/ n78: fc = 3.5 GHz

- FR1 TDD: SCS = 30 kHz, CBW = 40 MHz and 106 RBs.

- Channel models: TDLA and TDLC (30 ns delay spread)

- Max Doppler frequencies: up to 100 Hz (vehicular).

# 3	UE Demodulation Requirements

## 3.1 RF Parameters

In TS38.101-1 [2], it has been stated in Clause 7.2 that the UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination. In this WI, the UE will be equipped with 8 Rx, and the focus will be on TDD bands.

Proposal 3: Align the RF parameters to the RF enhancements in the core part, and consider as a starting point

- Tx EVM = 6% for up to 64QAM

- Tx EVM = 3% for 256QAM

- Tx EVM = 2.5% for 1024QAM

## 3.2 PDSCH demodulation requirements

The parameters specified in Table 5.2-1 [3] are valid for all PDSCH tests unless otherwise stated. However, to save efforts, it is preferrable to limit our requirements to PDSCH Type A.

Proposal 4: Define the 8 Rx UE demodulation requirements for PDSCH Type A only.

For 8 Rx UE, we can focus on 70% of the PDSCH peak throughput metric. Further discussions are needed to conclude whether 30% of peak throughput metric is needed or not.

Proposal 5: Define the 8 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

We summarize the needed parameters to define the PDSCH requirements in Proposals 6 and 7, respectively.

Proposal 6: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 3.5 |
| Subcarrier Spacing [kHz] | 30 |
| Channel bandwidth [MHz] | 40 |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA and TDLC (30 ns delay spread) |
| Antenna configuration | 2x8, 4x8, and 8x8 ULA Low |
| Velocity | 3 km/h, 30 km/h |
| PA Model | None |
| Pre-loaded Tx EVM | 6% for QPSK/16QAM/64QAM 3% for 256QAM 2.5% for 1024QAM |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4, Rank 6, and Rank 8 |
| DMRS Configuration | 2+2 DMRS symbols at (2,3,10,11) symbol indexes for Rank > 2 1+1DMRS symbols at (2, 11) symbol indexes for Rank = 2 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |

Proposal 7: Referring to the Table in Proposal 6, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow

FR1 TDD, 40 MHz/30 kHz, Rank > 4, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format an.d code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-4 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-5 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 20 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 4, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 2, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 2 | 2 | TDLC30-100 | 2x8, ULA Low | 70 | TBD |
| 3-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 7 | 2 | TDLC30-100 | 2x8, ULA Low | 70 | TBD |
| 3-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | TBD |
| 3-4 | 3.5 | 40 / 30 (106) | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

## 3.4	PDCCH demodulation

We propose to define PDCCH for 8 Rx UE with all possible aggregation levels 2, 4, 8, and 16 by reusing the existing test case specified in TS38.101-4 Clause 5.3.3.2 for 30 kHz in FR1.

Proposal 8: Define PDCCH demodulation requirements for 8 Rx UE in FR1 with the following test setup.

- FR1 TDD, 40 MHz/30 kHz

| Test num | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | 3 | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |

- Using the common test parameters

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 4: Non-interleaved Other tests: interleaved | Interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | 2 | 6 |  |
| Shift Index |  | 0 |  |  |

## 3.5	SDR

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs up to rank 4 for QPSK/16QAM/64QAM/256QAM, while up to rank 2 for 1024QAM.

Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 8 PDSCH MIMO layers using 8 Rx are considered.

According to Observation 1, we propose the following.

Proposal 9: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 27 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 2.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

# 4	CSI reporting requirements

RAN4 has defined CQI, PMI and RI reporting requirements for FR1 in Rel-15. In WF [1], it has been agreed

Whether define CQI requirements for 8RX - Introduce CQI requirements with rank 4 under static channel Test setup for CQI requirements for static conditions - 40 MHz/30 kHz, 106 RBs. - CQI index for up to 64QAM. - Rank 4. Whether define PMI requirements for 8RX - FFS whether to define PMI requirements Test setup for PMI requirements if agreed - Option 1: - 40 MHz/30 kHz - Propagation environment: TDLA30-5 - Single PMI- with 8 Tx Type I with single panel codebook and antenna configuration: High XP 8 x 8 with (N1, N2) = (4, 1) - Multiple PMI with 16 Tx Type II codebook and antenna configuration: Medium XP 16x8 with (N1, N2) = (4, 2) - Other options are not precluded. Whether to define RI requirements - FFS whether to define RI requirements

For 8 Rx UE, new requirements for PMI reporting could not be needed. We propose the following.

Proposal 10: Do not define new PMI reporting requirements for 8Rx UE in FR1.

In TS38.101-4 Clause 6.4.3.2, the reporting rank indicator has been limited to Rank 2 transmission even when considering 4 Rx. We believe that such a limitation will not change anything when considering 8 Rx. Therefore, we will not define RI reporting for 8 Rx.

Proposal 11: Do not define RI reporting for 8 Rx UE.

# 5	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR1 (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 5.1 | General | Updates on FR1 8Rx |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.2 | 2RX requirements |  |
| 5.2.3 | 4RX requirements |  |
| 5.2.4 | 8RX requirements | New section |
| 5.2.4.1 | FDD | Void |
| 5.2.4.2 | TDD | New section |
| 5.2.4.2.1 | Minimum requirements for PDSCH Mapping Type A | New section |
| 5.2A | PDSCH demodulation requirements for CA |  |
| 5.2A.4 | 8RX requirements | New section |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.1 | 1RX requirements |  |
| 5.3.2 | 2RX requirements |  |
| 5.3.3 | 4 RX requirements |  |
| 5.3.4 | 8RX requirements | New section |
| 5.3.4.1 | FDD | Void |
| 5.3.4.2 | TDD | New section |
| 5.5 | Sustained downlink data rate provided by lower layers | Updates on FR1 8 Rx |
| 5.5.1 | FR1 single carrier requirements | Updates on FR1 8 Rx |
| 5.5A | Sustained downlink data rate provided by lower layers |  |
| 5.5A.1 | FR1 CA requirements | Updates on FR1 8 Rx |

Table 2	Specification structure for UE CSI reporting requirements in FR1 (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 6.1 | General | Updates on FR1 8 Rx |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.4 | 8RX requirements | New section |
| 6.2.4.1 | FDD | Void |
| 6.2.4.2 | TDD | New section |
| 6.2A | Reporting of Channel Quality Indicator (CQI) for CA |  |
| 6.2A.5 | 8RX requirements | New section |
| 6.3 | Reporting of Precoding Matrix Indicator (PMI) |  |
| 6.3.4 | 8RX requirements | New section |
| 6.3.4.1 | FDD | Void |
| 6.3.4.2 | TDD | New section |

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR1 with:

- TDD mode as a priority

- Number of receive antennas: 8Rx

- Modulation order: Up to 1024QAM

- Rank: Up to 8 DL MIMO layers.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- FR1 TDD n77/ n78: fc = 3.5 GHz

- FR1 TDD: SCS = 30 kHz, CBW = 40 MHz and 106 RBs.

- Channel models: TDLA and TDLC (30 ns delay spread)

- Max Doppler frequencies: up to 100 Hz (vehicular).

Proposal 3: Align the RF parameters to the RF enhancements in the core part, and consider as a starting point

- Tx EVM = 6% for up to 64QAM

- Tx EVM = 3% for 256QAM

- Tx EVM = 2.5% for 1024QAM

Proposal 4: Define the 8 Rx UE demodulation requirements for PDSCH Type A only.

Proposal 5: Define the 8 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

Proposal 6: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 3.5 |
| Subcarrier Spacing [kHz] | 30 |
| Channel bandwidth [MHz] | 40 |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA and TDLC (30 ns delay spread) |
| Antenna configuration | 2x8, 4x8, and 8x8 ULA Low |
| Velocity | 3 km/h, 30 km/h |
| PA Model | None |
| Pre-loaded Tx EVM | 6% for QPSK/16QAM/64QAM 3% for 256QAM 2.5% for 1024QAM |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4, Rank 6, and Rank 8 |
| DMRS Configuration | 2+2 DMRS symbols at (2,3,10,11) symbol indexes for Rank > 2 1+1DMRS symbols at (2, 11) symbol indexes for Rank = 2 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). From MCS Table 2 (TS38.214): Up to MCS 20 (256QAM) |
| NOH | 0 |
| Number of HARQ Processes | 8 |

Proposal 7: Referring to the Table in Proposal 6, consider PDSCH test cases at 70% of the peak throughput for 40 MHz/30 kHz SCS as follow

FR1 TDD, 40 MHz/30 kHz, Rank > 4, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format an.d code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-4 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-5 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 20 | 6 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 4, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 4 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 13 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| 2-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM |  |  |  |  |  |  |  |  |

FR1 TDD, 40 MHz/30 kHz, Rank = 2, Metric: 70% Peak Throughput

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | 3.5 | 40 / 30 (106) | QPSK, 0.30 MCS 2 | 2 | TDLC30-100 | 2x8, ULA Low | 70 | TBD |
| 3-2 | 3.5 | 40 / 30 (106) | 16QAM, 0.48 MCS 7 | 2 | TDLC30-100 | 2x8, ULA Low | 70 | TBD |
| 3-3 | 3.5 | 40 / 30 (106) | 64QAM, 0.56 MCS 13 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | TBD |
| 3-4 | 3.5 | 40 / 30 (106) | 256QAM, 0.89 MCS 26 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | TBD |
| MCS were defined based on TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for QPSK/16QAM/64QAM Use Tx EVM 3% for 256QAM. |  |  |  |  |  |  |  |  |

Proposal 8: Define PDCCH demodulation requirements for 8 Rx UE in FR1 with the following test setup.

- FR1 TDD, 40 MHz/30 kHz

| Test num | Interleaver size | REG bundle size | CORESET RB | CORESET duration | Aggrega-tion level | Propagation condition | Antenna config | DCI format | Info Bit | SNR (dB) @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 102 | 1 | 2 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |
| 2 | 3 | 2 | 102 | 1 | 4 | TDLA30-10 | 1x8 ULA Low | 1_1 | 53 | New |
| 3 | 3 | 6 | 90 | 1 | 8 | TDLA30-10 | 2x8 ULA Low | 1_1 | 53 | New |
| 4 | NA | 6 | 48 | 2 | 16 | TDLA30-10 | 1x8 ULA Low | 1_0 | 41 | New |

- Using the common test parameters

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 4: Non-interleaved Other tests: interleaved | Interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | 2 | 6 |  |
| Shift Index |  | 0 |  |  |

Observation 1: RAN4 needs to extend the SDR applicability rule for FR1 where 8 PDSCH MIMO layers using 8 Rx are considered.

Proposal 9: Define SDR requirements applicable to FR1 CC, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 8’

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 6 | 1 | 27 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

- Update Table 5.5A-6 (1024QAM) to support 8Rx. Note the Maximum number of PDSCH MIMO layers is 2.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

Proposal 10: Do not define new PMI reporting requirements for 8Rx UE in FR1.

Proposal 11: Do not define RI reporting for 8 Rx UE.

# References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei.

- 3GPP TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, v17.7.0.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.7.0.

# Appendix	Simulation assumption

## A.1	PDSCH

### FR1

Please refer to TS.38.101-4 Clause 5.2.3.2.1 that will be updated.

Table 5.2.3.2.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 wideband for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 50, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9, 1-10, 1-11 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  | CSI-RS periodicity | Slots | Test 1-7, 1-10, 1-11: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-7, 1-10, 1-11: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  | Frequency Occupation |  | Test 1-7, 1-10, 1-11: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 16 for Test 1-4 10 for Test 1-9 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

## A.2	SDR

Table 5.5A-5: MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 1 | 8 | 1 | 26 |
| 1 | 8 | 0.8 | 21 |
| 1 | 8 | 0.75 | 20 |
| 1 | 8 | 0.4 | 11 |
| 1 | 6 | 1 | 27 |
| 1 | 6 | 0.8 | 23 |
| 1 | 6 | 0.75 | 22 |
| 1 | 6 | 0.4 | 14 |
| 1 | 4 | 1 | 16 |
| 1 | 4 | 0.8 | 16 |
| 1 | 4 | 0.75 | 16 |
| 1 | 4 | 0.4 | 10 |
| 1 | 2 | 1 | 9 |
| 1 | 2 | 0.8 | 9 |
| 1 | 2 | 0.75 | 9 |
| 1 | 2 | 0.4 | 4 |
| 2 | 8 | 1 | 26 |
| 2 | 8 | 0.8 | 21 |
| 2 | 8 | 0.75 | 20 |
| 2 | 8 | 0.4 | 11 |
| 2 | 6 | 1 | 27 |
| 2 | 6 | 0.8 | 23 |
| 2 | 6 | 0.75 | 22 |
| 2 | 6 | 0.4 | 14 |
| 2 | 4 | 1 | 16 |
| 2 | 4 | 0.8 | 16 |
| 2 | 4 | 0.75 | 16 |
| 2 | 4 | 0.4 | 10 |
| 2 | 2 | 1 | 9 |
| 2 | 2 | 0.8 | 9 |
| 2 | 2 | 0.75 | 9 |
| 2 | 2 | 0.4 | 4 |
| 4 | 8 | 1 | 26 |
| 4 | 8 | 0.8 | 23 |
| 4 | 8 | 0.75 | 22 |
| 4 | 8 | 0.4 | 12 |
| 4 | 6 | 1 | 27 |
| 4 | 6 | 0.8 | 24 |
| 4 | 6 | 0.75 | 23 |
| 4 | 6 | 0.4 | 14 |
| 4 | 4 | 1 | 16 |
| 4 | 4 | 0.8 | 16 |
| 4 | 4 | 0.75 | 16 |
| 4 | 4 | 0.4 | 11 |
| 4 | 2 | 1 | 9 |
| 4 | 2 | 0.8 | 9 |
| 4 | 2 | 0.75 | 9 |
| 4 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Table 5.5A-6: 1024QAM MCS indexes for indicated UE capabilities

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2RX | 1 | 10 | 1 | 23 |
|  | 1 | 10 | 0.8 | 21 |
|  | 1 | 10 | 0.75 | 19 |
|  | 1 | 10 | 0.4 | 9 |
| 4RX | 1 | 10 | 1 | 24 |
|  | 1 | 10 | 0.8 | 21 |
|  | 1 | 10 | 0.75 | 19 |
|  | 1 | 10 | 0.4 | 9 |
|  | 2 | 10 | 1 | 23 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

## A.3	CQI reporting in static condition

### A.3.1	Minimum requirement for CQI periodic reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.4.2.1.1-1: CQI reporting definition test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| andwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |