3GPP TSG-RAN WG4 Meeting #106	R4-2301895

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.8.4

Source:	Ericsson

Title:	UE demodulation and CSI reporting requirements for FR2 DL multi-Rx chain FR1

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on UE demodulation and CSI reporting requirements for FR2 4 layers DL multi-Rx chain UE. While RF and RRM are being discussed, in this paper, we will provide our views and which parameters to propose in this first meeting discussing the UE performance part. In RP-223527 [1], the objective for FR2 multi-Rx chain UE demodulation has been stated as follow.

### 4.2 Objective of Performance part WI - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - RRM Performance requirements and test cases corresponding to the new core requirements - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused - UE demodulation requirements: - Specify performance requirements for enhanced FR2-1 UEs supporting up to 4 DL MIMO layers with dual TCI with different QCL typeD on a single component carrier. - Demod requirements on up to 4 DL MIMO layers - CSI requirements to support up to 4-layer with 2 TCIs and Rel-17 mTRP Type I codebook - Extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined NOTE: − For the dual-TCI demodulation requirement case, focus on UEs supporting “simultaneousReceptionDiffTypeD-r16” UE capability, and for the 2- or 4-layer downlink MIMO reception, focus on UEs supporting the basic mTRP CSI reporting capability (FG 23-7-1/1b of NR FeMIMO).

# 2	General aspects for UE demodulation and CSI reporting requirements

As it has been stated in [1], this WI aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved UE demodulation performance. To define demodulation performance requirements for enhanced FR2-1 UEs supporting up to 4 DL MIMO layers on single component (CC), we consider receptions configured with 2 active TCI states, requiring beam reception from 2 directions (2 layers/direction). Dual TCI operation can be combined with the Rel-17 mTRP framework even if the base station is being deployed as a single TRP [1]. Accordingly, we will draw our views based on the following general aspects:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR2 DL multi-RX chain:

- TDD mode only

- Number of receive antennas: 4Rx

- Modulation order: Up to 64QAM

- Rank: Up to 4 DL MIMO layers.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- FR2-1 fc = 47 GHz

- FR2-1 TDD: SCS = 120 kHz, CBW = 100 MHz and 66 RBs.

- Phase noise @ 47 GHz

- Channel models: TDLA and TDLD (30 ns delay spread)

- Max Doppler frequencies: up to 300 Hz.

Proposal 3: RAN4 defines the UE demodulation and CSI reporting requirements for:

- Focus on demodulation performance requirements for Single Carrier.

# 3	UE Demodulation Requirements

## 3.1 RF Parameters

NR operations in FR2-1 band are susceptible to phase noise (PN) effects that should be assessed and addressed.

Proposal 4: As RF parameters, consider

- PN at UE side only.

- Tx EVM = 6% for QPSK/16QAM/64QAM.

For this WI, we can consider Rel-15 PTRS patterns for CP-OFDM.

Proposal 5: Consider Rel-15 PTRS pattern, K = 2, L = 1.

Proposal 6: Considering PN and Rel-15 PTRS patterns, define PDSCH demodulation and CSI reporting requirements using

- CPE compensation only.

- Consider performance degradation due to PN less than 1dB

## 3.2 PDSCH demodulation requirements

The parameters specified in Table 5.2-1 [3] are valid for all PDSCH tests unless otherwise stated. However, to save efforts, it is preferrable to limit our requirements to PDSCH Type A.

Proposal 7: Define the FR2 4 Rx UE demodulation requirements for PDSCH Type A only.

We can focus on 70% of the PDSCH peak throughput metric. Further discussions are needed to conclude whether 30% of peak throughput metric is needed or not.

Proposal 8: Define the FR2 4 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

For UL-DL pattern, we do not see the usefulness of defining a new pattern. We would prefer to stick to FR2.120-1 for PDSCH requirements’ definition.

Proposal 9: Define the FR2 4Rx multi-Rx chain UE demodulation requirements for PDSCH using FR2.120-1 TDD UL-DL pattern, given by DDDSU and S: 10D+2G+2U.

Considering m-DCI based transmission, we summarize the needed parameters to define the PDSCH requirements in Proposals 10 and 11, respectively.

Proposal 10: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 47 |
| Subcarrier Spacing [kHz] | 120 |
| Channel bandwidth [MHz] | 100 |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA30 |
| Antenna configuration | TRxP#1 2x4 ULA Low TRxP#2 2x4 ULA Low |
| Doppler spread | Up to 300 Hz |
| PA Model | None |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 4 |
| DMRS Configuration | 1+1 DMRS symbols |
| PTRS | K = 2, L = 1 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 19 (64QAM). |

Proposal 11: Referring to the Table in Proposal xx, consider PDSCH test cases at 70% of the peak throughput for 100 MHz/120 kHz SCS as follow

FR2 TDD, 100 MHz/120 kHz, Rank 4, Metric: 70% Peak Throughput

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | TBD | TBD | 100/120 | 16QAM, 0.64 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-2 | TBD | TBD | 100 / 120 | 64QAM, 0.50 | FR2.120-1 | TDLA30-75 | 2x4, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] |  |  |  |  |  |  |  |  |  |

## 3.3	PDCCH demodulation

We propose to not define PDCCH for FR2 4Rx UE since it will be decoded along with PDSCH.

Proposal 12: Do not define PDCCH demodulation requirements for FR2 DL 4 layers MIMO with dual TCI.

## 3.4	PBCH demodulation

SSB will be sent from one TRP. Thus, we believe that RAN4 does not need to define PBCH requirements.

Proposal 13: Do not define PBCH demodulation requirements for FR2 DL 4 layers MIMO with dual TCI.

## 3.6	SDR

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing Table 7.5A.1-3 has already covered the configuration applicable for UEs up to rank 2 for QPSK/16QAM/64QAM.

Proposal 14: Do not define SDR requirements for FR2-1 4Rx multi-Rx UE.

# 4	CSI reporting requirements

Rel-17 has introduced per-TRP CSI reporting and RAN4 only defined s-DCI-based PMI reporting test using TypeI-SinglePanel codebook. While this WI is defined for mDCI, the CSI reporting are received independently. Consequently, we do not see any need for CSI reporting requirements.

Proposal 15: Do not define the CQI reporting requirements for FR2 4Rx multi-Rx UE.

Proposal 16: Do not define PMI reporting requirements for FR2 DL 4Rx multi-Rx chain UE.

Proposal 17: Do not define RI reporting for FR2 DL 4Rx multi-Rx chain UE.

# 5	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for FR2 DL multi-RX chain:

- TDD mode only

- Number of receive antennas: 4Rx

- Modulation order: Up to 64QAM

- Rank: Up to 4 DL MIMO layers.

Proposal 2: RAN4 defines the UE demodulation and CSI reporting requirements with:

- FR2-1 fc = 47 GHz

- FR2-1 TDD: SCS = 120 kHz, CBW = 100 MHz and 66 RBs.

- Phase noise @ 47 GHz

- Channel models: TDLA and TDLD (30 ns delay spread)

- Max Doppler frequencies: up to 300 Hz.

Proposal 3: RAN4 defines the UE demodulation and CSI reporting requirements for:

Focus on demodulation performance requirements for Single Carrier.

Proposal 4: As RF parameters, consider

- PN at UE side only.

- Tx EVM = 6% for QPSK/16QAM/64QAM.

Proposal 5: Consider Rel-15 PTRS pattern, K = 2, L = 1.

Proposal 6: Considering PN and Rel-15 PTRS patterns, define PDSCH demodulation and CSI reporting requirements using

- CPE compensation only.

- Consider performance degradation due to PN less than 1dB

Proposal 7: Define the FR2 4 Rx UE demodulation requirements for PDSCH Type A only.

Proposal 8: Define the FR2 4 Rx UE demodulation requirements for PDSCH with 70% of the peak throughput metric only.

Proposal 9: Define the FR2 4Rx multi-Rx chain UE demodulation requirements for PDSCH using FR2.120-1 TDD UL-DL pattern, given by DDDSU and S: 10D+2G+2U.

Proposal 10: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 47 |
| Subcarrier Spacing [kHz] | 120 |
| Channel bandwidth [MHz] | 100 |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA30 |
| Antenna configuration | TRxP#1 2x4 ULA Low TRxP#2 2x4 ULA Low |
| Doppler spread | Up to 300 Hz |
| PA Model | None |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 4 |
| DMRS Configuration | 1+1 DMRS symbols |
| PTRS | K = 2, L = 1 |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 19 (64QAM). |

Proposal 11: Referring to the Table in Proposal xx, consider PDSCH test cases at 70% of the peak throughput for 100 MHz/120 kHz SCS as follow

FR2 TDD, 100 MHz/120 kHz, Rank 4, Metric: 70% Peak Throughput

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | TBD | TBD | 100/120 | 16QAM, 0.64 | FR2.120-1 | TDLA30-300 | 2x4, ULA Low | 70 | TBD |
| 1-2 | TBD | TBD | 100 / 120 | 64QAM, 0.50 | FR2.120-1 | TDLA30-75 | 2x4, ULA Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] |  |  |  |  |  |  |  |  |  |

Proposal 12: Do not define PDCCH demodulation requirements for FR2 DL 4 layers MIMO with dual TCI.

Proposal 13: Do not define PBCH demodulation requirements for FR2 DL 4 layers MIMO with dual TCI.

Proposal 14: Do not define SDR requirements for FR2-1 4Rx multi-Rx UE.

Proposal 15: Do not define the CQI reporting requirements for FR2 4Rx multi-Rx UE.

Proposal 16: Do not define PMI reporting requirements for FR2 DL 4Rx multi-Rx chain UE.

Proposal 17: Do not define RI reporting for FR2 DL 4Rx multi-Rx chain UE.

# References

- RP-223527, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.7.0.