3GPP TSG-RAN WG4 Meeting # 106	R4-2302162

# Athens, Greece, Februray 25th – March 3rd, 2023

Title: 	Updated simulation results for FR2-2 CQI reporting

Source: 	Huawei, HiSilicon

Agenda item:       6.2.6.1.3

Document for:	Discussion

Background

As per [1], RAN4 agreed to define FR2-2 CQI reporting requirements for Rank 1 and SNR Test point for high modulation order is still to be confirmed. In this paper we provide our simulation results and provide our views on test point.

Simulation results

In last meeting, high SNR test point is still to be confirmed:

SNR Test point for high modulation order - Agreement: [7/8] dB, companies can further check;

Simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD Slot Configuration |  |  |  | FR2.120-2 |  |  |  |
| SNRBB |  |  | dB | 0 | 1 | 7 | 8 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 1 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8.375 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| PN |  |  |  | Model set 2 |  |  |  |

Simulation results are captured in Figure 2-2. We can observe that median CQI is 9 for SNR=7dB and 10 for SNR=8dB which are reasonable and phase noise impact can be negligible under such conditions. We can accept this configuration.

Figure 2-2: Simulation results for CQI reporting

Proposal 1: Confirm higher SNR point to be [7, 8] dB

Conclusion

In this paper we updated simulation results for FR2-2 CQI requirements. The proposal is:

Proposal 1: Confirm higher SNR point to be [7, 8] dB

Reference

[1] R4-2220187 WF for FR2-2 UE demodulation. Qualcomm.