3GPP TSG-RAN WG4 Meeting #106	R4-2302333

Athens, Greece, Feb 27 – Mar 3, 2023

Agenda item:	9.10.2

Source: 	Qualcomm Incorporated

Title: 	On joint requirements for Rel-17 measurement gap enhancements

Document for:	Approval

# Introduction

RAN4 continued discussing joint RRM requirements for Rel-17 concurrent measurement gaps (MG), pre-configured measurement gaps and NCSG in RAN4#105. Further agreements and open issues were captured in a WF [1].

In this paper, we provide further proposals on joint requirements for the concurrent MG, pre-configured MG and NCSG within the context of Case 1 and Case 2 in the WID [2].

# Discussion

## Scope

Regarding Type-1 measurement gaps [3], we note that RAN4 requirements in Rel-17 do not fully support configuring them together with concurrent gaps. E.g. there is no explicit assignment of measurement objects to Type-1 MG. If RAN4 is going to spend time discussing this issue, there should be a clear motivation for adding new requirements.

Observation 1: There is no clear need to include Type-1 MG as part of the joint RRM requirements objective of this WI.

## Case 1

The first issue for Case 1 considers whether to support two pre-configured MG simultaneously in the same FR.

Issue 3-1-1: [Case 1] Whether to consider Pre-MG + Pre-MG in an FR

< Agreement >

- Narrow down options to Option 1 and 1a.

  - Option 1: Yes

  - Option 1a: Yes, with UE capability

In our view, two pre-configured MGs in the same FR would not be a typical configuration. It could be supported optionally with UE capability.

Proposal 1: Two pre-configured MGs in the same FR can be supported with UE capability.

Continuing with the theme of UE capabilities, let’s address the next issue below from RAN4#105. The question is whether to define a single capability for Case 1 or multiple capabilities to support different variants of Case 1. In the previous proposal, we indicated our preference for adding one UE capability to signal support of two pre-configured gaps. Since we do not consider that scenario to be typical on, it would be better to keep it under a separate capability rather than lumping it together with the more typical scenario of pre-configured MG + Type-2 MG. We support option 2 below.

Issue 3-1-2: [Case 1] Discussion on UE signalling capability

< Wayforward >

- FFS: Signalling capability shall be defined

- Option 1: A unified capability to indicate support of case 1, including Pre-MG + Type-2 MG and Pre-MG + Pre-MG

- Option 2: Two separate capabilities to indicate support of Pre-MG + Type-2 MG and Pre-MG + Pre-MG.

- Option 3: Others.

Proposal 2: Introduce two separate UE capabilities to indicate support of Pre-MG + Type-2 MG and Pre-MG + Pre-MG in Case 1.

The next two issues concern requirements for activation/deactivation of pre-configured MGs within Case 1.

Issue 3-1-3: [Case 1] Whether to support the following scenarios for Pre-MG + Pre-MG

< Wayforward >

- FFS: RAN4 should further study the activation/deactivation options for Pre-MG + Pre-MG

  - Option 1: Simultaneous multiple Pre-MGs activation/deactivation

  - Option 2: Non-simultaneous multiple Pre-MGs activation/deactivation

For the first issue above, we understand that RAN4 should discuss requirements for both cases. Our understanding is that option 2 proposes not to specify any requirements in case where two pre-configured MGs would be activated/deactivated at the same time. By simultaneous activation/deactivation, we understand that a status change of both gaps would be triggered by the same event or two events occurring at the same time. If that were the case, then Pre-MG + Pre-MG combinations may not be useable in practice. i.e. in general, simultaneous activation/deactivation may be hard to avoid with autonomous rules.

Proposal 3: RAN4 will discuss requirements for simultaneous and non-simultaneous activation/deactivation of pre-MGs in Case 1.

Issue 3-1-4: [Case 1] Whether to revisit the Con-MGs rules among the following Pre-MG status change

< Wayforward >

- Activation/activation

- Activation/deactivation

- Deactivation/deactivation

- Deactivation/activation

The second issue (3-1-4) also relates to status changes of two pre-configured MGs in the same FR. This issue relates also to dynamic collisions. It was agreed earlier by RAN4 that a pre-configured MG can cause collisions only when it is active. If the status of a pre-configured MG changes at some point in time, it may cause future gap instances of other concurrent gaps (Type-2 or another pre-configured MG) to be dropped or not to be dropped. The UE and the network need sufficient time to react and resolve such dynamic collisions. Note that UEs that do not support Case1 combinations with dynamic collisions will not be faced with these issues.

Since issue 3-1-4 is primarily about how to resolve dynamic collisions, we prefer to address it as part of issue 3-2-3.

Observation 2:  Issue 3-1-4[Case 1] “Whether to revisit the Con-MGs rules among the following Pre-MG status changes” is relevant for Case 1 combinations that cause dynamic collisions.

Moving on to the topic of collisions between gaps, it was agreed in RAN4#104-e to leverage the priority rule framework developed in Rel-17 [4]. However, in the most recent meeting there was still some discussion about whether to consider defining a gap sharing rule. RAN4 reached a further agreement not to consider a gap sharing rule between gaps that have been assigned different priorities [1]. Whether to consider gap sharing between gaps that have the same priority remains FFS.

## Issue 3-2-2: [Case 1] Whether to consider gap sharing rule

< Agreement >

- Gap sharing rules shall not be considered when the two gaps are with different priority.

< Wayforward >

- FFS whether RAN4 to consider the gap sharing rule when two gaps configured with equal priority.

  - TBD a deadline to cut off the discussion

During the Rel-17 MG_enh WI phase, RAN4 discussed several schemes for resolving collisions between measurement gaps, including a gap sharing rule. Ultimately the gap sharing rule was not adopted because potential benefits did not justify the higher complexity. In our view, unless clear, significant benefits are identified there is no motivation to discuss it again in Rel-18.

Proposal 4: RAN4 not to consider gap sharing rule to resolve collisions in Case 1 unless clear benefits are identified.

One important aspect of collisions between pre-configured MGs and other gaps (e.g. Type-2 MGs) is dynamic behavior that may occur as a result of changes in the activation status of pre-configured MGs. RAN4 reached an initial agreement regarding dynamic collisions in RAN4#105 [1].

Issue 3-2-4: [Case 1] dynamic collisions

< Wayforward/Agreement > FFS the following proposals

- Support of gap combinations including pre-configured MGs (Case 1) that cause dynamic collisions will be subject to new UE capability(ies).

  - FFS: Dynamic collisions are gap collisions involving a pre-configured MG, where gap instances of other MGs are dropped.

  - FFS: Gap combinations that cause dynamic collisions when at least one Pre-MGs with higher priority are involved in gap collision.

  - FFS: Gap combinations that does not cause dynamic collisions when at most one Pre-MG involved in the gap collision, and the Pre-MG is assigned the lowest priority level among all the colliding gaps.

  - FFS: Define separate UE capability for the scenario where pre-MG is colliding with the other component gap and pre-MG has higher priority

The initial agreement above states that Case 1 gap configurations that cause dynamic collisions will be subject to new UE capabilities. In our view, it would be useful to clarify what is meant by “dynamic collisions” before proceeding to discuss other FFS items in the above way forward.

Proposal 5: To facilitate RAN4 discussions of requirements for Case 1, adopt the following definition:

- Dynamic collisions are gap collisions involving at least one pre-configured MG, where gap instances of other MGs are dropped.

An example of dynamic collisions is shown in Figure 1. At the top of the figure, some instances of a lower-priority MG (purple) are dropped due to collisions with an active pre-configured MG. When the pre-configured MG is deactivated (bottom diagram), there are no collisions and no instances of the lower-priority gap are dropped. Any measurement objects mapped to the lower-priority gap would experience longer delay in the first case, where some of the gap instances are dropped. In the extreme case, all instances of the lower-priority MG could be dropped, resulting in long measurement delays (until the pre-configured MG is deactivated). All these conditions can occur without any change in the gap configuration.

Figure 1: Example of dynamic collisions

Dynamic collisions could occur for either Pre-MG + Type-2 MG or Pre-MG + Pre-MG combinations. As indicated above, we favor adding separate UE capabilities to support each of these combinations. According to the above agreement from RAN4#105, at least a third capability would be added to indicate whether the UE supports dynamic collisions for both of those gap combinations.

Proposal 6: Add a UE capability to indicate whether the UE supports Case 1 gap combinations that cause dynamic collisions. Details of the new capability can be FFS.

Issue 3-2-3: [Case 1] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion

< Wayforward >

- FFS the options

  - Option 1: Apple, Xiaomi

    - UE is allowed to drop the collided concurrent gap occasion, when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion.

  - Option 1a: Huawei

    - UE is allowed to drop the collided concurrent gap occasion, when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion.

      - Provided that concurrent gap is colliding with Pre-MG of higher priority.

  - Option 2: CATT

    - UE shall drop the activation procedure, when the pre-configured MG activation is overlapped with the other working (activated) component gap

  - Option 3: OPPO, Nokia, Intel

    - Not consider additional gap dropping due to the overlapping with Pre-MG activation procedure

  - Option 4: E///, Intel

    - RAN4 to define a clear UE behaviour to guarantee both NW and UE to understand whether data scheduling is expected within the Type-2 MG occasions during Pre-MG activation/deactivation period

  - Option 5: Xiaomi

    - UE perform the measurement on the overlapped concurrent MG occasion, and the pre-MG activation/deactivation delay is extended.

  - Option 6: vivo

    - The scenario where the activation procedure of multiple Pre-MG are overlapped needs be further configuration. Particularly on the user case under which kind of MO(s) association, after a BWP switch, multiple Pre-MG will be activated simultaneously

  - Option 7: E///

    - During Pre-MG activation/deactivation period, the gap dropping rule is invalid since NW doesn’t know the Pre-MG’s status

Issue 3-2-3 is generally about how to resolve dynamic collisions, including cases where there are status changes either a) close to the start of a Type-2 or active pre-configured MG instance, b) during a MG instance, or c) shortly after the end of a MG instance. Some examples of problematic scenarios are shown in Figure 2.

Before addressing this issue, it is worth clarifying that, in our view, measurement period requirement should not apply when there are changes in the status of pre-configured MGs in Case 1. This is already the case in Rel-17 for a single pre-configured MG. The main issue here is to clarify the gap interruptions that the network can expect. In this sense we agree with Option 4. However, option 4 does not propose a solution to the issue.

Figure 2: Examples of potential collisions due to changes in the activation/deactivation status of pre-configured MG.

Option 1/1a propose allowing a gap instance to be pre-empted (cut short after it has already started) if a pre-configured MG (of higher priority) is activated during the gap instance. Option 1/1a say “the UE is allowed to drop” the gap instance as opposed to “the UE shall drop.” Then we understand that the network will still assume an interruption during the full duration of the gap instance. This point would need to be clarified. Moreover, what if the activation happens very close (say 1 ms) before the start of the gap instance or shortly after the end of the gap instance? In the first case, does the UE have enough time to make the decision to drop the gap instance? In the second case, can a gap instance of the newly activated pre-configured MG start within 4 ms of the end of the previous gap instance? These corner cases should be addressed as well.

Observation 3: Option 1/1a in issue 3-2-3[Case1], it needs to be clarified whether the network would still assume an interruption lasting the full length of the on-going gap instance. Also, cases where a pre-MG gap is activated within 4 ms before the start or within 4 ms after the end of another concurrent gap instance should be addressed.

Option 2 proposes to drop the activation of a pre-configured MG if it overlaps is a gap instance of another concurrent MG. This proposal seems too conservative. It could cause unnecessarily long delays in the activation of a pre-configured MG.

Observation 4: Option 2 in issue 3-2-3[Case1] could cause unnecessarily long delays in the activation of pre-configured MGs.

Option 3 seems to suggest that the on-going gap instance would not be dropped. Therefore, interruption during the full duration of the on-going gap instance is expected. In that sense, it seems equivalent to option 1/1a.

Option 5 proposes to prioritize the on-going gap instance and measurement and delay the activation of the pre-configured MG. How long would activation be delayed? More details are needed to make this option a complete proposal.

Option 7 is not clear. Both the UE and the network should know the status of the pre-configured MG. The problem is how to address potential collisions with an on-going gap instance. The fact that a pre-configured MG is activated during a gap instance of another concurrent MG does not imply that there will be a collision; it depends on the starting time of the next pre-configured MG instance.

Our proposal below is to delay any changes in status of pre-configured MG in Case 1 to avoid potential collisions with a gap instance that is either already in progress, close to starting or recently ended.

Proposal 7: For Case 1 gap configurations:

- A collision between a change in the status of a pre-configured MG and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG.

- If a change in the status of a pre-configured MG collides with a gap instance, the change in status is delayed (until the end of the gap instance plus 5 ms) to avoid the collision.

Figure 3: Illustration of solution in Proposal 7

Lastly, new measurement requirements may need to be specified for MG combinations that fall under Case 1 [1].

## Issue 3-4-1: [Case 1] Measurement delay requirements

< Agreement >

- The options can be captured in high-level as below

  - The measurement requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF, Kp and Kgap and P factor

    - A scaling factor Kgap needs to be account for collisions with other measurement gaps

    - CSSF for each component gap is defined separately

  - FFS whether to consider additional applicability to the requirements.

In our view, RAN4 needs to address/clarify applicability of the requirements when the status of one or more pre-configured MGs change in Case 1. Note that Rel-17 already precludes applicability of some requirements when there is a change in the status of pre-configured MG.

For the K and P factors and for CSSF, the main issue would be dynamic collisions, as discussed previously. For example, consider the case of a measurement object mapped to a non-pre-configured concurrent MG (call it mgA). For gap combinations that do not include a pre-configured MG, Kgap is well-defined and static once the MG configuration is known. Now, if the gap combination includes a pre-configured MG that dynamically collides with mgA, then Kgap for that measurement object would change (Navailable changes) depending on the status of the pre-configured MG. Note that in this case the measurement delay would change even though the measurement object in question is not mapped to a pre-configured MG.

Similarly, CSSF may be affected by dynamic collisions because the calculation of CSSF needs to account for dropped gap instances.

The most straightforward way to address the issues above would be to specify that measurement requirements when Case 1 MG combinations are configured do not apply if a change in the status of any pre-configured MGs changes any of the requirement parameters during the measurement period. i.e. applicability is not precluded if a pre-configured MG is activated but does not collide with any other MG or RS.

Proposal 8: When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

## Case 2

Similar to Case 1, the first issue for Case 2 is whether to support two NCSGs simultaneously in the same FR.

Issue 4-1-1: [Case 2] Whether to consider NCSG + NCSG in an FR

< Agreement >

- Narrow down options to Option 1 and 3

  - Option 1: Yes. FFS whether the same RF chain is assumed for the two NCSG patterns.

  - Option 3: Yes, up to UE capability

In our view, two NCSGs in the same FR would not be a typical configuration. It could be supported optionally with UE capability. Regarding the FFS in option 1, the same RF chain cannot be assumed and there should not be a need for any assumption. Configuration of NCSG would be subject to UE signalling of NeedForGapNCSG-InfoNR-r17.

Proposal 9: Two NCSGs in the same FR can be supported with UE capability.

In addition, a separate UE capability should be added to signal support of NCSG + Type2 MG in Case 2.

Proposal 10: Introduce two separate UE capabilities to indicate support of NCSG + Type-2 MG and NCSG + NCSG in Case 2.

Moving on to the topic of collisions between gaps, it was agreed in RAN4#104-e to leverage the priority rule framework developed in Rel-17 [4]. However, in the most recent meeting there was still some discussion about whether to consider defining a gap sharing rule. RAN4 reached a further agreement not to consider a gap sharing rule between gaps that have been assigned different priorities [1]. Whether to consider gap sharing between gaps that have the same priority remains FFS.

Issue 4-2-2: [Case 2] Whether to consider gap sharing rule

< Agreement >

- Gap sharing rules shall not be considered when the two gaps are with different priority.

< Wayforward >

- FFS whether RAN4 to consider the gap sharing rule when two gaps configured with equal priority.

  - TBD a deadline to cut off the discussion

Similar to our observation for Case 1, RAN4 considered introducing a gap sharing rule during the Rel-17 MG_enh WI phase to resolve gap collisions and, ultimately, it was not adopted because potential benefits did not justify the higher complexity. In our view, unless clear, significant benefits are identified there is no motivation to discuss it again in Rel-18.

Proposal 11: RAN4 not to consider gap sharing rule to resolve collisions in Case 2 unless clear benefits are identified.

Additionally, RAN4 reached an agreement in RAN4#105 that confirms reusing the Rel-17 proximity condition to determine gap collisions in Case 2 [1]. In our view, RAN4 has a complete set of agreements to address collisions for Case 2.

Issue 4-2-1: [Case 2] Potential changes to Rel-17 proximity condition

< Agreement>

- Reuse Rel-17 proximity conditions for Case 2 (NCSG and Concurrent MG).

Observation 5: RAN4 has reached a complete set of agreements to address gap collisions in Case 2.

In RAN4#105, there was also some discussion about defining requirements for parallel measurements with two NCSGs configured in the same FR [1].

Issue 4-2-4: [Case 2] Potential changes to UE behaviour upon gap collision

< Wayforward >:

- FFS the following options

  - Option 1: RAN4 shall consider potential enhanced requirements on UE behaviour for collision handling in Case 2 (e.g. optimized/enhanced dropping rules).

  - Option 2: If the assumption is that UE cannot perform measurement on the two RF chains in parallel, there is no need to define further enhancement on gap collision handling except priority rule.

Our understanding is that this issue is more about asking the UE to perform measurements in parallel with two overlapping gaps. In that sense, the issue is not specific to Case 2; it applies to any concurrent measurement gap scenario. RAN4 did not define such requirements for Rel-17 concurrent MGs, hence the need to resolve collisions between gaps. The same direction is being followed for Case 1 in this WI. In our view, Case 2 should not be any different.

Proposal 12: RAN4 not to consider enhanced collisions allowing time overlap between two NCSGs or between an NCSG and other gaps (e.g. pre-configured MG, Type-2, etc.).

The last issue (4-3-1) below for Case 2 considers an optimization for measurements of deactivated SCells in a particular scenario where SSB is outside the active BWP. Note that this type of scenario could be handled adequately with a single NCSG. Our preference is not to pursue the optimizations proposed in options 1, 3 and 5.

Proposal 13: Do not pursue optimizations for deactivated SCell measurements with NCSG in Case 2.

Issue 4-3-1: [Case 2] Potential changes for de-activated SCell

< Wayforward >

- Background: When NW configures a NCSG and a Type-2 MG, the scenario for this deactivated SCell issue is as follow.

  - The deactivated SCell is measured within NCSG.

  - After MAC-based SCell activation, the deactivated SCell’s MO needs to be measured within MG if the related SSB is outside the active BWP

- FFS the options

  - Option 1: Apple

    - Introduce a new indication for supporting NCSG for deactivated SCell only. FFS potential changes to gap association

  - Option 2: Intel

    - NCSG can be associated with SCells to be activated/deactivated is indicated explicitly by “ncsgInd-r17”

  - Option 3: Intel

    - The NCSG can be configured as Pre-MG

  - Option 4: vivo, OPPO, Nokia, MTK

    - No change to Rel-17 rules

  - Option 5: Huawei, Ericsson

    - When the SCell is activated, the gap association is based on NW configuration

    - When the SCell is deactivated, the MO is implicitly associated to NCSG that is partially or fully overlapped with the SMTC

  - Option 6: ZTE

    - UE to be responsible for the capability report considering all possible MAC CE triggered NW configuration update, which is the most reliable and simple solution comparing with to discuss all potential implicit association rule

# Conclusions

Observation 1: There is no clear need to include Type-1 MG as part of the joint RRM requirements objective of this WI.

Proposal 1: Two pre-configured MGs in the same FR can be supported with UE capability.

Proposal 2: Introduce two separate UE capabilities to indicate support of Pre-MG + Type-2 MG and Pre-MG + Pre-MG in Case 1.

Proposal 3: RAN4 will discuss requirements for simultaneous and non-simultaneous activation/deactivation of pre-MGs in Case 1.

Observation 2:  Issue 3-1-4[Case 1] “Whether to revisit the Con-MGs rules among the following Pre-MG status changes” is relevant for Case 1 combinations that cause dynamic collisions.

Proposal 4: RAN4 not to consider gap sharing rule to resolve collisions in Case 1 unless clear benefits are identified.

Proposal 5: To facilitate RAN4 discussions of requirements for Case 1, adopt the following definition:

- Dynamic collisions are gap collisions involving at least one pre-configured MG, where gap instances of other MGs are dropped.

Proposal 6: Add a UE capability to indicate whether the UE supports Case 1 gap combinations that cause dynamic collisions. Details of the new capability can be FFS.

Observation 3: Option 1/1a in issue 3-2-3[Case1], it needs to be clarified whether the network would still assume an interruption lasting the full length of the on-going gap instance. Also, cases where a pre-MG gap is activated within 4 ms before the start or within 4 ms after the end of another concurrent gap instance should be addressed.

Observation 4: Option 2 in issue 3-2-3[Case1] could cause unnecessarily long delays in the activation of pre-configured MGs.

Proposal 7: For Case 1 gap configurations:

- A collision between a change in the status of a pre-configured MG and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG.

- If a change in the status of a pre-configured MG collides with a gap instance, the change in status is delayed (until the end of the gap instance plus 5 ms) to avoid the collision.

Proposal 8: When gap combinations including pre-configured MGs (Case 1) are provided to the UE, measurement requirements do not apply if the following parameters change during the measurement period due to changes in the status of any pre-configured MGs:

- Kp for intra-frequency and inter-frequency measurements without gaps

- Kgap for intra-frequency and inter-frequency measurements with gaps

- Kgap_EUTRA for inter-RAT measurements

- Kp_CSI-RS for CSI-RS L3 measurements

- Kp,PRS,i for NR positioning measurements

- CSSFintra for intra-frequency measurements

- CSSFinter for intra-frequency measurements

- CSSFinterRAT for intra-RAT measurements

- P scaling factor for L1-RSRP and L1-SINR measurements

Proposal 9: Two NCSGs in the same FR can be supported with UE capability.

Proposal 10: Introduce two separate UE capabilities to indicate support of NCSG + Type-2 MG and NCSG + NCSG in Case 2.

Proposal 11: RAN4 not to consider gap sharing rule to resolve collisions in Case 2 unless clear benefits are identified.

Observation 5: RAN4 has reached a complete set of agreements to address gap collisions in Case 2.

Proposal 12: RAN4 not to consider enhanced collisions allowing time overlap between two NCSGs or between an NCSG and other gaps (e.g. pre-configured MG, Type-2, etc.).

Proposal 13: Do not pursue optimizations for deactivated SCell measurements with NCSG in Case 2.

# References

[1] R4-2220359, WF on NR_MG_enh2 Part 1, RAN4#105.

[2] RP-222337, Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, RAN#97-e.

[3] R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, RAN4#104-bis-e.

[4] R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, RAN4#104-e.