3GPP TSG-RAN WG4 Meeting#106						R4-2302349

Athens, Greece,  27th Feb – 4th Mar, 2023

Source: 	Qualcomm Incorporated

Title: 	8Rx for CPE/FWA/vehicle/industrial devices

Agenda item:	9.6.6.1

Document for:	Discussion

- Introduction

In this paper, the scope of applicable scenarios and our views on demodulation performance requirements with 8 Rx antennas is discussed based on [1]. Demodulation performance requirements up to 4 Rx antennas in various scenarios are mentioned in [2]. The objective of the paper is to list scenarios that can be further discussed for specifications and others that can be omitted.

- Scenarios

2.1 PDSCH Requirements

In this section, the proposals are based on settings:

- TDD: 7D1S2U with S:6D+4G+4U

- SCS: 30KHz; BW: 40MHz

- TxEVM for QPSK/16QAM/64QAM: 6%

- Antenna configuration:

  - 8Layer: 8Tx, 8Rx

  - 4Layer: 4Tx, 8Rx

  - 2Layer: 2Tx, 8Rx

  - Correlation matrix: ULA

- 8L case: same MCS configuration for both codewords

- DMRS, Number of HARQ process, PDSCH, PDCCH symbols start and length as in R4-2220613 [1]

Based on our studies, proposals are:

2.1.1 	8Layers

Proposal 1: RAN4 to use MCS13 for 8 layers, 8Rx simulation assumptions

MCS13 achieves maximum throughput at less than 20dB of SNR which is close to the SNR limit with TxEVM of 6%.

Proposal 2: Don’t have a PDSCH grant in special slot for 8 layers, 8Rx

With only front loaded DMRS, channel estimation accuracy particularly for 8L case can get affected and degrade the 8L performance. Also, the resources in special slot are dominated by 2 PDCCH, 2 DMRS symbols and only 2 symbols are available for PDSCH. Moreover, this setting reduces PDSCH symbols by in a slot by nearly 5%. Hence, we are of the view to have no PDSCH grant in the special slot.

2.1.2	4Layers

Proposal 3: RAN4 to use MCS 17 for 4 layers, 8Rx simulation assumptions

The above MCS serves as a good candidate with reasonably high throughput to measure 4L demod performance. PDSCH grant allocation in special slot is not restricted in this case.

2.1.3 2Layers

Proposal 4: 2 Layers, 8Rx demod requirements are not essential.

We are of the view that the requirements discussion with 8Rx be focused more on spatial multiplexing gain since low layers such as 2 or below is like 1 layer with 4Rx whose performance is already part of [2]. Moreover, since high throughputs are of interest with CPE 8Rx devices, and throughput with 2L is lower than that with 4L or 8L at a given SNR, explicit specifications for 2 layers are not really required.

2.2 Sustained Data Rate (SDR)

Following proposals are with DMRS Type 1 with 2 + 2 symbols # 2,3,10, 11.

2.2.1 256 QAM 4Layers

Proposal 5: Propose maximum MCS 26 (scaling factor =1) for 256 QAM, 4 layers, 8Rx

Based on our studies, we propose to have 256 QAM 4L requirements be with MCS 26 and lower MCS.

2.2.2 CSI-RS for Tracking

Proposal 6: Symbol 4, 8 shall be locations of CSI-RS for tracking in 8 layers, 8Rx

For 4Rx case, CSI-RS for tracking OFDM symbols is specified in 6 and 10 [2]. With DMRS on symbol 10 for 8L case, the CSI-RS position needs to be changed.

2.2.3 NZP CSI-RS for CSI acquisition

Proposal 7: Choose row 6 or 7 of Table 7.4.1.5.3-1 of 38.211 specification [4] for 8 layers, 8Rx

With 8 layers, the CSI-RS for CS locations be changed from those for 4Rx case according to Table 7.4.1.5.2-1 of [4].

2.2.4 256 QAM 8Layers

Proposal 8: Propose maximum MCS 26 (scaling factor =1) for 256 QAM, 8 layers, 8Rx

Based on our studies, we propose to have 256 QAM 8L requirements with MCS 25 and lower MCS.

2.2.5 1024 QAM

Proposal 9: Further discuss 1024 QAM and corresponding TxEVM requirements

SDR requirements are primarily to verify achievable maximum throughput with the highest possible MCS.  With TxEVM @ 3% for 1024 QAM 8L, maximum throughput achievable with the highest MCS is hard to achieve. With 4L or 2L cases, the throughput achievable is less than that of 256 QAM with 8layers. Hence, we would like to further discuss tightening 1024 QAM TxEVM requirements.

2.3 Channel Type

Proposal 10: Do not define demod requirements for PDCCH, and PBCH

With 8Rx antenna ports, we think 4Tx or 8Tx antenna ports is a reasonable setting for performance specifications. With 4TX and 4Rx in Clauses 5.3, and 5.4 of [2], SNR thresholds in test cases for PBCH and PDCCH are low, e.g., with aggregation level 8 and 2Tx, the SNR requirement is close to cell detection side condition -6dB. Therefore, we don’t see a need to measure additional diversity gain with 8Rx for PBCH, PDCCH.

2.4 Reporting of CQI

Proposal 11: SNR points be (2,3) and (8,9) of Table 6.2.3.1.1.1-1 of [2] for 4L, 8Rx in static channel

Above proposal is assumes 4L in static channel conditions as mentioned in [1]. SNR pairs (5,6) and (11,12) are in Table 6.2.3.1.1.1-1 of [2] and SNR points be (2,3) and (8,9).

2.5 Reporting of Rank Indicator (RI)

In clause 6.4.3 of [2], only fixed rank 1 and 2 based tests are considered. Our view is to adopt LTE approach and not specify new 8Rx specific requirements.

Proposal 12: Follow LTE approach for Rank Indicator and do not specify new 8Rx requirements

2.6 Reporting of PMI

Proposal 13: Follow LTE approach for PMI reporting and do not specify new 8Rx requirements

- Conclusions

Following is the summary of the proposals.

Proposal 1: RAN4 to use MCS13 for 8 layers, 8Rx simulation assumptions

Proposal 2: Don’t have a PDSCH grant in special slot for 8 layers, 8Rx

Proposal 3: RAN4 to use MCS 17 for 4 layers, 8Rx simulation assumptions

Proposal 4: 2 Layers, 8Rx demod requirements are not essential

Proposal 5: Propose maximum MCS 26 (scaling factor =1) for 256 QAM, 4 layers, 8Rx

Proposal 6: Symbol 4, 8 shall be locations of CSI-RS for tracking in 8 layers, 8Rx

Proposal 7: Choose row 6 or 7 of Table 7.4.1.5.3-1 of 38.211 specification [4] for 8 layers, 8Rx

Proposal 8: Propose maximum MCS 26 (scaling factor =1) for 256 QAM, 8 layers, 8Rx

Proposal 9: Further discuss 1024 QAM and corresponding TxEVM requirements

Proposal 10: Do not define demod requirements for PDCCH, and PBCH

Proposal 11: SNR points be (2,3) and (8,9) of Table 6.2.3.1.1.1-1 of [2] for 4L, 8Rx in static channel

Proposal 12: Follow LTE approach for Rank Indicator and do not specify new 8Rx requirements.

Proposal 13: Follow LTE approach for PMI reporting and do not specify new 8Rx requirements

# References

[1] 3GPP WF R4-2220613, “8Rx UE demodulation and CSI requirements”

[2] 3GPP TS 38.101-4 version 17.5.0 “5G; NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”.

[3] 3GPP WID “Further RF requirements enhancement for NR frequency range 1 (FR1),” 3GPP TSG RAN meeting#95e, online, March 17 – 23, 2022.

[4] 3GPP TS 38.211 version 17.3.0 “5G; NR; Physical channels and modulation”.