3GPP TSG-RAN WG4 Meeting # 106	R4-2302350

# Athens, Greece, Feb 27 – Mar 03, 2023

Title: 	Discussion on new NR band n54 UE RF requirements

Agenda Item:	8.33.2

Source: 	MediaTek Inc.

Document for:	Approval

Introduction

Regarding the new approved WI of “New WID: Introduction of NR TDD band in 1670 – 1675 MHz” in RAN-Plenary#98e meeting, to define the UE RF TX and RX requirements for the new NR band n54 are needed. In the contribution, the potential UE RF TX and RX requirements are provided.

RF requirements

- 2.1	General analysis and view for each requirement

The table below provides analysis on the UE RF requirements for NR band n54. For each requirement, we indicate whether to reuse the existing band 54 requirements from TS 36.101. We also provide comments where some changes may be required with respect to TS 36.101.

Table 2.1-1

| Requirement name | Band-specific? | Reuse 36.101 requirement? | Comments |
| --- | --- | --- | --- |
| Clause 6: Transmitter Characteristics |  |  |  |
| UE maximum output power | No | Yes | 23dBm with +/-2dB tolerance |
| UE additional maximum output power reduction | Yes | No. See Comments | CBW of 5MHz; A-MPR(dB) is N/A Need to confirm whether Network Signalling NS_62 could be used. |
| Spurious emissions for UE co-existence | Yes | Yes. See Comments | Expect the requirements in TS 36.101 Table 6.6.3.2-1 as reference |
| Additional spurious emissions | Yes | Yes | Expect the same requirement in TS 36.101 Table 6.6.3.3.36-1 |
| 7: Receiver characteristics |  |  |  |
| Minimum requirements (QPSK) | No | Yes | Assume -100dBm/5MHz |
| In-band blocking | No | No | Expect the same requirement as majority NR bands in TS 38.101-1 Table 7.6.2-2 |
| Out-of-band blocking | No | No | Expect the same requirement as majority NR bands in TS 38.101-1 Table 7.6.3-2 |
| Narrow band blocking | No | No | Expect the same requirement as majority NR bands in TS 38.101-1 Table 7.6.4-1 |
| RX spurious emission | No | No | Expect the same general receiver spurious emission requirements in TS 38.101-1 Table 7.9-1 |

Conclusion

In the contribution, the potential UE RF requirements are provided.

Observation 1: Regarding NR band n54 UE RF requirements for TS 38.101-1, to leverage band 54 most RF requirements from TS 36.101 are feasible.

Proposal 1: To agree the proposal in the Table 2.1-1, or at least use them as the starting points for further discussions on NR band n54 UE TX/RX requirements for TS 38.101-1.

- References

[1] RP-223514, “New WID: Introduction of NR TDD band in 1670 – 1675 MHz”, Ligado Networks