3GPP TSG-RAN WG4 Meeting # 106	R4-2302383

# Athens, Greece, Feb 27 – Mar 03, 2023

Title: 	Discussion on 25MHz CBW REFSENS for NR band n8

Agenda Item:	8.26.2

Source: 	MediaTek Inc.

Document for:	Approval

Introduction

In RAN-Plenary#98e meeting, the revised basket WID [1] about “adding new channel bandwidth(s) support to existing NR bands” was approved. The revised basket WID requests to add additional 25MHz for NR band n8. In the contribution, the evaluation of 25 MHz CBW REFENS for NR band n8 is provided.

Discussion

As indicated in the revised WID [1], the justification about additional CBW for NR band n8 is as follows.

Band n8: 25 MHz for both BS and UE To fully utilize the licensed bandwidth of some operators on n8, 25MHz channel bandwidth are proposed to be added.

By studying the REFSENS about low band, it is observed that REFSENS degradation is affected by small duplex spacing between TX and RX. In order words, RX REFSENS is affected by TX interference. When TX interference is getting closer to RX, the impact or REFENS degradation becomes more significant.

Table: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode | Duplex Spacing (MHz) |
| --- | --- | --- | --- | --- |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD | 45 MHz |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD | 45 MHz |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD | 30 MHz |

The REFSENS for FDD low band n5, n8 and n12 from TS 38.101-1 are shown below for information and reference.

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 |  |  | -78.4 |  |  |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -78.5 |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |

Based on the evaluation, the proposed 25MHz CBW REFSENS for NR band n8 is -83.8dBm/25MHz and -83.9dBm/25MHz for 15kHz SCS and 30kHz SCS separately.

Proposal 1: Regarding the 25MHz CBW REFSENS of NR band n8, the REFSENS of 15kHz SCS and 30kHz SCS are -83.8dBm/25MHz and -83.9dBm/25MHz respectively.

Conclusion

In the contribution, the 25MHz CBW REFSENS for NR band n8 is provided.

Proposal 1: Regarding the 25MHz CBW REFSENS of NR band n8, the REFSENS of 15kHz SCS and 30kHz SCS are -83.8dBm/25MHz and -83.9dBm/25MHz respectively.

- References

[1] RP-222898, “Revised Basket WID: adding new channel bandwidth(s) support to existing NR bands”, Ericsson