3GPP TSG-RAN WG4 Meeting # 106	R4-2302524

# Athens, Greece, February 27 – March 3, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Band 5 and 26 protection aspect related to the new 900 MHz NR Band in the US

Agenda Item:	8.32.3	UE RF requirements	[NR_900MHz_US-Core]

Document for:	Approval

# Introduction

In RP#98e, the introduction of a new 900MHz band in the US was agreed upon and a new work item granted in [1]. This new band overlaps with band 8 and is only 2MHz above band 5 DL. In this contribution we discuss UE filter aspects in relation to band 5 and band 26 protection.

# Discussion

## Band n5 and n26 band protection aspects

The paired FDD spectrum allocated in 896-901 MHz (UL) and 935-940 MHz (DL) is introduced in WI [1]. The UL range is only 2MHz above the band 5 (869-894MHz) or 26 (859-894) DL which are also deployed in the U.S.

On the other end, band 26 and 5 UL are 86MHz away from the new band DL range. It is thus feasible for the band 5 and 26 duplexers, including in legacy UEs, to provide the 20dB rejection needed to grant a -50dBm/MHz protection level to the new band.

With only a 2MHz gap, it is not feasible to have any significant rejection in the band 5/26 DL upper range even with a dedicated filter and since the band is only 5MHz wide, without any filtering help, the SEM and spurious emissions requirements guarantees:

- -13dBm/MHz at 0.1-6MHz offset thus for the 4MHz at the top of band 5/26

- -25dBm/MHs at 6-10MHz offset thus for the next lower 4MHz

- Beyond 10MHz offset, the -30dBm/MHz requirement applies, but this is already the third adjacent channel equivalent range which already decays below -30dBm/MHz.

Given that a dedicated filter may not solve the issue for the upper band 5/26 range we suggest studying a solution that enables the support of the new band in UEs by implementing a band 8 duplexer, at least for small form factor UEs and to seek appropriate ways to manage the band 5/26 protection.

The possible techniques may involve:

- Compromise on band protection level for the upper part of the bands 5 and 26

- Use of specific RB allocation, improved TRX impairments or power reduction

- Any combination of the above.

Proposal 1: band 5 and 26 protect the new band with the default -50dBm/MHz protection level.

Proposal 2: Study solutions that enables the support of the new band small form factor UEs by implementing a band 8 duplexer at and seek for appropriate ways to manage the band 5/26 protection.

# Conclusions

In this contribution, we discussed band 5 and 26 coexistence aspect related to the new 900MHz NR U.S. band frequency range and make the following proposals.

Proposal 1: band 5 and 26 protects the new band with the default -50dBm/MHz protection level.

Proposal 2: Study solutions that enables the support of the new band small form factor UEs by implementing a band 8 duplexer at and seek for appropriate ways to manage the band 5/26 protection.

# References

- RP-223512 New WID on Introduction of 900 MHz NR Band in the US, Anterix, RP#98e