3GPP TSG-RAN WG4 Meeting #106	R4-2302535

# Athens, Greece, February 27th – March 3rd, 2023

# Title:	NTN Simulation Parameters for above 10 GHz Coexistence Studies

# Source:	THALES

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	9.25.2

Release	Rel-18

# Introduction

The following Satellite 5G NR NTN (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

From Article 5 of the ITU Radio Regulations (RR) the mobile operation of ESIMs is also described in the following footnotes,

- 5.527A The operation of earth stations in motion communicating with the FSS is subject to Resolution 156 (WRC-15). (WRC-15)

- 5.517A The operation of earth stations in motion communicating with geostationary fixed-satellite service space stations within the frequency bands 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) shall be subject to the application of Resolution 169 (WRC-19). (WRC-19)

At RAN4#105 the following WFs have been approved:

- R4-2220239 (source: THALES, “WF for above 10GHz band definition and system parameters”) with the latest references for Ka-Band frequency bands;

- R4-2220241 (source: Samsung, “Simulation assumptions for NTN co-existence study in bands above 10GHz”) with the latest discussions for the coexistence studies.

Agreements R4-2220239 1/ Companies are invited to study (general) ITU/national regulations and bring contributions/provide more information at this meeting (RAN#105) and next RAN4 meeting. 2/ Since it has been agreed to define several NTN Ka-bands to address the diversity of spectrum allocation (see agreement 15/11/2022 during online session), moderator proposes recommendation based on Option 2b. Option 2b (as starting point for next meeting discussion): RAN4 to consider defining/continue to discuss at next meeting (all options have same priority, companies need more time to check): - n511 with consideration of US/FCC regulations. - [n512 with consideration of CEPT regulations.] Note 1: All companies to further check impacts of latest revision of ECC Decision(05)01 on ECC Decision(13)01 cited in the current WF. - [n510 with consideration of US/FCC regulations.] Note 1: Provide the 3GPP definition that specifies the prevention of the use of a fixed terminal in the Fixed Service Spectrum (FSS). Note 2: Also provide information of a movable NTN user terminal in FSS spectrum in the US. with the following considerations: - DL: 17.7-20.2 GHz (n512, n511, n510); - UL: 27.5-30.0 GHz (n512), 28.35-30.0 GHz (n511), 27.5-28.35 GHz (n510).

Agreements R4-2220241 Table 1.1-1 Aggressor and victim combination No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; NTN UL; TN UL. 2; TN with NTN; TN UL; NTN UL. 3; TN with NTN; NTN UL; TN DL. 4; TN with NTN; TN DL; NTN UL. 5; TN with NTN; TN DL; NTN DL. 6; TN with NTN; NTN DL; TN DL. 7; TN with NTN; NTN DL; TN UL. 8; TN with NTN; TN UL; NTN DL. NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified..

This contribution therefore provides material for discussion with respect to Ka-Band NTN-TN coexistence simulations and NTN UE terminal and Satellite antenna parameters.

# Discussion with respect to Coexistence Use Cases

First of all, it seems that the easiest and most elegant way for the coexistence simulations is to consider scenarios with NTN DL in order to obtain relevant SAN (ACLR) and NTN UE (ACS) requirements, even if practically there is no TN band today in the close proximity of NTN DL. Therefore, in this contribution we therefore propose to update the coexistence scenarios/use cases (for “Co-existence study for above 10 GHz bands” [NR_NTN_enh-Core]) as follows:

Proposal 1: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands (compared to the agreed WF in R4-2220241) as follows:

Table 1.1-1 Aggressor and victim combination (in above 10 GHz)

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | i1, with fc=27GHz for simulation purposes |  |
| 2 | TN with NTN | TN UL | NTN UL | i2, with fc=27GHz for simulation purposes |  |
| 3 | TN with NTN | NTN UL | TN DL | i3, with fc=27GHz for simulation purposes |  |
| 4 | TN with NTN | TN DL | NTN UL | i4, with fc=27GHz for simulation purposes |  |
| 5 | TN with NTN | TN DL | NTN DL | i5, with fc=17GHz for simulation purposes |  |
| 6 | TN with NTN | NTN DL | TN DL | i6, with fc=17GHz for simulation purposes |  |
| 7 | TN with NTN | NTN DL | TN UL | i7, with fc=17GHz for simulation purposes |  |
| 8 | TN with NTN | TN UL | NTN DL | i8, with fc=17GHz for simulation purposes |  |
| NOTE 1: For coexistence between Ka-Band DL and surroundingadjacent TN bands, this need more discussions since currently there are no 3GPP defined/specified TN bands specified. |  |  |  |  |  |

Proposal 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

In fact, the purposes of the studies are as reflected as below, in the following proposal.

Proposal 3: RAN4 to update the coexistence simulations TDoc with the simulation scope mentioned below:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |  |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |  |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |  |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |  |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |  |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |  |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |  |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |  |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

# Discussion with respect to Satellite (SAN) parameters

## 3.1 Ka-Band Satellite Antenna pattern

The following normalised antenna pattern, corresponding to a circular aperture theoretical antenna pattern can be considered by RAN4 coexistence studies for SAN parameterization:

| Options | Equations |
| --- | --- |
| Option 1 | Fθ=2 J1uu |
| Option 2 | Fθ=232 J1uu+4 J2uu2 |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

The theoretical antenna pattern can be applied to a circular aperture, either for a passive antenna or for an active antenna. Figure 3 below shows an example with the normalised antenna pattern of a satellite transmit antenna as a function of u parameter with D/=333 (corresponding to e.g. D=5m and =0,015m).

Figure 3. Antenna pattern as a function of u parameter, with D/=333.

As seen in Figure 3, Option 2 has for instance lower adjacent lobes, which can benefit to the coexistence scenarios.

Moreover, the half-power beam-width (2θ-3dB) is given by the following relation(s):

| Options | 2θ-3dB Value |
| --- | --- |
| Option 1 | 2θ-3dB=Arcsin1.616×λπD |
| Option 2 | 2θ-3dB=Arcsin1.720×λπD |

which results in equivalent values.

## 3.2 Ka-Band SAN transmit and receive parameters

Satellite transmit and receive parameters with respect to satellite orbit are summarised in the tables below (Option 1 or Option 2 expressed above give similar results).

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Satellite altitude (km) | 35786 | 1200 | 600 |
| 3 dB beamwidth (°) | 0,18 | 1,76 | 1,76 |
| Satellite beam diameter at nadir (km) | 110 | 37 | 18 |

One can notice that the values are very similar with the ones from TR 38.821: 110, 40 and 20 km beam diameter respectively.

Moreover, the following set of SAN parameters are proposed for Ka-Band in DL (Set-1 from TR 38.821):

Ka-Band DownLink (i.e. ~20 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,0 | 0,5 | 0,5 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

In TR 38.821 one can find the following set of SAN parameters proposed for Ka-Band in UL:

Ka-Band UpLink from previous TR 38.821 (i.e. ~30 GHz for UL)

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,33 | 0,33 | 0,33 |
| G/T max (dB/K) | 28,0 | 13,0 | 13,0 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 5,9 | 0,9 | 0,9 |

Recent state of the art indicates lower NF values for Ka-band compared to the ones proposed in TR 38.821. In “Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload” (S. K. Jain and all), a value between 2.6-2.8 dB NF is considered. Therefore, in order to have more realistic NTN-TN coexistence simulations in Ka-band, we therefore propose to consider a 3.5 dB NF value for all satellite orbits/SAN types. This will have natural implications on the G/T values which have been updated accordingly in the table below.

Therefore, the following set of SAN parameters are proposed for Ka-Band in UL:

Ka-Band UpLink (i.e. ~30 GHz for UL) – [new proposal, more realistic]

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,33 | 0,33 | 0,33 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 3,5 | 3,5 | 3,5 |

An example of antenna pattern is shown in Figure 4 considering the following parameters:

| Parameter | Unit | Value |
| --- | --- | --- |
| Frequency | [MHz] | 20000 |
|  | [m] | 0,015 |
| D | [m] | 5,0 |
| Efficiency | N/A | 0,65 |
| Gmax | [dBi] | 58,5 |
| 2 -3dB | [°] | 0,19 |
| D/ | N/A | 333 |

Figure 4. Ka-Band transmit antenna pattern

RAN4 could consider the following normalised SAN antenna pattern options for Ka-Band coexistence studies:

| Options | Equations |
| --- | --- |
| Option 1 | Fθ=2 J1uu |
| Option 2 | Fθ=232 J1uu+4 J2uu2 |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

RAN4 could use for SAN NTN coexistence in Ka-Band the following half-power beam-width (2θ-3dB) given by the following relation(s):

| Options | 2θ-3dB Value |
| --- | --- |
| Option 1 | 2θ-3dB=Arcsin1.616×λπD |
| Option 2 | 2θ-3dB=Arcsin1.720×λπD |

If no strong concern from any company, the following option (Option 2) could be proposed for SAN parameters with respect to Ka-Band coexistence studies :

Fθ=232 J1uu+4 J2uu2

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

with the half-power beam-width (2θ-3dB) given by 2θ-3dB=Arcsin1.720×λπD.

Proposal 4: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB). Either Option 1 or Option 2 (optimised with respect to secondary lobes) can be used, with a preference for Option 2:

| Options | Equations | Half-power beam-width 2θ-3dB Value |
| --- | --- | --- |
| Option 1 (see TR 38.811) | Fθ=2 J1uu | 2θ-3dB=Arcsin1.616×λπD |
| Option 2 (new equation) | Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 5: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB):

| Equations (Option 2) | Half-power beam-width 2θ-3dB Value |
| --- | --- |
| Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 6: Moreover, the following set of SAN parameters are proposed for Ka-Band in DL (Set-1 from TR 38.821):

Ka-Band DownLink (i.e. ~20 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,0 | 0,5 | 0,5 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

Proposal 7: Moreover, the following set of SAN parameters are proposed for Ka-Band in UL (Updated Set-1 from TR 38.821):

Ka-Band UpLink (i.e. ~30 GHz for UL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,33 | 0,33 | 0,33 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Noise Figure (dB) of the receiver on board the satellite | 3,5 | 3,5 | 3,5 |

# Discussion with respect to UE NTN terminal parameters

## 4.1	General aspects

All NTN UE antenna parameters have been adapted from TR 38.821 by considering the following parameters adapted to Ka satellite band:

- Downlink frequency: 17.7 - 20.2 GHz

- Uplink frequency: 27.5 GHz - 30 GHz

- 60 cm antenna aperture diameter (2*a=60cm)

The Technical Report TR 38.811 provides typical RF parameters shown in the table below:

Table 1. Typical minimum RF characteristics of NTN UE

|  | Very Small Aperture NTN UE Terminal (fixed or mounted on moving platforms) |
| --- | --- |
| Transmit Power | 2 W (33 dBm) |
| Antenna type | 60 cm equivalent aperture diameter (circular polarisation) |
| Antenna gain | Tx: 43.2 dBi Rx: 39.7 dB |
| Noise figure | 1.2 dB |
| EIRP | 45.75 dBW |
| G/T (NOTE 1) | 18.5 dB/K |
| Polarisation (NOTE 2) | Circular |
| NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta - Noise Figure: NF in dB including feeder loss |  |

## 4.2	NTN UE antenna parameters

## 4.2.1	Parabolic antenna

The following normalised antenna pattern corresponding to a theoretical parabolic (reflector) antenna with circular aperture can be considered for coexistence analysis:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

FudB=10 logFu2

Figure 5 shows the antenna pattern of a NTN UE transmit antenna reflector 0.6 m diameter and  operating at  28750 MHz. Figure 5 therefore shows an example with parabolic/dish antenna of 0.6m diameter and  operating at  28750 MHz.

Figure 5. Antenna gain pattern of a NTN UE transmit parabolic antenna operating at 28750 MHz

## 4.2.2	Phased-array antenna

A phased-array antenna with a square aperture of side length D and tapered illumination over the aperture is considered. The antenna pattern at ϕ=0 is described by the following relation:

Fu=π22cosuπ22-u2

where:

- is the angle in a θ;ϕ spherical coordinates system,

- u=πDλsin⁡(θ-θ0),

- θ0 is the steering angle,

- : wavelength,

- D is the side length.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

Fθ,ϕdB=10 logFθ,ϕ2

The antenna gain can be evaluated with the following relation:

Gθ0=η4π×D2 λ2cosθ0

where η is the antenna efficiency.

Figure 6 shows the antenna pattern of a NTN UE transmit antenna with D=0.6 m, operating at 28750 MHz.

Figure 6. Antenna gain pattern of a NTN UE transmit phased array antenna operating at 28750 MHz

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

With respect to the adjacent antenna lobes (i.e. adjacent antenna lobes to the main directional lobes, or secondary antenna lobes from the antenna radiation pattern), the following recommendation can be used (see [ITU-R S.465-6]):

For information, for a 60cm diameter antenna in Ka-Band (λ=1cm), we have D/λ = 60.

If one compares the ITU-R S.465-6 limit with the one of NTN UE using parabolic/dish antenna (as seen in Figure 5) or using phased-array antenna (as seen in Figure 6), one will easily notice that both NTN UE antenna characteristics (i.e. using parabolic/dish antenna or using phased-array antenna) respect the ITU-R S.465-6 limit. Therefore, ITU-R S.465-6 limit can be used as reference for coexistence studies instead of complete antenna pattern with all the side lobes, which would require intensive simulations.

Proposal 8: RAN4 shall consider only parabolic/dish antenna NTN UE for coexistence studies (worst case).

Proposal 9: RAN4 shall use for coexistence studies the NTN UE antenna pattern as described by the following relation:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

FudB=10 logFu2

For information, for a 60cm diameter antenna in Ka-Band (λ=1cm), we have D/λ = 60.

Proposal 10: With respect to NTN UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-6:

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

Proposal 11: For secondary NTN UE lobes, RAN4 shall use ITU-R S.465-6 limit as reference for coexistence studies instead of complete antenna pattern with all the side lobes (which would require intensive simulations):

## 4.2.3 Transmit and receive performances

The NTN UE transmit and receive performances are summarised in the Table 2 considering a circular reflector antenna.

Proposal 12: RAN4 to select between [Option1] and [Option2] to define the NTN UE parameters:

[Option1] For the coexistence analysis in Ka-Band, RAN4 to use the following NTN UE parameters:

Table 2. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

NOTE1: T_a = T_Sky + T_Ground

NOTE2: The antenna temperatures are based on e.g. ITU-R Rec. P372 and Rec. P618.

NOTE3: T_sky is computed using [ITU-R Rec. P.618-12] as expressed below

In general, antenna efficiency depends on the antenna manufacturer design. Herein, the proposed values are indicative and consistent with Ka-band definition for UL and DL respectively.

Moreover, in order to simplify the definition of NTN UE, one can compute an equivalent receiver Noise Figure at ambient temperature, and therefore the second option could be defined as below.

[Option2] RAN4 to use the following NTN UE parameters:

Table 3. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

## 4.3 Link Budget Parameters

With respect to link budget parameters, is proposed that RAN4 should follow similar methodology as in TR 38.863, with the adaptations required for Ka-Band with channel models/link budget parameters described in TR 38.811 and TR 38.821.

# Conclusions

Proposal 1: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands (compared to the agreed WF in R4-2220241) as follows:

Table 1.1-1 Aggressor and victim combination (in above 10 GHz)

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | i1, with fc=27GHz for simulation purposes |  |
| 2 | TN with NTN | TN UL | NTN UL | i2, with fc=27GHz for simulation purposes |  |
| 3 | TN with NTN | NTN UL | TN DL | i3, with fc=27GHz for simulation purposes |  |
| 4 | TN with NTN | TN DL | NTN UL | i4, with fc=27GHz for simulation purposes |  |
| 5 | TN with NTN | TN DL | NTN DL | i5, with fc=17GHz for simulation purposes |  |
| 6 | TN with NTN | NTN DL | TN DL | i6, with fc=17GHz for simulation purposes |  |
| 7 | TN with NTN | NTN DL | TN UL | i7, with fc=17GHz for simulation purposes |  |
| 8 | TN with NTN | TN UL | NTN DL | i8, with fc=17GHz for simulation purposes |  |
| NOTE 1: For coexistence between Ka-Band DL and surroundingadjacent TN bands, this need more discussions since currently there are no 3GPP defined/specified TN bands specified. |  |  |  |  |  |

Proposal 2: RAN4 to update the NTN-TN coexistence scenarios for above 10 GHz bands with the following figures:

Figure 1. Coexistence scenarios for use cases 1-4 (in above 10 GHz)

Figure 2. Coexistence scenarios for use cases 5-8 (in above 10 GHz)

Proposal 3: RAN4 to update the coexistence simulations TDoc with the simulation scope mentioned below:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |  |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |  |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |  |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |  |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |  |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |  |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |  |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |  |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

Proposal 4: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB). Either Option 1 or Option 2 (optimised with respect to secondary lobes) can be used, with a preference for Option 2:

| Options | Equations | Half-power beam-width 2θ-3dB Value |
| --- | --- | --- |
| Option 1 (see TR 38.811) | Fθ=2 J1uu | 2θ-3dB=Arcsin1.616×λπD |
| Option 2 (new equation) | Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 5: RAN4 shall consider a normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB):

| Equations (Option 2) | Half-power beam-width 2θ-3dB Value |
| --- | --- |
| Fθ=232 J1uu+4 J2uu2 | 2θ-3dB=Arcsin1.720×λπD |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 6: Moreover, the following set of SAN parameters are proposed for Ka-Band in DL (Set-1 from TR 38.821):

Ka-Band DownLink (i.e. ~20 GHz for DL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 5,0 | 0,5 | 0,5 |
| Satellite EIRP density (dBW/MHz) | 40 | 10 | 4 |
| Satellite Tx max Gain (dBi) | 58,5 | 38,5 | 38,5 |

Proposal 7: Moreover, the following set of SAN parameters are proposed for Ka-Band in UL (Updated Set-1 from TR 38.821):

Ka-Band UpLink (i.e. ~30 GHz for UL) for different satellite orbits

| SAN parameters | GEO | LEO-1200 km | LEO-600 km |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture (m) | 3,33 | 0,33 | 0,33 |
| G/T max (dB/K) | 30,4 | 10,4 | 10,4 |
| Satellite RX max Gain (dBi) | 58,5 | 38,5 | 38,5 |
| Earth temperature (K) | 290 | 290 | 290 |
| Satcom Repeater Noise Figure (dB) | 3,5 | 3,5 | 3,5 |

Proposal 8: RAN4 shall consider only parabolic/dish antenna NTN UE for coexistence studies (worst case).

Proposal 9: RAN4 shall use for coexistence studies the NTN UE antenna pattern as described by the following relation:

Fu=2J1uu

where:

- J1x  is the Bessel function of first type and first order with argument x,

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ),

- D : antenna diameter,

- : wavelength.

The normalised antenna pattern, expressed in decibels, is given by the following relation:

FudB=10 logFu2

Proposal 10: With respect to NTN UE secondary lobes and related coexistence analysis, RAN4 to use the recommendation from ITU-R S.465-6:

The following reference radiation patterns should be adopted for angles between the direction considered and the axis of the main beam for frequencies in the range from 2 to 31 GHz:

G    32  –  25 log 		dBi		for  min          48°

  –10				dBi		for    48°        180°

where:

min    1° or 100 /D degrees, whichever is the greater, for D/ ≥ 50.

min =  2° or 114 (D/)–1.09 degrees, whichever is the greater, for D/ < 50.

Proposal 11: For secondary NTN UE lobes, RAN4 shall use ITU-R S.465-6 limit as reference for coexistence studies instead of complete antenna pattern with all the side lobes (which would require intensive simulations):

Proposal 12: RAN4 to select between [Option1] and [Option2] to define the NTN UE parameters:

[Option1] For the coexistence analysis in Ka-Band, RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power | (W) | 2 |  |
| Output power | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| EIRP |  | 44,9 |  |
| Receiver noise figure | (dB) |  | 1,2 |
| Feeder loss | (dB) |  | -0,50 |
| Sky temperature | (K) |  | 30 |
| Ground temperature | (K) |  | 10 |
| Antenna temperature | (K) |  | 40 |
| G/T figure of merit | (dB/K) |  | 16,5 |

[Option2] RAN4 to use the following NTN UE parameters:

Table x. NTN UE Parameters

| NTN UE Parameters |  | Tx (Uplink) | Rx (Downlink) |
| --- | --- | --- | --- |
| Polarisation |  | Circular | Circular |
| Low Frequency | (MHz) | 27 500 | 17 700 |
| Centre frequency |  | 28 750 | 18 950 |
| High frequency |  | 30 000 | 20 200 |
| Efficiency |  | 60% | 57% |
| On-axis antenna gain at Fc | (dBi) | 42,9 | 39,0 |
| Output power at antenna input | (W) | 2 |  |
| Output power at antenna input | (dBW) | 3,0 |  |
| Output loss | (dB) | -1,0 |  |
| Peak EIRP (on-axis) |  | 44,9 |  |
| Equivalent Receiver Noise Figure | (dB) |  | 2,1 |
| Feeder loss | (dB) |  | -0,50 |

# References

- R4-2220239, “WF for above 10GHz band definition and system parameters”, THALES.

- R4-2220241, “Simulation assumptions for NTN co-existence study in bands above 10GHz”, Samsung.

- R4-2213361, “Discussion on Ka adjacent band NTN-TN NR coexistence scenarios”.

- TR 38.811.

- TR 38.821.

- Ka-Band Low Noise Amplifier Sub-System Module for Communication Satellite payload”, S. K. Jain and all.

- ITU-R Rec. P.618-12.

- ITU-R Rec. S.465-6.

- R.C. Hansen “Phased Arrays” in “Antenna Engineering Handbook” by John L. Volakis, editor. McGraw Hill. 4th edition. 2007.

-END-