3GPP TSG-RAN4 Meeting #106	R4-2302598

# Athens, Greece, February 27th 2023 – March 3rd 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3040 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Efficient activation/de-activation mechanism for SCells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correction to the activation delay for fast SCell activation if the target SCell belongs to FR2 and is known. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Tuncertainty_MAC to the activation delay if the target SCell belongs to FR2 is known. Adding the definition of Tuncertainty_MAC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Will lead to imcomplete specification not aligned with legacy procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of 1st change >

### 8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell

Aperiodic CSI-RS resources can be configured for fast SCell activation. The requirements in this clause shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated. The requirements in this clause shall apply for the UE provided with aperiodic CSI-RS resources for SCell activation for the target SCell.

Note: If UE is allocated A-TRS for fast Scell activation, the UE is not required to use the SSB of the target Scell.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstATRS+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than [2400ms].

-	TFirstATRS + Tgap + TATRS + 5ms, if the measurement period of the SCell being activated is larger than [2400ms].

Note: The RSs on the activated serving cell in the same band are not required to be transmitted in the same slot as the temporary RS.

Note: UE may report inaccurate non-zero CQI for any activated Cell during the fast SCell activation procedure only if the RSs on the activated serving cell in the same band are not transmitted in the same slot as the aperiodic CSI-RS for fast SCell activation.

If the SCell is unknown and belongs to FR1, and SCell is contiguous to an active serving cell in the same band, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

-	TFirstATRS + Tgap + TATRS + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

Note: The RSs on the activated serving cell in the same band are not required to be transmitted in the same slot as the temporary RS.

Note: UE may report inaccurate non-zero CQI for any activated SCell during the fast SCell activation procedure only if the RSs on the activated serving cell in the same band are not transmitted in the same slot as the aperiodic CSI-RS for fast SCell activation.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstATRS+ 5ms provided:

-	The UE is provided with aperiodic CSI-RS resources for SCell activation for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell being activated fulfil the condition defined in clause 3.6.3,

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell.

-	SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is in FR1 or in FR2, and assuming PDCCH TCI and PDSCH TCI (when applicable) are associated with the triggered aperiodic CSI-RS resources for fast SCell activation, and when the following conditions are fulfillied:

-	One of the candidate TCI states configured in TCI-StatesPDCCH-ToAddList has the same QCL source of the triggered A-TRS,

-	The QCL source of CSI-RS for CQI reporting is the same as the triggered A-TRS,

-	The TCI state for PDCCH/PDSCH is the same as A-TRS remain unchanged during SCell activation,

-	then

-	If the target SCell belongs to FR2 is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(TFirstATRS + 2ms + Tuncertainty_MAC, Tuncertainty_SP), where Tuncertainty_SP=0 if UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

-	If the target SCell belongs to FR2 is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max(TFirstATRS + 5ms + Tuncertainty_MAC, Tuncertainty_RRC + TRRC_delay-THARQ).

where,

TFirstATRS: is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

TATRS is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

Tgap is a gap length between two aperiodic CSI-RS bursts,

-	2 slots for 15kHz and 30kHz

-	3 slots for 60kHz

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to SCell activation command.

Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to SCell activation command for known case.

TRRC_delay is the RRC procedure delay as specified in TS38.331 [2].

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to SCell activation command for known case, if the UE does not receive the SCell activation command and TCI state activation command simultaneously. Otherwise, Tuncertainty_MAC = 0.

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

-	the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	TFirstATRS, for any scenario where Tactivation_time includes TFirstATRS;

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.x apply provided that the SSB and A-TRS of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from slot n + THARQ + 3 ms where slot n is the slot where SCell activation command is received (as specified in clause 4.3 of TS 38.213 [3]) and until the SCell activation completion at UE, after at least one CSI-RS transmission occasion for the channel measurement and reporting (specified in clause 5.2.2.5 of TS 38.214 [26]), the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

< End of changes >