3GPP TSG-RAN WG4 Meeting RAN4#106      				R4-2302746 Athens, Greece

February 27 – March 3, 2022

Agenda Item:	9.6.3

Source:	Lenovo, Motorola Mobility

Title:	On ΔTRxSRS Measurement

Document for:  Approval

- Introduction

In TS 38.101-1, large transmit power relaxations are allowed when transmitting SRS from some ports, and this issue has come up again with respect to the case of 8RX antennas.  For some bands, relaxations as large as 6 dB have been proposed. Since [1] will only specify the maximum relaxation values, the actual relaxation values are unknown to the gNB.  Concerns have been expressed in [2-4] over the impact of these possibly large and unknown relaxation values on channel estimation and on the impact on downlink channel throughput.

In this contribution, we consider the impact of SRS power relaxations on channel estimation and the possible impact on channel throughput.  In order to enable the gNB to correct the channel estimate, the UE can signal these power relaxations to the network if known with sufficient accuracy. Alternatively, we consider the possibility that the gNB can determine the SRS relaxation values with the assistance of channel measurements taken by the UE.

- Impact of Power Relaxations on Channel Measurement

# Let the M x N matrix H denote the channel where Hij denotes the complex gain from the j-th UE transmit port to the i-th gNB receive port, the gNB has M ports, and the UE has N ports. Let the diagonal matrix R denote the transmit power relaxations taken by the UE when transmitting SRS.  Thus, we have

Rii= 10-ri/20

where ri denotes the actual transmit power relaxation taken by the i-th SRS port, and Rij=0 for i≠j.  The channel measured by the gNB is then given by

H= H R .

From this, the gNB infers that the downlink channel is given by

HT=RT HT  .

and uses this as the basis for selecting the rank, the precoding matrix, and the code rates for the downlink transmission.

If the power relaxations are known, the gNB can compensate the channel measurement for the transmit power relaxations as

HcompT=RT-1 HT=RT-1RT HT= HT

# However, while the gNB knows the maximum power relaxation that is allowed for each SRS port, it does not know the actual value (if any) of the power relaxation taken by the UE. If the gNB cannot accurately compensate the SRS transmit power relaxations, then downlink throughput may be negatively impacted (as in [2]) since the SRS-based channel estimates are used for precoder selection, rank determination and power allocation.

# An example can be used to illustrate the problem. We consider the simple case in which both the UE and the gNB have two antennas (or antenna ports).  A randomly selected channel was normalized so that the sum of the squares of the singular values is equal to 1. This first channel matrix is given by

# H=-0.2091-0.3524 i-0.2206+0.0328 i-0.3865-0.1292 i-0.7837-0.0456 i

# This same channel was then multiplied by the attenuation matrix R for the case that a 4.5 dB transmit power relaxation was taken for the SRS transmitted on the second antenna.  The resulting measured channel is then given by

# H= H R=-0.2091-0.3524 i-0.1314+.0195 i-0..3865-0.1292 i-0.4668-0.0272 i

# The water-pouring channel capacity was computed for the first and second channels as a function of signal-to-noise ratio with the results are given in Figure 1.  To compute capacity for the second channel, the following steps were taken

- The singular value decomposition of the matrix H= H R was used to determine the precoding vectors and the power allocation for the two layers;

- The precoding vectors were passed through the actual channel H;

- At the receiver, a zero-forcing receiver was used to separate the layers.

# For this example, it can be observed that there is a noticeable reduction of water-pouring channel capacity if the SRS-based channel estimate is not corrected.

Figure 1: Example of capacity reduction due to unknown 4.5 dB SRS relaxation

# No general conclusion can be drawn from any single example, including this one.  However, it may be useful to consider whether the SRS power relaxations, if unknown, can significantly impact the quality of the resulting downlink CSI.

# Throughput loss due to unknown SRS relaxations has also been demonstrated previously in [2]

# Observations 1:  Unknown SRS relaxations can result in a significant loss of downlink throughput.

# Observation 2:   If the SRS relaxations R are known to the gNB, the downlink channel estimate

# HT=RT HT

# can be corrected as

HcompT=RT-1 HT=RT-1RT HT= HT.

- Reporting SRS Relaxations

# As indicated in contributions [1], if the UE knows its SRS relaxations, these values can be reported to the gNB and the gNB can correct its channel estimate.

# Alternatively, the SRS relaxations can be determined by the network with the assistance of the UE as indicated in Figure 2 below.  As above, we consider the simplest case in which the UE and the gNB each have two antennas.  As above, let H denote the channel between the UE and the gNB given by

# H=H1,1H1,2H2,1H2,2

# where  Hij denotes the complex gain from the j-th UE antenna port to the i-th gNB antenna port.

Figure 2: Channel measurements taken by the gNB and UE

As indicated in Figure 2, the gNB uses the SRS transmitted by the UE to measure the channels α1 Hj,1 and α2 Hj,2.  The UE can then use any reference symbols transmitted from gNB antenna port j to measure the channels Hj,1 and Hj,2.  If the UE reports the measured ratio Hj,2 Hj,1 to the gNB, then the gNB can use its measured ratio α2Hj,2α1 Hj,1 to compute α2 α1, as

α2α1=α2Hj,2α1Hj,1 Hj,2Hj,1-1.

# It can be noted than any of the gNB antenna ports can be used to estimate α2 α1.  Alternatively, multiple gNB antenna ports can be used to form independent estimates of α2 α1 which can then be averaged.

# With the value of α2 α1, the relaxation matrix can be expressed as

# R=α1 100α2 α1 .

In the case of N UE antenna ports, the N x N relaxation matrix can be expressed as the diagonal matrix

# R=α11     0     ⋯00α2 α1⋯⋮⋮0      ⋮     ⋯⋱00αN α1 .

# If no relaxation is allowed for the first UE antenna port so that α1=1, then the matrix R is fully defined as

# R=1     0     ⋯00α2 α1⋯⋮⋮0      ⋮     ⋯⋱00αN α1

# Alternatively, if no relaxation is allowed for the j-th UE port, then the matrix R can be fully defined in terms of the j-th UE antenna port as

# R=α1 αj0     ⋯0      0 α2 αj⋯⋮   ⋮0      ⋮     ⋯⋱00αN αj .

# Depending on the accuracy with which the UE knows it relaxation values ΔTRxSRS, the measured values may be more or less accurate than the values reported by the UE.

# Proposal 1:  	If the UE has accurate knowledge of its relaxation values ΔTRxSRS, then these values should be signaled to the network.

# Proposal 2:  	If the UE does not have accurate knowledge of its relaxation values, then the UE should assist the gNB in determining these values with measurements.

- Summary

# In this contribution, we consider the possible impact of SRS transmit power relaxations on the channel measurement and the resulting quality of the downlink CSI.  While the maximum value of the transmit power relaxations is known, the actual values are not known to the gNB. In the provided example, it can be shown that these relaxations are not corrected at the gNB, then there can be significant negative impact on the quality of the downlink CSI extracted from these measurements and the resulting downlink throughput.

# Based on the above discussion, we have the following observations and proposals.

# Observations 1:  Unknown SRS relaxations can result in a significant loss of downlink throughput.

# Observation 2:   If the SRS relaxations R are known to the gNB, the downlink channel estimate

# HT=RT HT

# can be corrected as

HcompT=RT-1 HT=RT-1RT HT= HT.

# Proposal 1:  	If the UE has accurate knowledge of its relaxation values ΔTRxSRS, then these values should be signaled to the network.

# Proposal 2:  	If the UE does not have accurate knowledge of its relaxation values, then the UE should assist the gNB in determining these values with measurements

# References

- 3GPP TS 38.101-1 User Equipment radio transmission and reception; Part 1: Range1 Standalone v17.2.0

[2] R4-1813468, “SRS Switching loss impact to system performance”, Qualcomm Inc., RAN4 #88bis.

[3] R4-2216347, “8RX UE RF requirements”, Qualcomm, RAN4 #104bis-e.

[4] R4- 2216587, “On FR1 8Rx UE RF requirements”, Huawei, Hi Silicon, #104bis-e.

[5] R4-2218549, “On antenna virtualization and reporting delta TRxSRS”, Nokia, #105.