3GPP TSG-RAN WG4 Meeting # 106													     	R4-2303173

Athens, Greece, 27 February –03 March, 2023

Title: 	WF on NR FR2 HST Enhancements RRM requirements

Agenda Item:	9.12.5

Source: 	Samsung

Document for:	Approval

# Topic #1: Simultaneous multi-panel operation for train roof-mounted FR2 high power devices

## Sub-topic 1-1 Deployment scenario for FR2 HST multi-panel simultaneous reception

### Issue 1-1-1: Deployment to specify RRM requirement

Agreements:

- RAN4 to focus on bi-directional deployment in Scenario-A and Scenario-B to identify RRM requirement.

## Sub-topic 1-2 RF chains for FR2 HST multi-panel simultaneous reception

### Issue 1-2-1: HST FR2 RRM specific issues for simultaneous multi-panel operation

Agreement:

- RAN4 to follow Rel-17 NR FR2 HST for CPE configuration assumptions where the CPE UE is equipped with 2 panels each for both TX and RX pointing in opposite directions

Way forward:

- RRM requirement impact from HST FR2 for simultaneous multi-panel operation should only be focused on

    - Measurements

    - L1 measurement

    - Simultaneous data reception and L1 measurement

  - Scheduling restriction between simultaneous data reception and L1 measurement in multi-panel operation

- FFS other potential requirement impacts including, simultaneous data reception and L3 measurements, L1 and L3 simultaneous measurements and accordingly measurement/scheduling restriction,

  - FFS based on identifying potential gains or issues

### Issue 1-2-2: The configuration of supporting CA with multi-RX chains

Agreement:

- Rel-18 FR2 PC6 UE can be configured with multiple carriers even with multi-RX chains enabled for intra-band contiguous CA:

  - FFS intra-band non-contiguous CA.

Way forward:

- The configuration options are FFS:

    - Option 1: Rel-18 FR2 PC6 UE can be configured with multiple carriers even with multi-RX chains enabled, but multi-Rx chain is enabled only on one of the component carriers

    - Option 2: CPE specific issues.

    - Option 2-1: Rel-18 FR2 PC6 UE can be configured with multi-RX chains enabled on more than on component carrier.

    - Option 2-2: Multi-Rx chain is enabled always only on one of the CC (PCC) is baseline, on other CC (SCC) is optional

    - Option 3: For intra-band FR2 CA, multi-Rx chains are supposed to be enabled on all serving CCs, if intra-band CA with multi-Rx is supported

## Sub-topic 1-3 MRTD for FR2 HST multi-panel simultaneous reception

Agreement in Chair Notes:

- For Rel-18 PC6 UE supporting multi-panel simultaneous reception further investigate and, if needed, specify MRTD requirements for signals received on two panels on the same carrier

    - Case 1: MRTD < CP

    - Case 2: MRTD > CP

    - FFS whether the support of MRTD > CP is up to UE capability

    - FFS whether to define requirements for Case 1 and/or Case 2

## Sub-topic 1-4 Transmission Scheme for FR2 HST multi-panel simultaneous reception

Way forward:

- RAN4 to focus on NC JT as the main transmission scheme in Rel-18 HST FR2 enhanced deployments

## Sub-topic 1-5 UL and DL beam switching for FR2 HST multi-panel simultaneous reception

Way forward:

- FFS whether enhancements are needed in uplink spatial relation switch delay requirement.

  - Option 1: Keep current uplink spatial relation switch delay requirement

  - Option 2: RAN4 to discuss whether MAC-CE based spatial relation switch to the target associated to the unknown DL RS is possible in HST FR2 scenarios.

  - Option 3: The necessity of enhancement in uplink relation switch delay requirement is not in the scope of the WI

  - Other options are not precluded

    - Companies are encouraged to clarify the enhancement if other options are preferred.

## Sub-topic 1-6 RX beam sweep number for FR2 HST multi-panel simultaneous reception

Way forward:

- FFS RX sweep beam number reduction

## Sub-topic 1-7 Switch between multi-panel simultaneous reception and single-panel simultaneous reception

Way forward:

- FFS the necessity of signalling to UE used by the UE to switch between multi-panel and single-panel simultaneous reception with respect to NW configuration

# Topic #2: Intra-band carrier aggregation (CA) scenario

## Sub-topic 2-1 Impacted RRM requirement for CA

### Issue 2-1-1: Whether basic CA requirements should be further enhanced can be discussed after more clarification is achieved

Agreements

- RAN4 to specify intra-band CA RRM requirements in HST FR2 scenarios only for open space deployment

Way forward:

- FFS whether to specify CA RRM requirements for PC6 UEs capable of multi-panel operation

- FFS RAN4 to discuss what is the minimal set of UE functionalities need for the support of CA in HST FR2 scenario and defined requirements only for those.

- Other clarifications are not precluded

### Issue 2-1-2: Discussion on the necessity of the requirements

Way forward:

- For Rel-18 FR2 PC6, the necessity of the following requirements shall be FFS:

  - PSS/SSS detection, Time index detection for Intra-frequency and inter-frequency measurements

  - FFS IDLE mode in inter-frequency measurement requirements

  - Beam failure recovery on SCell

  - L1-RSRP measurement on SCell

  - TCI state switch in SCell

  - Active SCell measurement requirement (measurement period)

  - Other requirements are not precluded

## Sub-topic 2-2 Network signaling for Rel-18 FR2 HST CA Scenario

Way forward:

- HST signaling for CA enhancement:

  - Option 1: RAN4 postpones the agreement on network signaling until the CA requirements are defined

  - Option 2: Reuse  PCell signaling for SCell in HST

  - Option 3: Other options are not precluded