| 3GPP TR38.786 V0.0.1 (2023-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR Sidelink evolution; User Equipment (UE) radio transmission and reception; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	8

5	Operating bands and channel arrangement for SL evolution	9

5.1	Operating bands	9

5.1.1	Operating bands for single carrier operation in unlicensed band	9

5.1.2	Operating band combinations for inter-band con-current operation	9

5.2	Channel bandwidth	9

5.2.1	Channel bandwidth for single carrier operation in unlicensed band	9

5.2.2	Channel bandwidth for inter-band con-current operation	9

5.3	Channel arrangement	9

5.3.1	Channel raster	9

5.3.2	Synchronization raster	9

6	Transmitter characteristics for NR SL evolution	10

6.1	Tx requirements for NR SL single carrier operation in unlicensed band	10

6.1.1	Maximum output power for NR SL-U	10

6.1.2	UE maximum output power reduction for NR SL-U	10

6.1.3	UE additional maximum output power reduction for NR SL-U	10

6.1.4	Configured transmitted power for NR SL-U	10

6.1.5	Minimum output power for NR SL-U	10

6.1.6	Transmit OFF power for NR SL-U	10

6.1.7	ON/OFF time mask for NR SL-U	10

6.1.8	Power control for NR SL-U	10

6.1.9	Transmit signal quality for NR SL-U	10

6.1.10	Spectrum emission mask for NR SL-U	10

6.1.11	ACLR requirements for NR SL-U	10

6.1.12	Spurious emissions for NR SL-U	10

6.1.13	Spurious emission band UE co-existence for NR SL-U	10

6.1.14	Transmit intermodulation for NR SL-U	10

6.2	Tx requirements for inter-band con-current operation	11

6.2.1	Maximum output power for inter-band con-current operation	11

6.2.2	UE maximum output power reduction for inter-band con-current operation	11

6.2.3	UE additional maximum output power reduction for inter-band con-current operation	11

6.2.4	Configured transmitted power for inter-band con-current operation	11

6.2.5	Minimum output power for inter-band con-current operation	11

6.2.6	Transmit OFF power for inter-band con-current operation	11

6.2.7	ON/OFF time mask for inter-band con-current operation	11

6.2.8	Power control for inter-band con-current operation	11

6.2.9	Transmit signal quality for inter-band con-current operation	11

6.2.10	Spectrum emission mask for inter-band con-current operation	11

6.2.11	ACLR requirements for inter-band con-current operation	11

6.2.12	Spurious emissions for inter-band con-current operation	11

6.2.13	Spurious emission band UE co-existence for inter-band con-current operation	11

6.2.14	Transmit intermodulation for inter-band con-current operation	11

7	Receiver characteristics for NR SL evolution	11

7.1	Rx requirements for NR SL single carrier operation in unlicensed bands	11

7.1.1	Reference sensitivity power level for NR SL-U	11

7.1.2	Maximum input level for NR SL-U	11

7.1.3	Adjacent Channel Selectivity for NR SL-U	11

7.1.4	Blocking characteristics for NR SL-U	11

7.1.5	Spurious response for NR SL-U	11

7.1.6	Intermodulation characteristics for NR SL-U	11

7.2	Rx requirements for inter-band con-current operation	12

7.2.1	Reference sensitivity power level for inter-band con-current operation	12

7.2.2	Maximum input level for inter-band con-current operation	12

7.2.3	Adjacent Channel Selectivity for inter-band con-current operation	12

7.2.4	Blocking characteristics for inter-band con-current operation	12

7.2.5	Spurious response for inter-band con-current operation	12

7.2.6	Intermodulation characteristics for inter-band con-current operation	12

8	Co-channel coexistence between LTE Sidelink and NR Sidelink	12

Annex A: Change history	12

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for NR sidelink evolution services in Rel-18. The purpose is to specify radio solutions that are necessary for NR to support sidelink services working on unlicensed spectrums.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

3	Definitions of terms, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

DMRS	Demodulation Reference Signal

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

ITS	Intelligent Transportation System

LTE	Long Term Evolution

MPR	Maximum Power Reduction

NF	Noise Figure

NR	New Radio

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

ProSe	Proximity-based Services

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

4	Background

In Rel-16, sidelink communication was developed in RAN mainly to support advanced V2X applications. In Rel-17, SA2 studied and standardized Proximity based service including public safety and commercial related service. As part of Rel-17, power saving solutions (e.g., partial sensing, DRX) and inter-UE coordination have been developed in RAN1 and RAN2 to improve power consumption for battery limited terminals and reliability of sidelink transmissions.

Although NR sidelink was initially developed for V2X applications, there is growing interest in the industry to expand the applicability of NR sidelink to commercial use cases. For commercial sidelink applications, two key requirements have been identified:

- Increased sidelink data rate

- Support of new carrier frequencies for sidelink

Increased sidelink data rate is motivated by applications such as sensor information (video) sharing between vehicles with high degree of driving automation. Commercial use cases could require data rates in excess of what is possible in Rel-17. Increased data rate can be achieved with the support of sidelink carrier aggregation and sidelink over unlicensed spectrum. Furthermore, by enhancing the FR2 sidelink operation, increased data rate can be more efficiently supported on FR2. While the support of new carrier frequencies and larger bandwidths would also allow to improve its data rate, the main benefit would come from making sidelink more applicable for a wider range of applications. More specifically, with the support of unlicensed spectrum and the enhancement in FR2, sidelink will be in a better position to be implemented in commercial devices since utilization of the ITS band is limited to ITS safety related applications.

Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop.

5	Operating bands and channel arrangement for SL evolution

5.1	Operating bands

5.1.1	Operating bands for single carrier operation in unlicensed band

5.1.2	Operating band combinations for inter-band con-current operation

5.2	Channel bandwidth

5.2.1	Channel bandwidth for single carrier operation in unlicensed band

5.2.2	Channel bandwidth for inter-band con-current operation

5.3	Channel arrangement

5.3.1	Channel raster

5.3.2	Synchronization raster

6	Transmitter characteristics for NR SL evolution

6.1	Tx requirements for NR SL single carrier operation in unlicensed band

6.1.1	Maximum output power for NR SL-U

6.1.2	UE maximum output power reduction for NR SL-U

6.1.3	UE additional maximum output power reduction for NR SL-U

6.1.4	Configured transmitted power for NR SL-U

6.1.5	Minimum output power for NR SL-U

6.1.6	Transmit OFF power for NR SL-U

6.1.7	ON/OFF time mask for NR SL-U

6.1.8	Power control for NR SL-U

6.1.9	Transmit signal quality for NR SL-U

6.1.10	Spectrum emission mask for NR SL-U

6.1.11	ACLR requirements for NR SL-U

6.1.12	Spurious emissions for NR SL-U

6.1.13	Spurious emission band UE co-existence for NR SL-U

6.1.14	Transmit intermodulation for NR SL-U

6.2	Tx requirements for inter-band con-current operation

6.2.1	Maximum output power for inter-band con-current operation

6.2.2	UE maximum output power reduction for inter-band con-current operation

6.2.3	UE additional maximum output power reduction for inter-band con-current operation

6.2.4	Configured transmitted power for inter-band con-current operation

6.2.5	Minimum output power for inter-band con-current operation

6.2.6	Transmit OFF power for inter-band con-current operation

6.2.7	ON/OFF time mask for inter-band con-current operation

6.2.8	Power control for inter-band con-current operation

6.2.9	Transmit signal quality for inter-band con-current operation

6.2.10	Spectrum emission mask for inter-band con-current operation

6.2.11	ACLR requirements for inter-band con-current operation

6.2.12	Spurious emissions for inter-band con-current operation

6.2.13	Spurious emission band UE co-existence for inter-band con-current operation

6.2.14	Transmit intermodulation for inter-band con-current operation

7	Receiver characteristics for NR SL evolution

7.1	Rx requirements for NR SL single carrier operation in unlicensed bands

7.1.1	Reference sensitivity power level for NR SL-U

7.1.2	Maximum input level for NR SL-U

7.1.3	Adjacent Channel Selectivity for NR SL-U

7.1.4	Blocking characteristics for NR SL-U

7.1.5	Spurious response for NR SL-U

7.1.6	Intermodulation characteristics for NR SL-U

7.2	Rx requirements for inter-band con-current operation

7.2.1	Reference sensitivity power level for inter-band con-current operation

7.2.2	Maximum input level for inter-band con-current operation

7.2.3	Adjacent Channel Selectivity for inter-band con-current operation

7.2.4	Blocking characteristics for inter-band con-current operation

7.2.5	Spurious response for inter-band con-current operation

7.2.6	Intermodulation characteristics for inter-band con-current operation

8	Co-channel coexistence between LTE Sidelink and NR Sidelink

Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-02 | RAN4#106 |  |  |  |  | Initial Skeleton | 0.0.1 |