3GPP TSG-RAN WG4 Meeting #106	R4-2303626

Athens, Greece, 27 February – 3 March 2023

Source:	Ericsson, Rogers

Title:	TP for TR 37.718-11-11: DC_71A_n7A

Agenda item:			8.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-11-11 to include DC_71A_n7A as defined in WID [1].

# ---Start of changes---

## 6.1	Inter-band DC within FR1

### 6.1.X	DC_71_n7

#### 6.1.X.1	Configuration for DC

Table 6.1.1.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_71A_n7A | DC_71A_n7A | No |

#### 6.1.X.2	Maximum output power for DC

Table 6.1.1.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_71A_n7A | 23 | +2/-3 |

#### 6.1.X.3	Spurious emission band UE co-existence for DC

Table 6.1.X.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_71A_n7A | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 6.1.X.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.X.4-1

Table 6.1.X.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1326 | 1396 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1802 | 3163 | 3268 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1244 | 1104 | 4302 | 4477 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3826 | 3966 | 5663 | 5838 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3814 | 3604 | 6326 | 6536 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 581 | 406 | 6802 | 7047 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4489 | 4664 | 8163 | 8408 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9617 | 9302 | 292 | 82 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10663 | 10978 | 5152 | 5362 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6384 | 6104 | 2906 | 3151 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8826 | 9106 | 6989 | 7234 |

Based on Table 6.1.X.4-1, 4th order harmonic of Band 71 Tx may fall into the own Rx Band of Band n7.

MSD values due to 4th order harmonic for DC_71_n7 are reused from DC_7_n71

Table 6.1.X.4-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 71 | n76,7 | 14.6 | 11.7 | 10.1 | 9 | 8 | 7.1 | 6.3 | 5.5 |  | 4 |  |  |  |  |
| NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.X.4-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 Hz | 25 MHz | 30 MHz | 35 MH | 40 MHz | 45 MH | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 71 | n7 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |

#### 6.1.X.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_7_n71 as specified in TS 38.101-1.

Table 6.1.X.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_71_n7 | 0.6 | 0.3 |

Table 6.1.X.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_71_n7 | 0.2 | 0 |

---End of changes---

# Reference

[1] 	 RP-223078, Rel-18 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL), CHTTL