3GPP TSG-RAN WG4 Meeting #106	R4-2303705

Athens, Greece, 27 February ‒ 04 March, 2023

Title: 	WF on power Class indications in TS 38.101-1 and related signaling

Agenda Item:	5.1.2

Source: 	Qualcomm Inc.

Document for:	Approval

# Interplay of power class related RRC parameters

Issue to solve: What is the correct way to interpret power class related RRC parameters so that minimum requirements are unambiguous. The scenarios under consideration include: a) dual-band UL CA; b) DL-CA only with single-carrier UL.

Options for further discussion:

- Option 1: The following rules apply as a package:

  - By default, UE shall meet the power class indicated by ue-PowerClass for each NR band of the CA configuration.

  - Power class of a band in band combination cannot be higher than the power class of the band combination, i.e. for a band in band combination power class min(ue-PowerClass, PowerClass) applies. If PowerClass is not indicated default power class applies to the band combination.

  - ue-PowerClassPerBandPerBC-r17 can be used to indicate lower power class for a band in band combination than given by the earlier rules. This does not change the power class of the band combination, i.e. PowerClass. If ue-PowerClassPerBandPerBC-r17 is not signalled the earlier rules apply.

- Option 2:

  - Other views not precluded

# Capturing interplay of power class related RRC parameters

Issue to solve: Whether/where to capture to interplay of power class related RRC parameters

Agreements:

- It is necessary to capture in RAN4 RF specification

Open issues:

Issue #1: Where to capture:

-	Option 1: Capturing this in configured maximum transmission power is sufficient

-	Option 2: It is necessary to clarify in power class requirement clause 6.2A

-   Option 3: Both option 1 and 2 are needed

-	Other options are not precluded

Issue #2: From which release:

- Option 1: Rel-16

- Option 2: Rel-17

- Option 3: Rel-18

# Capturing that MSD has been analysed for a HPUE configuration

Issue to solve: How to indicate that MSD has been analysed for a given HPUE configuration and avoid a case where all CA combinations/configurations have increased maximum power whenever a new band has higher maximum power

Agreements:

- It is necessary to capture in RAN4 RF specifications

- Capture this information in CA configuration table in clause 5 by re-wording of the related notes