| 3GPP TR 38.872 V0.4.0 (2023-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on enhancement for 700/800/900MHz band combinations (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	8

4.1	TR Maintenance	8

5	Specific Band combinations	9

5.1	CA_n5-n8	9

5.1.0	General	9

5.1.1	UE RF architecture assumption	9

5.1.2	Common for 1 band UL and 2 bands UL of CA_n5-n8	14

5.1.2.1	Operating bands for CA	14

5.1.2.2	Channel bandwidths per operating band for CA	14

5.1.2.3	UE co-existence studies	14

5.1.2.4	∆TIB,c and ∆RIB,c values	15

5.1.2.5	REFSENS evaluation	15

5.1.3	Specific for 2 bands UL of CA_n5-n8	15

5.1.3.0	General	15

5.1.3.1	Maximum output power for inter-band CA	16

5.1.3.2	UE co-existence studies	16

5.1.3.3	REFSENS evaluation	17

5.1.4	Feasible RF implementations	17

5.2	CA_n5-n28	18

5.2.1	UE RF architecture assumption	18

5.2.2	Common for 1 band UL and 2 bands UL of CA_n5-n28	20

5.2.2.0	General	20

5.2.2.1	Operating bands for CA	20

5.2.2.2	Channel bandwidths per operating band for CA	21

5.2.2.3	UE co-existence studies	21

5.2.2.4	∆TIB,c and ∆RIB,c values	21

5.2.2.5	REFSENS evaluation	22

5.2.3	Specific for 2 bands UL of fallback CA_n5-n28	22

5.2.3.1	Maximum output power for inter-band CA	22

5.2.3.2	UE co-existence studies	22

5.2.3.3	REFSENS evaluation	24

5.3	CA_n8-n20-n28	27

5.3.1	UE RF architecture assumption	27

5.3.1.0	General	27

5.3.1.1	UE RF architecture with three antennas	27

5.3.1.2	UE RF architecture with two antennas	27

5.3.1.3	UE RF architecture with four antennas	28

5.3.2	CA_n8-n20	29

5.3.2.0	General	29

5.3.2.1	Operating bands for CA	29

5.3.2.2	Channel bandwidths per operating band for CA	29

5.3.2.3	UE co-existence studies	29

5.3.2.4	∆TIB,c and ∆RIB,c values	29

5.3.2.5	REFSENS requirements	29

5.3.2.6	Maximum output power for UL inter-band CA	29

5.3.2.7	UE co-existence studies for UL inter-band CA	29

5.3.2.8	REFSENS evaluation for UL inter-band CA	31

5.3.3	CA_n8-n28	31

5.3.3.0	General	31

5.3.3.1	Operating bands for CA	31

5.3.3.2	Channel bandwidths per operating band for CA	32

5.3.3.3	UE co-existence studies	32

5.3.3.4	∆TIB,c and ∆RIB,c values	32

5.3.3.5	REFSENS requirements	32

5.3.3.6	Maximum output power for UL inter-band CA	32

5.3.3.7	UE co-existence studies for UL inter-band CA	32

5.3.3.8	REFSENS requirements for UL inter-band CA	34

5.3.4	CA_n20-n28	34

5.3.4.1	General	34

5.3.5	CA_n8-n20-n28	35

5.3.5.1	Operating bands for CA	35

5.3.5.2	Channel bandwidths per operating band for CA	35

5.3.5.3	∆TIB,c and ∆RIB,c values	35

5.3.5.4	UE co-existence studies for UL inter-band CA	36

5.3.5.5	REFSENS evaluation	39

Annex A: Change history	41

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is the technical report for the study on enhancement for 700/800/900MHz band combinations under R18 time frame. The purpose is to gather the relevant background information and studies in order to address the study on enhancement for 700/800/900MHz band combinations in Table 1-1.

Table 1-1: Band combinations in the SI

| Configuration | Uplink configuration |
| --- | --- |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 |
| CA_n5-n8 | CA_n5-n8 |
| CA_n5-n28 | CA_n5-n28 |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-221464, “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, RAN#96

[3]	TS 38.101-1

[4]	TS 38.101-3

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

<defined term>: <definition>.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

In RAN#95e, SI “enhancement for 700800900MHz band combinations” was approved. The objectives of SI are as follows:

1	Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands and simultaneous reception on two or three bands for the band combination of 700, 800 and 900MHz spectrum for smart phone form factor

2	The following band combinations will be considered. And the feasibility study on three band combination will start after the completion of feasibility study of all the fallback band combinations.

-	CA_n8-n20-n28 with uplink configurations of CA_n8-n20, CA_n8-n28, CA_n20-n28, and the fallback modes

-	CA_n5-n8 with uplink configuration of CA_n5-n8, and the fallback modes

Note1: Spectrum restrictions should be studied to solve overlap of band n5 downlink and band n8 uplink

Note2: The current filter is used as the baseline. Further study whether or not to have new solutions.

-	CA_n5-n28 (full range) with uplink configuration of CA_n5-n28.

Summary of band combinations considered in the SI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom |
| CA_n5-n8 | CA_n5-n8 | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |

The following aspects need be studied

-	UE architecture including n-plexing, PA

-	Study feasibility of low band wideband antenna

-	Performance due to impacts including inter-modulation products

-	Method to manage the inter-modulation product impacts

Note: Revisit in RAN#98 whether additional aspects need to be added.

Power class 3 (PC3) is considered in this study

3) Identify potential impacts to relevant RAN4 requirements.

The present document is the technical report for this Study Item.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor.

5	Specific Band combinations

5.1	CA_n5-n8

5.1.0	General

Generally, in this study item, some high level implementations were discussed and shown as below.

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

Option 1: Only support 1UL/2DL CA. Single UL in n5

Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

Note: Potential impacts on RAN2 are observed

2) Dedicated RF filters implementation with partial frequency range

Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

5.1.1	UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8: 2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

Due to the frequency range overlap between n5 DL and n8 UL, one possibility to enable CA_n5-n8 operation with 2-antenna implementation is to allow non-simultaneous Rx/Tx between n5 DL and n8 UL or single UL in n5 only to avoid n5 REFSENS impact due to n8 UL transmission. With that the filter isolation between n5 DL and n8 UL would no longer be needed and a triplexer can be used to provide the semi-full-duplex carrier aggregation to a single antenna in the main path, as is shown in Figure 5.1.1-1. On the other hand, since the triplexer center band range is over 5% bandwidth ratio (band range divided by center frequency) and the frequency gap between center band and n8 DL is only 10 MHz which may pose design challenge on achieving acceptable filter isolation and insertion loss, the feasibility of a single triplexer for CA_n5-n8 needs to be studied.

Figure 5.1.1-1 Possible CA_n5-n8 UE architecture and operation diagram for semi-full-duplex CA

Another architecture variant with 2-antenna implementation is to move n8 UL to the diversity path as is shown in Figure 5.1.1-2 which may potentially ease the triplexer design in the main TRx path owing to the narrower middle band range and wider frequency separation between n5 DL and n8 DL. The filter design challenge however would be shifted to the diversity TRx path duplexer due to the wider band range to cover n5 DL and n8 UL.

Figure 5.1.1-2 Possible CA_n5-n8 UE architecture based on 2-antenna implementation

For the 3-antenna implementation, the two of the three antenna are used in the main path to aggregate the n5 and n8 signals over the air, as shown in Figure 5.1.1-3. As n5 and n8 signals do not need to be combined through a multiplexer, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation.

Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation which can be up to 348 MHz (from n105 to n8) to support all the sub-1GHz bands above 600 MHz as defined in 3GPP. On the other hand, the additional antenna cannot be added without occupying more phone space. Therefore, the feasibility on placing more than two low-band antenna in a smartphone needs to be investigated with the concern of the expected narrower bandwidth and regressed radiating performance due to the limitation in form factor.

Figure 5.1.1-3 CA_n5-n8 UE architecture based on the 3-antenna implementation

Despite the 3-antenna implementation can help reduce n8 UL interfering n5 DL via antenna isolation which however is far from sufficient even under the restricted frequency ranges for CA operation where n8 UL does not overlap with n5 DL. Therefore, semi-full-duplex operation with non-simultaneous Rx/Tx between n5 DL and n8 UL as diagramed in Figure 5.1.1-1 is still required for the CA combination.

In consideration of the operation frequency ranges restriction for this specific band combination as is shown below,

-	n5r: UL 824 - 835 MHz; DL 869 - 880 MHz

-	n8r: UL 904 - 915 MHz; DL 949 - 960 MHz

Figure 5.1.1-4 CA_n5-n8 restricted spectrum range

Dedicated filter implementation may be considered to potentially enable 2UL/2DL simultaneous operation. Due to the frequency range restriction on the dedicated filters, additional full-band duplexers for either or both n5 and n8 would be required in order to support the single-band full-band operation. Figure 5.1.1-5 and Figure 5.1.1-6 show the architecture diagrams with dedicated quadplexer in main TRx path based on 2-antenna implementation where the former uses both partial ranges for n5 and n8 in the quadplexer and the latter uses partial range only for n8 UL in the quadplexer.

Figure 5.1.1-5 CA_n5-n8 architecture with dedicated quadplexer using partial ranges in both n5 and n8

Figure 5.1.1-6 CA_n5-n8 architecture with dedicated quadplexer using partial range in n8 UL only

Figure 5.1.1-7 and Figure 5.1.1-8 show the architecture diagrams with dedicated duplexers for both n5 and n8 and n8 UL only respectively in main TRx path based on 3-antenna implementation.

Dedicated filter based on specific frequency range restriction for operation bands usually is not a common practice in UE implementation as it adds additional cost and area to the devices.

Figure 5.1.1-7 CA_n5-n8 architecture using dedicated duplexers with partial ranges in both n5 and n8

Figure 5.1.1-8 CA_n5-n8 architecture using dedicated duplexers with partial range in n8 UL only

Figure 5.1.1-9 CA_n5-n8 architecture using dedicated triplexers with partial range in n8 and n5

In the architecture of Figure 5.1.1-9, dedicated triplexers are used. In this architecture, UL’s are transmitted from different antennas, which may help in optimizing the TRP for each of the UL’s resulting in better coverage.

5.1.2	Common for 1 band UL and 2 bands UL of CA_n5-n8

5.1.2.1	Operating bands for CA

UE should support the full range of spectrum for single band operations on both n5 and n8

Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8.

Table 5.1.2.1-1: CA band combination of band n5+n8

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

The frequency relation between band n8 and n5 is shown below as UL band n8 is overlapping with DL band n5

Figure 5.1.2.1-1: The frequency relation between band n8 and n5

5.1.2.2	Channel bandwidths per operating band for CA

Based on WF R4-2214445, BCS0 for CA_n5A-n8A can be specified as below.

Table 5.1.2.2-1: Supported bandwidths per CA band combination of band n5+n6

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |

5.1.2.3	UE co-existence studies

Table 5.1.2.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n8.

Table 5.1.2.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 | 4400 | 4575 |

Table 5.1.2.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 | 4625 | 4800 |

Based on the analysis above, there is no need to specify harmonics and harmonics mixing exception for CA_n5-n8 as we didn’t specify them in current spec.

5.1.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n8, the TIB,c and RIB,c are given in the tables below considering both 2-antenna and 3–antenna implementations. If 3-antenna implementation is assumed, there isn’t any additional insertion loss observed for CA_n5-n8 compared with single carrier operation. If it’s assumed to derive the the delta Tib and Rib requirements for CA_n5-n8 based on 3–antenna implementation, all the values can be zero. If 2-antenna implementation is assumed to derive these values, the some delta Tib and Rib values for CA_n5-n8 are provided from interested companies as below.

Table 5.1.2.4-1: ΔTIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | Huawei | Skyworks | Qualcomm |
| --- | --- | --- | --- | --- |
| CA_n5-n8 | n5 | 0.4 | 0.2 | 0.5 |
|  | n8 | 0.4 | 0.4 | 0.5 |

Table 5.1.2.4-2: ΔRIB,c for 2-antenna implementation

| Inter-band CA Configuration | NR Band | Huawei | Skyworks | Qualcomm |
| --- | --- | --- | --- | --- |
| CA_n5-n8 | n5 | 0 | 0.3 | 0.5 |
|  | n8 | 0 | 0.2 | 0.5 |

5.1.2.5	REFSENS evaluation

For option 1 of full band n5 and n8 RF filters implementation, the scheduling is restricted as DL_n5-n8_UL_n5 and the concurrent operation between DL band n5 and UL band n8 can’t work. There may be some MSD for band n8 DL due to band n5 UL interference. Some legacy filters exhibit the flyback effect into n8 DL reducing the rejection of the n5 TX OOB leakage for no MSD

For option 3 of dedicated RF filters implementation with partial frequency range, the MSD test point and values due to cross band isolation between DL band n5 and UL band n8 in Table 5 are provided referring to R4-2300759.

Table 5.1.2.5-1: n8R UL cross band MSD in n5 for CA_n5-n8

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | [12.3] | ACLR2 |

5.1.3	Specific for 2 bands UL of CA_n5-n8

5.1.3.0	General

Option 2 full band n5 and n8 RF filters implementation can support both DL_n8_UL_n5-n8 and DL_n5-n8_UL_n5 features.

Option 3 dedicated RF filter(s) implementation with partial frequency range can support DL_n5-n8_UL_n5-n8 feature.

Thus, the following technical analysis in clause 5.1.3 is based on these kinds of candidate options and implementations.

5.1.3.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n8.

5.1.3.2	UE co-existence studies

Table 5.1.3.2-1 lists Band n5 + Band n8 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.3.2-1: Band n5 and Band n8 UL IMD products with frequency range restriction

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 835 | 904 | 915 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 69 | 91 | 1728 | 1750 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 733 | 766 | 973 | 1006 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 2585 | 2632 | 2665 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1557 | 1601 | 1877 | 1921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3420 | 3536 | 3580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 182 | 138 | 3456 | 3500 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2836 | 2781 | 2436 | 2381 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1097 | 1042 | 697 | 642 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4440 | 4495 | 4200 | 4255 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4360 | 4415 | 4280 | 4335 |

Based on Table 5.1.3.2-1, there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz – 835 MHz	DL: 869 MHz – 880 MHz;

n8	UL: 904 MHz – 915 MHz	DL: 949 MHz – 960 MHz.

Table 5.1.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.1.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n8 | E-UTRA Band 1, 28, 31, 34, 38, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.1.3.3	REFSENS evaluation

Based on the assumption of restricted frequency range for CA_n5-n8, there is no MSD issue due to intermodulation interference. 2UL Cross-band MSD may be needed.

5.1.4	Feasible RF implementations

Referring to clause 5.1.0 and 5.1.1 UE RF architecture assumption, it can be observed that two sets of different RF implementations are included currently:

One is full band n5 and n8 RF filters implementation.

Another one is dedicated RF filters implementation with partial frequency range.

These two implementation may have different RF requirements and scheduling restriction. In study phase, we list all the implementations on the table and further trade off each implementation as below.

Table 5.1.4-1: Trade-off between the feasible RF implementations.

| Type of implementation | Options | Pros | Cons |
| --- | --- | --- | --- |
| Full band n5 and n8 RF filters implementation | Option 1 | - Current RF components can be reused easily. - The same RF filters can support both single carrier operation and CA operation. - UE cost may not be improved too much. | - UL_n5-n8 can’t be supported. Only DL_n5-n8_UL_n5 can be supported. |
|  | Option 2 | - Current RF components can be reused. - The same RF filters can support both single carrier operation and CA operation. - Compared to option 1, DL_n8_UL_n5-n8 can be supported additionally. | - Non-concurrent n5 DL and n8 UL limiting DL throughput - At least one additional n8 RX filter is required compared to option 1 - There may be RF additional performance impacts caused by managing the non-concurrency between n5DL and n8UL - Potential impacts on RAN2 are observed |
| Dedicated RF filters implementation with partial frequency range | Option 3 | - DL_n5-n8_UL_n5-n8 can be supported by UE within partial frequency range. Operators’ demands can be met. - There is no scheduling restriction for CA operation, but the specific partial frequency range. | - At least, but not necessary limited to one n8 UL dedicated filter with UL partial frequency range need to be developed for this specific CA operation. - n5 RX filtering may be impacted to avoid n8 UL at 904-914 blocking n5RX - Dedicated filter and full band filter need to be switched between single carrier operation and CA operation. |

As all the implementations are not excluded and UE can support them optionally, a solution is needed to further distinguish these different implementations. The following options can be further discussed.

- Option 1: New bands would be required.

- Option 2: New bands would not be required. An optional signalling/capability solution can be designed for the BS and UE to distinguish these different implementations.

- Option 3: Other are not precluded.

5.2	CA_n5-n28

5.2.1	UE RF architecture assumption

The n5 filter parameter assumption in the analysis is based on without sharing path of band n26.

The UE RF architectures with 2 antenna and 3 antenna can be assumed in the feasibility study for CA_n5-n28.The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands. Figure 5.2.1.0-1 shows the aggregated spectrum allocation for CA_n5-n28 where no frequency range restriction has been indicated for either of the constituent bands.

Figure 5.2.1.0-1 CA_n5-n28 aggregated spectrum allocation

#### 5.2.1.1	UE RF architecture with two antennas

For CA_n5-n28 with 2-antenna implementation, there are two potential UE RF architectures as shown in Figure 5.2.1.1-1.

Figure 5.2.1.1-1 Potential UE architecture variants to support CA_n5-n28 with 2 antenna

The topology for 2 antenna architecture a is listed as below.

-	1) n28UL+n28DL+n5UL+n5DL quadplexer on antenna 1

-	2) n5DL+n28DL duplexer on antenna 2

The topology for 2 antenna architecture b is listed as below.

-	1) n5UL+n5DL+n28DL triplexer on antenna 1

-	2) n28UL+n28DL+n5DL triplexer on antenna 2

[Considering the maximum simultaneously covered spectrum range and closely spaced UL and DL bands, the following challenges and issues impacting UE RF performance for 2-antenna architecture variant (a) can be anticipated:

-	The main antenna design needs to cover the entire spectrum range of 191 MHz simultaneously as shown in Figure 5.2.1-1, which is equivalent to a 24% bandwidth ratio and that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised.

-	The feasibility of a single quadplexer is questionable as n28 alone already requires a dual duplexer to reduce the band range to duplex gap ratio in order to achieve the acceptable filter isolation and insertion loss performance. Dual quadplexer has the implication on UE front-end design complexity, area, and cost impacts.

-	REFSENS degradation (MSD) on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation.

For 2-antenna architecture variant (b), the following challenges and issues impacting UE RF performance can be anticipated:

-	Compared to variant (a), though the main path frequency range is reduced from 191 MHz to 136 MHz, the diversity path frequency range however is increased from 136 MHz to 191 MHz. From antenna design perspective, the same challenge with 24% antenna bandwidth ratio as in variant (a) is expected.

-	The feasibility of a single triplexer for diversity path is questionable as n28 alone already requires a dual duplexer to reduce the band range to duplex gap ratio in order to achieve the acceptable filter isolation and insertion loss performance. Dual triplexer in diversity path in conjunction with a single or dual triplexer in main path has the implication on UE front-end design complexity, area, and cost impacts.

-	REFSENS degradation (MSD) on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation.]

#### 5.2.1.2	UE RF architecture with three antennas

For CA_n5-n28 with 3-antenna implementation, the following UE RF architectures can be considered, as shown in Figure 5.2.1.2-1.

Figure 5.2.1.2-1 CA_n5-n28 UE architecture based on the 3-antenna implementation

The topology for the above 3 antenna architecture is listed as below:

1) n28UL+n28DL duplexer on antenna 1

2) n5UL+n5DL duplexer on antenna 2

3) n5DL+n28DL duplexer on antenna 3

5.2.2	Common for 1 band UL and 2 bands UL of CA_n5-n28

5.2.2.0	General

According to the current spec TS 38.101-1, DL CA_n5A-n28A_BCS0 has been specified. Thus, some of RF requirements can be reused as below.

5.2.2.1	Operating bands for CA

Table 5.2.2.1-1:  CA band combination of band n5+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

5.2.2.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n5A-n28A_BCS0 has been specified

5.2.2.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n5-n28 as we didn’t specify them in current spec.

5.2.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n28 with 2 antenna implementation, the TIB,c and RIB,c values has been specified in TS 38.101-1.

Referring to contribution R4-2300657, it is suggest to reuse the DC_28_n5 TIB,c and RIB,c values with an additional 0.2dB to all to enable the two-antenna n-plexers in the future and including the support of full band n28, which are shown below:

Table 5.2.2.4-0: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 |
|  | n28 | 0.7 |

Table 5.2.2.4-0a: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.2 |
|  | n28 | 0.2 |

For CA_n5-n28 with 3 antenna implementation, the TIB,c and RIB,c values are shown  below.

Table 5.2.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

Table 5.2.2.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

5.2.2.5	REFSENS evaluation

For CA_n5-n28 with 2 antennas implementation, referring to contribution R4-2202036, the n28 MSD evaluation for CA_n5-n28 is shown as below.

Figure 5.2.2.5-1 the n28 MSD evaluation for CA_n5-n28

Based on the agreed CR R4-2206134, one MSD test configuration (n28 30MHz) can be  a reference considering the impacts from band n5 UL.

Table 5.2.2.5-1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

For CA_n5-n28 with 3 antennas implementation, referring to contribution R4-2301266, the n28 MSD evaluation on the same MSD test configuration (n28 30MHz) as 2 antennas implementation is shown as below.

Table 5.2.2.5-2 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 18.6 | ACLR2 |

5.2.3	Specific for 2 bands UL of fallback CA_n5-n28

5.2.3.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n5-n28.

5.2.3.2	UE co-existence studies

Table 5.2.3.2-1 lists Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.2.3.2-1: Band n5 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 869 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 166 | 76 | 1527 | 1617 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 900 | 1035 | 537 | 672 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2351 | 2486 | 2230 | 2365 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1724 | 1904 | 1240 | 1420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3175 | 3355 | 2933 | 3113 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 152 | 332 | 3054 | 3234 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2168 | 1943 | 2773 | 2548 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 596 | 371 | 1201 | 976 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3636 | 3861 | 3999 | 4224 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3757 | 3982 | 3878 | 4103 |

Based on Table 5.2.3.2-1, there is no IMD issue for CA_n5-n28.

Table 5.2.3.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.2.3.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n28 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 31, 34, 38, 40, 73 NR Band n79 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 4, 41, 42, 43, 50, 51, 52, 65, 66, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

5.2.3.3	REFSENS evaluation

Since there is no two UL IMD issue for CA_n5-n28, no need to specify the MSD due to two UL IMD interference. The following reference exception can be further studied.

Referring to contribution R4-2300657, the estimation of the n5+n28 UL interference in band n28 DL versus n5 and n28 1UL case can be found in table 5.2.3.3-1.

Table 5.2.3.3-1: Estimation of the n5+n28 UL interference in band n28 DL versus n5 and n28 1UL case

| n28DL CBW (MHz) at the top of band 28 | 5 |  | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| n28 UL transmitter output noise at 23dBm (dBm) | -58.5 |  | -52.2 | -50.0 | -42.3 | -28.1 |
| 38.101-1 n28 REFSENS de-sense due to Tx (dB) | 0.0 |  | 0.0 | 1.4 | 7.0 | 11.9 |
| n28 UL transmitter output noise at 20dBm (dBm) | -61.5 |  | -55.3 | -53.8 | -50.5 | -39.2 |
| n5 UL transmitter output noise at 23dBm (dBm) | -45.2 |  | -41.1 | -41.0 | -40.8 | -40.7 |
| n5 UL transmitter output noise at 20dBm (dBm) | -53.8 |  | -49.6 | -48.9 | -47.8 | -48.9 |
| n28UL -> n28DL quadplexer isolation (dB) | 50 |  | 50 | 50 | 50 | 50 |
| n5UL -> n28DL quadplexer isolation (dB) | 45 |  | 45 | 45 | 45 | 45 |
| n28 transmitter output noise at 23dBm (dBm) at main LNA | -108.5 |  | -102.2 | -100.0 | -92.3 | -78.1 |
| n5+n28 transmitter output noise at 23dBm (dBm) at main LNA | -90.4 |  | -94.8 | -94.1 | -93.3 | -88.0 |
| delta n5+n28 vs n28 at 23dBm output noise at main LNA | 18.1 |  | 7.4 | 5.8 | -1.0 | -9.9 |

The resulting MSD (on top of n28 without self de-sense) with MRC calculations is provided in Table 5.2.3.3-1 to enable a two-antenna architecture in the long-term.

Table 5.2.3.3-2: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 793 | 20 | [3.6] | ACLR2 |
| n28 |  | 738 | 20 | 15 | 25 (Rbstart=81) |  |  |  | ACLR2 |

Referring to contribution R4-2302097, the relationship for n28 DL impacted by both band n5 UL and n28 UL is shown in figure 1, when both band n5 UL and n28 UL are configured for CA_n5-n28, band n28 DL receiver will be impacted by the leakage interference from band n5 UL and n28 UL.

Figure 5.2.3.3-1 The relationship for band n28 DL impacted by both band n5 UL and n28 UL

Some measurement results were proposed in R4-2202036 for band n28 30MHz case. -32.4dBm Tx noise in n28 Rx band can be observed from band n5 PAout. Since 45dB Tx-Rx isolation between n5 UL band and n28 DL band was assumed, 2.9dB MSD can be observed based on n28 30MHz -78.5dBm REFSENS for 2 antenna implementation, which is shown in Table 5.2.3.3-3:

Table 5.2.3.3-3: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 2.9 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

Referring to contribution R4-2302253, the most straightforward way would be to use the same test point for 2UL MSD due to Cross-Band Isolation as has been used for 1UL Cross-band Isolation for 2 antenna implementation, which is shown in Table 5.2.3.3-4:

Table 5.2.3.3-4: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 800.5 | 5 | 17.5 | n5 ACLR2 |
| n28 |  | 5 | 5 | 15 | 25 (Rbstart=0) |  |  |  |  |

Referring to contribution R4-2218129, for 2UL MSD due to Cross-Band Isolation as has been used for 1UL Cross-band Isolation for 3 antenna implementation is shown in Table 5.2.3.3-5. It includes a legacy filter rejection for n5 TX OOB leakage in n28DL. The antenna isolation helps attenuate the TX leakage to give a similar MSD result specified in Table 5.2.3.3-3 for a 30MHz DL.

Table 5.2.3.3-5: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 4.5 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

5.3	CA_n8-n20-n28

5.3.1	UE RF architecture assumption

5.3.1.0	General

The UE RF architectures with 2 antenna, 3 antenna, 4 antenna can be assumed to analyze and study for CA_n8-n20-n28.The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands. UE RF architecture with three antennas for CA_n8-n20-n28 is used as baseline for evaluation. The UE RF architectures with two or four antennas are not precluded in the study item.

5.3.1.1	UE RF architecture with three antennas

The assumption 1A for UE RF architecture with three antennas for CA_n8-n20-n28 are shown as below.

1) n28UL/n20+n28DL/n20UL triplexer on antenna 1

2) n8UL/n8DL duplexer on antenna 2

3) n20+n28DL/n8DL duplexer on antenna 3

4) 10dB antenna isolation is used for MSD calculations

The assumption 1B for UE RF architecture with three antennas for CA_n8-n20-n28 are shown as below. Comparing to assumption 1a, n8UL/n8DL duplexer on antenna 2 was replaced by n8UL/n8DL/n20UL/n20DL quadplexer.

1) n28UL/n20+n28DL/n20UL triplexer on antenna 1

2) n8UL/n8DL/n20UL/n20DL quadplexer on antenna 2

3) n20+n28DL/n8DL duplexer on antenna 3

4) 10dB antenna isolation is used for MSD calculations

The assumption 1C for UE RF architecture with three antennas for CA_n8-n20-n28 are shown as Figure 5.3.1.1-1. These are two other variants based on assumption 1a. They may allow easier implementation of either triplexer or duplexer in main path owing to that the frequency separations between the sub-bands are wider than the architecture assumed for DC_8-20_n28. However, the downside with two variants is that the widest frequency range which needs to be covered by any of the antenna is still relatively wide at 212 MHz and 257 MHz respectively.

Figure 5.3.1.1-1 Potential UE architecture variants to support CA_n8-n20-n28 with 3 antenna

5.3.1.2	UE RF architecture with two antennas

The assumption 2A for UE RF architecture with two antennas for CA_n8-n20-n28 are shown as below. Pentaplexer design is considered.

1) n28UL/n20+n28DL/n20UL/n8UL/n8DL pentaplexer on antenna 1

2) n20+n28DL/n8DL duplexer on antenna 2

3) 10dB antenna isolation is used for MSD calculations

The assumption 2B for UE RF architecture with two antennas for CA_n8-n20-n28 are shown as below. Quadplexer design is considered.

1) n28UL/n20+n28DL/n20UL/n8DL quadplexer on antenna 1

2) n20+n28DL/n8UL/n8DL triplexer or n20+n28DL/n20UL/n8UL/n8DL quadplexer on antenna 2

3) 10dB antenna isolation is used for MSD calculations

There are two potential implementation challenges with 2-antenna architectures. The first challenge is that the antenna design needs to cover the entire spectrum range of 257 MHz simultaneously as shown in Figure 5.3.1-1 for either of the variants, which is equivalent to a 31% bandwidth ratio and that would far exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination is expected to be compromised.

The second challenge is on the feasibility of a pentaplexer design for variant (a) to aggregate five closely spaced spectrum ranges as shown in Figure 5.3.1-1, not only with sufficient filter isolation between self-band and cross-band but also with acceptable insertion loss. The variant (b) though avoids the use of pentaplexer, the design of the intended quadplexer is still relatively challenging considering the closely spaced spectrum ranges with nearly 8% bandwidth ratio for the n20/n28 combined DL range.

5.3.1.3	UE RF architecture with four antennas

The assumption 3A and 3B for UE RF architecture with four antennas for CA_n8-n20-n28 are shown as below.

Figure 5.3.1.3-1 The assumption 3A for UE RF architecture to support CA_n8-n20-n28 with 4 antenna

Figure 5.3.1.3-2 The assumption 3B for UE RF architecture to support CA_n8-n20-n28 with 4 antenna

For the 4-antenna implementation, the three of the four antenna are used in the main path to aggregate the n8, n20, and n28 signals over the air, as shown in Figures 5.3.1.3-1 and 5.3.1.3-2. As n8, n20, and n28 signals do not need to be combined through a multiplexer, there is no additional insertion loss in n8, n20, and n28 main signal paths as compared to single-band implementation.

Having 3 antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the 3 antenna as compared to single-antenna implementation. However, the additional antenna cannot be added without occupying more phone space.

5.3.2	CA_n8-n20

5.3.2.0	General

According to the current spec TS 38.101-1, DL CA_n8A-n20A_BCS0 has been specified. Thus, some of RF requirements can be reused as below. Both DL_n8A-n20A_BCS0 and DL_n8A-n20A_UL_n8A-n20A are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n8A-n20A.

5.3.2.1	Operating bands for CA

Table 5.3.2.1-1:  CA band combination of band n8+n20

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

5.3.2.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n8A-n20A_BCS0 has been specified.

5.3.2.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n8-n20 as we didn’t specify them in current spec.

5.3.2.4	∆TIB,c and ∆RIB,c values

For CA_n8-n20, the TIB,c and RIB,c values have been specified in TS 38.101-1.

5.3.2.5	REFSENS requirements

As CA_n8-n20 has been specified into current spec TS 38.101-1, there is no harmonics, cross band isolation and harmonics mixing exception.

5.3.2.6	Maximum output power for UL inter-band CA

Power class 3 is assumed for UL CA_n8-n20.

5.3.2.7	UE co-existence studies for UL inter-band CA

Table 5.3.2.7-1 lists Band n8 + Band n20 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.2.7-1: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 83 | 18 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 844 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592 | 2692 | 2544 | 2639 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 | 1913 | 1581 | 1706 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 | 3607 | 3376 | 3501 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 36 | 166 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2568 | 2413 | 2828 | 2658 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 | 4363 | 4352 | 4522 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4416 | 4304 | 4469 |

Based on Table 5.3.2.6-1, 3rd and 5th order IMD may also fall into Rx frequencies of bands n8 or band n20.

Table 5.3.2.7-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.3.2.7-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n20 | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8, 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.3.2.8	REFSENS evaluation for UL inter-band CA

IMD3 MSD of DC_20_n8A in band 20 and n8 specified in TS 38.101-3 is a good reference for CA_n8-n20 as belo

Table 5.3.2.8-1: REFSENS exceptions due to IMD interference for CA_n8-n20

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20 | n8 | 892.5 | 5 | 25 | 937.5 | 25 | FDD | IMD3 |
|  | n20 | 849.5 | 5 | 25 | 808.5 | 25 | FDD | IMD3 |

5.3.3	CA_n8-n28

5.3.3.0	General

According to the current spec TS 38.101-1, DL CA_n8A-n28A_BCS0 has been specified. Thus, some of RF requirements can be reused as below. Both DL_n8A-n28A_BCS0 and DL_n8A-n28A_UL_n8A-n28A are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n8A-n28A.

5.3.3.1	Operating bands for CA

Table 5.3.3.1-1:  CA band combination of band n8+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

5.3.3.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1e of TS 38.101-1, DL_n8A-n28A_BCS0 has been specified as below.

Table 5.3.3.2-1: Supported bandwidths per CA band combination of band n8+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n28A | CA_n8A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |

5.3.3.3	UE co-existence studies

There is no need to specify harmonics and harmonics mixing exception for CA_n8-n28 as we didn’t specify them in current spec.

5.3.3.4	∆TIB,c and ∆RIB,c values

For CA_n8-n28, the TIB,c and RIB,c values have  been specified in TS 38.101-1.

5.3.3.5	REFSENS requirements

As CA_n8-n28 has been specified into current spec TS 38.101-1, there is no harmonics, cross band isolation and harmonics mixing exception.

5.3.3.6	Maximum output power for UL inter-band CA

Power class 3 is assumed for UL CA_n8-n28.

5.3.3.7	UE co-existence studies for UL inter-band CA

Table 5.3.3.7-1 lists Band n8 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.3.7-1: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Based on Table 5.3.3.7-1, there is no IMD issue for CA_n8-n28.

Table 5.3.3.7-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.3.3.7-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n28 | E-UTRA Band 20, 31, 34, 38, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79, n100 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

5.3.3.8	REFSENS requirements for UL inter-band CA

There is no REFSENS exception due to IMD interference for CA_n8A-n28A.

5.3.4	CA_n20-n28

5.3.4.1	General

According to the current spec TS 38.101-1, DL_n20A-n28A_UL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS1 have already been specified. Thus, all of the related RF requirements have been analysed in clause 6.31 from TR 38.716-02-00 and can be reused as below. Both DL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS0 are fallback CA band combinations of DL_n8A-n20A-n28A_UL_n20A-n28A.

1) The operating bands for CA_n20-n28 were specified in table 5.2A.2.1-1 from TS 38.101-1 with note 2 that The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.

2) The UL/DL configurations and bandwidth combination sets for DL_n20A-n28A_UL_n20A-n28A_BCS0 and DL_n20A-n28A_UL_n20A-n28A_BCS1 were specified in table 5.5A.3.1-1g from TS 38.101-1.

3) 23dBm Power class 3 for UL CA_n20A-n28A was specified in Table 6.2A.1.3-1 from TS 38.101-1.

4) 0.5dB TIB,c for CA_n20-n28 was specified in Table 6.2A.4.2.3-1 from TS 38.101-1.

5) Spurious emissions for UE co-existence for Inter-band CA_n20-n28 were specified in Table 6.5A.3.2.3-1 from TS 38.101-1.

6) Referring to UE co-existence studies in clause 6.31.2.1 from TR 38.716-02-00, there are no harmonic/harmonic mixing or IMD issues affecting own Rx frequencies of either band n20 or n28. Therefore, there is no need to specify the additional REFSENS exceptions for CA_n20-n28.

7) 0dB RIB,c for CA_n20-n28 was specified in Table 6.31.1.4-2 from TR 38.716-02-00.

5.3.5	CA_n8-n20-n28

5.3.5.1	Operating bands for CA

Table 5.3.5.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

It’s noted that the following sentence was clarified in clause 5.5A.0 from TS 38.101-1.

“For a higher order band combination of which CA_n20-n28 is a subset, the frequency range in band n28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.”

Thus, this frequency range restriction is also applicable to CA_n8-n20-n28.

5.3.5.2	Channel bandwidths per operating band for CA

Table 5.3.5.2-1: Supported bandwidths per CA band combination of band n8+n20+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n20A-n28A | CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 |  |

5.3.5.3	∆TIB,c and ∆RIB,c values

For CA_n8-n20-n28, the TIB,c and RIB,c values for 2-antenna and 3-antenna architectures are captured in this part. However, only one set of values will be defined in the specification in the follow-up WI allowing all possible UE RF architectures.

For CA_n8-n20-n28, the TIB,c and RIB,c values are derived from the existing CA_n8-n28 and CA_n20-n28 based on 3-antennas UE architecture as the baseline.

Table 5.3.5.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.6 |
|  | n20 | 0.5 |
|  | n28 | 0.5 |

Table 5.3.5.3-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.2 |
|  | n20 | 0 |
|  | n28 | 1 |

For CA_n8-n20-n28, the ΔTIB,c and ΔRIB,c values are derived in Table 5.3.5.3-3 and Table 5.3.5.3-4 and enable a two-antenna architecture in the long-term.

Table 5.3.5.3-3: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.8 |
|  | n20 | 0.7 |
|  | n28 | 0.7 |

Table 5.3.5.3-4: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.3 |
|  | n20 | 0.2 |
|  | n28 | 0.2 |

5.3.5.4	UE co-existence studies for UL inter-band CA

Table 5.3.5.4-1 lists Band n8 + Band n20 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n28.

Table 5.3.5.4-1: Band n8 and Band n20 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 832 | 862 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 83 | 18 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 898 | 998 | 749 | 844 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2592 | 2692 | 2544 | 2639 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1778 | 1913 | 1581 | 1706 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3472 | 3607 | 3376 | 3501 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 36 | 166 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2568 | 2413 | 2828 | 2658 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 826 | 666 | 1081 | 916 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4208 | 4363 | 4352 | 4522 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4256 | 4416 | 4304 | 4469 |

Table 5.3.5.4-2 lists Band n8 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n20.

Table 5.3.5.4-2: Band n8 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Table 5.3.5.4-3 lists Band n20 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n8.

Table 5.3.5.4-3: Band n20 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 703 | 733 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 159 | 99 | 1535 | 1595 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 931 | 1021 | 544 | 634 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2367 | 2457 | 2238 | 2328 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1763 | 1883 | 1247 | 1367 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3199 | 3319 | 2941 | 3061 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 198 | 318 | 3070 | 3190 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2100 | 1950 | 2745 | 2595 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 535 | 385 | 1180 | 1030 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3644 | 3794 | 4031 | 4181 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3773 | 3923 | 3902 | 4052 |

Based on Table 5.3.5.4-1, 3rd and 5th order IMD may also fall into Rx frequencies of third DL band n28 when CA_n8-n20 is the UL configuration.

Based on Table 5.3.5.4-2, there is no IMD interference which may also fall into Rx frequencies of third DL band n20 when CA_n8-n28 is the UL configuration.

Based on Table 5.3.5.4-3, 3rd order IMD may also fall into Rx frequencies of third DL band n8 when CA_n20-n28 is the UL configuration.

5.3.5.5	REFSENS evaluation

The MSD test configurations can be evaluated as below for band n8 MSD due to IMD3 of Tx band n20 + band n28 and band n28 MSD due to IMD3 of Tx band n8 + band n20.

Table 5.3.5.5-1: REFSENS exceptions due to IMD interference for CA_n8-n20-n28

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n20-n28 | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | FDD | N/A |
|  | n8 | 908.5 | 5 | 25 | 953.5 | 23.5 | FDD | IMD3 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 726.5 | 5 | 25 | 781.5 | 23 | FDD | IMD3 |

CA_n8-n20-n28 2UL IMD3 MSDs are derived in Table 5.3.5.5-2 and enable a two-antenna architecture in the long-term.

Table 5.3.5.5-2: 3DL/2UL IMD3 MSDs for CA_n8-n20-n28

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n8-n20-n28 | n8 | N/A | 5 | 25 | 951.5 | [25] | FDD | IMD3 |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 717.5 | 5 | 25 | 772.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | 25 | 781.5 | [25] | FDD | IMD3 |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | RAN4#104be | R4-2217712 |  |  |  | TR skeleton | 0.1.0 |
| 2022-10 | RAN4#104be | R4-2217713 |  |  |  | TP and discussion on CA_n5-n28 | 0.2.0 |
| 2022-10 | RAN4#104be | R4-2217714 |  |  |  | TP and discussion on CA_n8-n20-n28 | 0.2.0 |
| 2022-10 | RAN4#104be | R4-2217715 |  |  |  | TP and discussion on CA_n5-n8 | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2220504 |  |  |  | TP for TR 38.872 on CA_n5-n8 architectures | 0.3.0 |
| 2022-11 | RAN4#105 | R4-2220505 |  |  |  | TP on CA_n5-n28 | 0.3.0 |
| 2022-11 | RAN4#105 | R4-2220506 |  |  |  | TP for TR 38.872 to capture the RF architecture assumption and IMD evaluation for CA_n8-n20-n28 | 0.3.0 |
| 2023-03 | RAN4#106 | R4-2300560 |  |  |  | TP for TR 38.872: scope and references | 0.4.0 |
| 2023-03 | RAN4#106 | R4-2303544 |  |  |  | TP for TR 38.872 to complete the open issues for CA_n5-n8 | 0.4.0 |
| 2023-03 | RAN4#106 | R4-2303545 |  |  |  | TP for TR 38.872 to complete the open issues for CA_n5-n28 | 0.4.0 |
| 2023-03 | RAN4#106 | R4-2303546 |  |  |  | TP for TR 38.872 to complete the open issues for CA_n8-n20-n28 | 0.4.0 |