3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304014

Online Meeting, 17 – 26 April 2023

3GPP TSG-RAN WG2 Meeting #121	R2-2302074

Athens, Greece, 27th February – 3rd March 2023

Title:	LS to RAN4 on autonomous denial for IDC

Response to:

Release:	Release 18

Work Items:	NR_IDC_enh-Core

Source:	RAN WG2

To:	RAN WG4

CC:

Contact Person:

Name:		Xiaodong Yang, Yumin Wu

E-mail Address:	yangxiaodong5g@vivo.com,  wuyumin@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 has agreed to introduce per CG autonomous denial configuration for the NR IDC issue. The time unit for autonomous denial configuration is slot. This means that the UE is allowed to deny any transmission in a particular slot if during the number of slots indicated by autonomousDenialValidity, preceeding and including this particular slot, it autonomously denied fewer slots than indicated by autonomousDenialSlots. For the autonomous denial parameters, the following values are agreed for both single CC and carrier aggregation cases. RAN2 also observed that the autonomous denial solution might require new RRM requirements in RAN4, as LTE.

autonomousDenialParameters-r18		SEQUENCE {

autonomousDenialSlots-r18			ENUMERATED { n2, n5, n10, n15,

n20, n30, spare2, spare1},

autonomousDenialValidity-r18		ENUMERATED { n200, n500, n1000, n2000,

spare4, spare3, spare2, spare1}

FFS On other values.

# 2	Actions

To RAN4

ACTION: RAN2 kindly asks RAN4 to take the above information into account for the further work, and define the corresponding RRM requirements if needed.

# 3	Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #121bis-e			April 17th – April 26th, 2023		Online

TSG RAN WG2 Meeting #122			May 22nd – May 26th, 2023		Incheon,KR