3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304041

Electronic Meeting, April 17-26, 2023

Source:	Nokia, Ericsson

Title:	Discussion on viable UE architectures

Agenda item:	5.12.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss feasibility of UE architecture types 3 and 4.

# 2	Discussion

The different UE architectures discussed in this WI are listed in [1]:

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |

Table 1: UE architectures

So far RAN4 has prioritized type 3 architecture as it has been WI proponents wish. However discussion has stalled because no agreement has been reached for power imbalance and MRTD/MTTD and BS TAE. What makes this discussion very difficult that all these issues are intertwined and furthermore cannot be decided only in RF or RRM room. In a way discussion is similar that RAN4 had during Rel16/17 for FR2 interband CA for CBM UE architecture, that discussion lasted two releases without conclusion. We anticipate that this case is not much simpler. In order to keep WI time schedule i.e. completion in RAN#100 we propose

Proposal 1: Stop discussion on Type 3 in RAN4#106bis to keep WI timeline.

But in order to deliver 4Rx functionality to WI proponent RAN4 should define requirements for Type 4 UE. Technically this is easy as Type 2 UE power imbalance requirement can be re-used, similarly also Type 2 MRTD/MTTD requirements can be reused. BS TAE can be kept in 3us.

Proposal 2: RAN4 defines requirements for UE Type 4.

Proposal 3: UE Type 2 power imbalance is re-used for UE Type 4 in Rel-18.

Proposal 4: BS TAE is 3 us.

# 3 	Conclusion

In this contribution we have proposed

Proposal 1: Stop discussion on Type 3 in RAN4#106bis to keep WI timeline.

Proposal 2: RAN4 defines requirements for UE Type 4 in Rel-18.

Proposal 3: UE Type 2 power imbalance is re-used for UE Type 4.

Proposal 4: BS TAE is 3 us.

# References

[1] 		R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI, RAN4#104bis-e