3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304048

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Supporting results for 4Tx Demodulation

Agenda item:	5.5.4.2

Document for:	Discussion

Introduction

In RAN #96, a new RAN4 WI “NR_ENDC_RF_FR1_enh2” was approved [1] with the following objectives defined regarding the performance aspects of Rel-18 FR1:

The objectives of performance part for Rel-18 RF FR1 requirement focus evolution include: - Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices - Specify the BS demodulation performance requirements to support UL 4-layer MIMO UE operation - Enable 8Rx for CPE/FWA/vehicle/industrial devices - Specify RLM test cases to support 8Rx - Investigate if the existing 4Rx RLM test can be reused or the new test will be specified - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers - Specify the SDR requirements with 8 MIMO layers - Specify release independence requirements in TS 38.307 if needed.

Hence, given this plenary agreement, it is needed that the performance requirements of the BS demodulator are agreed, in order to define the minimum performance requirements of a 5G NR BS, for the demodulation of PUSCH when utilizing 4Tx in FR1.

The objectives of core part for Rel-18 RF FR1 requirement focus evolution include: Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4] - Investigate framework and architecture Example bands: - TDD bands: n41, n77/n78 - FDD bands: n1 - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 4Tx - First priority: 4x4 UL MIMO - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port - PA configuration assumption: - First priority: 4x23dBm - Second priority: 2x23dBm + 2x26dBm, 4x26dBm - UE power class - First priority: PC 1.5 - Second priority: PC2/PC3, and/or new power class if needed - Note 1: PC1.5 is only applicable for TDD bands - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97

In this contribution we provide simulation results to support our companion discussion tdoc [3], based upon the parameters defined in [2]

Discussion

This paper presents the results of simulations which have been conducted by Nokia based on the parameters defined [1]. This simulation parameters agreed at RAN4 #106 and distributed in the WF for 4Tx [2] can be seen summarised in Table 1.

Table 1 : Extended Simulation Parameters for 4Tx following RAN4# 106

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Disabled |  |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence | 0, 2, 3, 1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |  |
|  | DM-RS port | {0, 1, 2, 3} |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain | PUSCH mapping type | A and B |  |
| resource | Start symbol | 0 |  |
| assignment | Allocation length | 14 |  |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |  |
| assignment | Frequency hopping | Disabled |  |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0, FFS on 4 |  |
| Code block group based PUSCH transmission |  | Disabled |  |
| Antenna Configuration |  | 4T4R and 4T8R |  |
| Bandwidth and SCS |  | BW | SCS |
|  |  | 5 MHz | 15 kHz |
|  |  | 20 MHz |  |
|  |  | 50 MHz |  |
|  |  | 10 MHz | 30 kHz |
|  |  | 100 MHz |  |
| Propagation Conditions and MCS |  | Propagation | MCS (Table 1) |
|  |  | TDLA 30 - 10 | 16 |
|  |  |  | 17 |
|  |  |  | 20 |
|  |  | TDLB 100-400 | 2 |
|  |  | TDLC 300-100 | 12 |
|  |  |  | 16 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |  |

All the values represent the SNR value in dB at which 70% of the maximum throughput could be reached.

Results will be presented broken down by propagation type, please note that results for TDLB 100-400 include results for the medium correlation channel, which we propose to be included in the 4Tx performance specifications, specifically including a sparse (idx 0) and dense TPMI (idx 4), this is discussed explicitly in our companion tdoc [3] and was marked FFS within [2].

Results for TDLA 30-10

Table 2 presents the results for TDLA 10-30 propagation environment, the results are presented for both PUSCH mapping types, and all results are using TPMI 0 and ‘low’ correlation.

Table 2 : Results for TDLA 10-30, TPMI 0

| Parameter |  |  |  |  | SNR Value (@ 70% TPUT) ideal |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | Ant. | SCS | BW | MCS | PUSCH Mapping Type A | PUSCH Mapping Type B |
| Rank4 | 4T4R | 15kHz | 5MHz | 16 | 15.7 | 15.8 |
| Rank4 | 4T4R | 15kHz | 5MHz | 17 | 16.2 | 16.2 |
| Rank4 | 4T4R | 15kHz | 5MHz | 20 | 19.4 | 19.4 |
| Rank4 | 4T4R | 15kHz | 20MHz | 16 | 17.0 | 16.9 |
| Rank4 | 4T4R | 15kHz | 20MHz | 17 | 17.3 | 17.3 |
| Rank4 | 4T4R | 15kHz | 20MHz | 20 | 21.4 | 21.4 |
| Rank4 | 4T4R | 15kHz | 50MHz | 16 | 18.3 | 18.4 |
| Rank4 | 4T4R | 15kHz | 50MHz | 17 | 18.5 | 18.5 |
| Rank4 | 4T4R | 15kHz | 50MHz | 20 | 23.3 | 23.3 |
| Rank4 | 4T4R | 30kHz | 100MHz | 16 | 16.0 | 16.0 |
| Rank4 | 4T4R | 30kHz | 100MHz | 17 | 16.4 | 16.4 |
| Rank4 | 4T4R | 30kHz | 100MHz | 20 | 19.4 | 19.4 |
| Rank4 | 4T4R | 30kHz | 100MHz | 16 | 17.9 | 18.0 |
| Rank4 | 4T4R | 30kHz | 100MHz | 17 | 18.1 | 18.2 |
| Rank4 | 4T4R | 30kHz | 100MHz | 20 | 22.8 | 22.8 |
| Rank4 | 4T8R | 15kHz | 5MHz | 16 | 8.4 | 8.4 |
| Rank4 | 4T8R | 15kHz | 5MHz | 17 | 9.0 | 9.0 |
| Rank4 | 4T8R | 15kHz | 5MHz | 20 | 11.7 | 11.7 |
| Rank4 | 4T8R | 15kHz | 20MHz | 16 | 8.8 | 8.9 |
| Rank4 | 4T8R | 15kHz | 20MHz | 17 | 9.4 | 9.4 |
| Rank4 | 4T8R | 15kHz | 20MHz | 20 | 12.3 | 12.4 |
| Rank4 | 4T8R | 15kHz | 50MHz | 16 | 9.3 | 9.4 |
| Rank4 | 4T8R | 15kHz | 50MHz | 17 | 9.8 | 9.9 |
| Rank4 | 4T8R | 15kHz | 50MHz | 20 | 12.7 | 12.7 |
| Rank4 | 4T8R | 30kHz | 100MHz | 16 | 8.6 | 8.6 |
| Rank4 | 4T8R | 30kHz | 100MHz | 17 | 9.2 | 9.2 |
| Rank4 | 4T8R | 30kHz | 100MHz | 20 | 11.7 | 11.8 |
| Rank4 | 4T8R | 30kHz | 100MHz | 16 | 9.3 | 9.4 |
| Rank4 | 4T8R | 30kHz | 100MHz | 17 | 9.8 | 9.9 |
| Rank4 | 4T8R | 30kHz | 100MHz | 20 | 12.8 | 12.8 |

Results for TDLB 100-400

Table 3 presents the results for TDLB 100-400 propagation environment. For this channel, results have been included for a dense TPMI, specifically TPMI 4 (as indicated as FFS in [2]), further to this, as due to having no spatial component in ‘low’ correlation channels, results for ‘medium’ correlation have also been included, these are specified in [4].

The performance delta between these TPMIs in low and medium correlation are discussed in [3].

Results with a yellow box were unable to be produced in time for RAN4 #106-bis-e and results with ‘/’ were not simulated, as these columns are for results to support the study on dense TPMIs in ‘medium’ correlation channels.

Table 3 : Results for TDLB 100-400

| Parameters |  |  |  |  | SNR Value (@ 70% TPUT) ideal |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PUSCH Mapping Type A |  |  |  | PUSCH Mapping Type B |  |  |  |
|  |  |  |  |  | Low Correlation |  | Medium Correlation |  | Low Correlation |  | Medium Correlation |  |
| Rank | Ant. | SCS | BW | MCS | TPMI 0 | TPMI 4 | TPMI 0 | TPMI 4 | TPMI 0 | TPMI 4 | TPMI 0 | TPMI 4 |  |
| Rank4 | 4T4R | 15kHz | 5MHz | 2 | -1.2 | -1.2 | 14.2 | 10.3 | -1.2 | -1.2 | 14.8 | 10.9 |  |
| Rank4 | 4T4R | 15kHz | 5MHz | 12 | 13.0 | 13.1 | / | / | 15.1 | 15.3 | / | / |  |
| Rank4 | 4T4R | 15kHz | 20MHz | 2 | -0.9 | -0.9 | 16.6 | 12.7 | -0.9 | -0.9 | 19.0 | 14.0 |  |
| Rank4 | 4T4R | 15kHz | 20MHz | 12 | 14.8 | 14.7 | / | / |  |  | / | / |  |
| Rank4 | 4T4R | 15kHz | 50MHz | 2 | -0.9 | -0.9 | 17.0 | 12.3 | -0.7 | -0.8 | 21.2 | 14.5 |  |
| Rank4 | 4T4R | 15kHz | 50MHz | 12 | 16.0 | 15.9 | / | / |  |  | / | / |  |
| Rank4 | 4T4R | 30kHz | 100MHz | 2 | -1.4 | -1.4 | 13.2 | 9.5 | -1.4 | -1.4 | 13.5 | 9.6 |  |
| Rank4 | 4T4R | 30kHz | 100MHz | 12 | 11.5 | 11.5 | / | / | 11.6 | 11.5 | / | / |  |
| Rank4 | 4T4R | 30kHz | 100MHz | 2 | -1.0 | -1.0 | 15.0 | 11.0 | -1.0 | -1.0 | 15.3 | 11.2 |  |
| Rank4 | 4T4R | 30kHz | 100MHz | 12 | 12.2 | 12.2 | / | / | 12.3 | 12.3 | / | / |  |
| Rank4 | 4T8R | 15kHz | 5MHz | 2 | -4.6 | -4.6 | 16.7 | 12.1 | -4.5 | -4.5 | 17.5 | 12.6 |  |
| Rank4 | 4T8R | 15kHz | 5MHz | 12 | 5.5 | 5.5 | / | / | 5.9 | 5.9 | / | / |  |
| Rank4 | 4T8R | 15kHz | 20MHz | 2 | -4.2 | -4.2 | 20.9 | / | -4.1 | -4.2 | 26.0 | 19.4 |  |
| Rank4 | 4T8R | 15kHz | 20MHz | 12 | 6.0 | 6.0 | / | / | 7.4 | 7.4 | / | / |  |
| Rank4 | 4T8R | 15kHz | 50MHz | 2 | -4.2 | -4.2 | 22.7 | 15.2 | -4.1 | -4.0 |  | 23.3 |  |
| Rank4 | 4T8R | 15kHz | 50MHz | 12 | 6.2 | 6.2 | / | / | 7.9 | 7.9 | / | / |  |
| Rank4 | 4T8R | 30kHz | 100MHz | 2 | -4.6 | -4.6 | 15.6 | 11.1 | -4.5 | -4.6 | 16.2 | 11.3 |  |
| Rank4 | 4T8R | 30kHz | 100MHz | 12 | 5.1 | 5.1 | / | / | 5.1 | 5.1 | / | / |  |
| Rank4 | 4T8R | 30kHz | 100MHz | 2 | -4.1 | -4.1 | 19.2 | 13.2 | -4.0 | -4.1 | 19.8 | 13.5 |  |
| Rank4 | 4T8R | 30kHz | 100MHz | 12 | 5.6 | 5.6 | / | / | 5.7 | 5.6 | / | / |  |

Results for TDLC 300-100

Table 4 presents the results for TDLA 300-100 propagation environment, the results are presented for both PUSCH mapping types, and all results are using TPMI 0 and ‘low’ correlation.

Table 4 : Results for TDLC 300-100

| Parameter |  |  |  |  | SNR Value (@ 70% TPUT) ideal |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rank | Ant. | SCS | BW | MCS | PUSCH Mapping Type A | PUSCH Mapping Type B |
| Rank4 | 4T4R | 15kHz | 5MHz | 16 | 17.9 | 17.9 |
| Rank4 | 4T4R | 15kHz | 20MHz | 16 | 18.2 | 18.1 |
| Rank4 | 4T4R | 15kHz | 50MHz | 16 | 18.8 | 18.7 |
| Rank4 | 4T4R | 30kHz | 100MHz | 16 | 19.2 | 19.1 |
| Rank4 | 4T4R | 30kHz | 100MHz | 16 | 19.9 | 19.9 |
| Rank4 | 4T8R | 15kHz | 5MHz | 16 | 9.4 | 9.4 |
| Rank4 | 4T8R | 15kHz | 20MHz | 16 | 9.5 | 9.6 |
| Rank4 | 4T8R | 15kHz | 50MHz | 16 | 9.8 | 9.9 |
| Rank4 | 4T8R | 30kHz | 100MHz | 16 | 10.0 | 10.1 |
| Rank4 | 4T8R | 30kHz | 100MHz | 16 | 10.4 | 10.6 |

Conclusion

In this contribution, simulation results were provided on the 4Tx parameters as agreed in R4-2302926 [2].

These results are discussed, and a series of observations and proposals made in [3].

- References

- RP-221496 Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)

- R4-2302926, WF on FR1 4Tx demodulation requirement, Ericsson, RAN4 #106, Athens, Greece, February 27th ‒ March 3rd, 2023.

- R4-2304047, Discussion on 4Tx BS Demodulation, RAN4 #106-bis-e, Electronic Meeting, April 17 – 26, 2023.

- TS 38.104 v18.0.0, Technical Specification Group Radio Access Network; NR; Base Station (BS) radio transmission and reception (Release 18), December 2022