3GPP TSG-RAN WG4 Meeting # 106bis-e			R4-2304096

# Online, April 17 – April 26, 2023

# Source: 	Nokia, Nokia Shanghai Bell

# Title: 	Discussion on BS EMC enhancements

# Agenda item:	5.18.2

# Document for:	Approval

- Introduction

In the WF document at RAN4#106 meeting [1], it was agreed that at least based on some of vendors’ observations, E-UTRA can cover UTRA for MSR BS with “common HW”. We feel strongly that this would be an excellent way to reduce the ever-growing complexity of EMC testing and in this document, we share our views on the topic.

- Discussion

The general trend is to switch off legacy networks and refarm spectrum in many regions. Some forecasts suggest that UTRA networks will be switched off earlier than GSM, as GSM is widely used for M2M and IoT use cases such as smart metering. This trend should be taken into account when optimizing testing.

As we stated in [2], there is a clear view from experience with MSR BS EMC immunity testing that E-UTRA could cover UTRA. Using E-UTRA to cover UTRA was largely supported by other companies as stated in the agreement. Furthermore, tests provided by Ericsson in [3] show the measurement results supporting this as well.

If E-UTRA would cover UTRA, the standard already includes all necessary capability sets and configurations, thus it wouldn’t require other changes to standard. Table 1 lists the capability sets for MSR BS and indicates the capability sets that are applicable to UTRA [4,5]. Using E-UTRA to cover UTRA would reduce the amount of testing by approximately 1/3, as six capability sets (CS3, CS6, CS7, CS14, CS15 and CS19) would be left out.

Table 1. Multi-standard radio (MSR) base station (BS) capability sets and CS applicability to UTRA.

| Capability Set Supported by the BS | Supported RATs | Tested CS (E-UTRA covers UTRA) |
| --- | --- | --- |
| CS1 | UTRA | CS1 |
| CS2 | E-UTRA | CS2 |
| CS3 | UTRA, E-UTRA | CS2 |
| CS4 | GSM, UTRA | CS4 |
| CS5 | GSM, E-UTRA | CS5 |
| CS6 | GSM, UTRA, E-UTRA | CS5 |
| CS7 | GSM, UTRA, E-UTRA | CS5 |
| CS8 | NB-IoT standalone | CS8 |
| CS9 | GSM, NB-IoT standalone | CS9 |
| CS10 | UTRA, NB-IoT standalone | CS10 |
| CS11 | E-UTRA, NB-IoT standalone | CS11 |
| CS12 | GSM, UTRA, NB-IoT standalone | CS12 |
| CS13 | GSM, E-UTRA, NB-IoT standalone | CS13 |
| CS14 | UTRA, E-UTRA, NB-IoT standalone | CS11 |
| CS15 | GSM, UTRA, E-UTRA, NB-IoT standalone | CS13 |
| CS16 | NR, E-UTRA | CS16 |
| CS17 | NR, E-UTRA, NB-IoT standalone | CS17 |
| CS18 | GSM, E-UTRA, NR | CS18 |
| CS19 | UTRA, E-UTRA, NR | CS16 |

Estimating the impact of narrowband RATs (NB-IoT and GSM) dropping out of the tests is more difficult as they all have different sensitivity levels.

Observation 1: Using E-UTRA to cover UTRA in the tests reduces the amount of EMC immunity testing by approximately 33%.

Proposal 1: Use E-UTRA to cover UTRA with reducing capability sets CS3, CS6, CS7, CS14, CS15 and CS19 from the tests.

- Conclusion

In this paper, we provide some discussion for the BS EMC enhancements. The conclusions are shown below:

Observation 1: Using E-UTRA to cover UTRA in the tests reduces the amount of EMC testing by approximately 33%.

Proposal 1: Use E-UTRA to cover UTRA with reducing capability sets CS3, CS6, CS7, CS14, CS15 and CS19 from the tests.

- References

- R4-2302990, WF on BS EMC enhancement, 3GPP TSG-RAN WG4 Meeting #106

- R4-2300443, Analysis of EMC Enhancements, 3GPP TSG-RAN WG4 Meeting #106

- R4-2302206, Investigation of susceptibility of different RATs, 3GPP TSG-RAN WG4 Meeting #106

- 3GPP TS 37.141; NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing

- 3GPP TS 37.113; NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)