3GPP TSG-RAN WG4 Meeting #106bis-e                	R4-2304098

Electronic Meeting, April 17 – 26, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for 8Rx UE demodulation

Agenda item: 	5.5.4.1.1

Document for: 	Discussion

Introduction

The achieved agreements from previous RAN4 meetings (RAN4#105 and RAN#106) are detailed in [1] and [2].

In this contribution, we provide the simulation results for 8 Rx PDSCH UE demodulation requirements.

PDSCH simulation results

Test parameters

We provide in this section the initial simulation results for PDSCH based on simulation assumption showed in Table 1.

Table 1: Simulation assumption for PDSCH

| Simulation parameters |  |  |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD pattern |  | 7D1S2U |
|  |  | S=6D:4G:4U (Not schedule PDSCH in special slot for 8 layers cases) |
| SCS(kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A and B |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM:6% |
|  |  | 256QAM:3% |
| Test metric |  | • Only 70% of maximum TP |
|  |  | Rank 2, 4 and 8 |
| Antenna configuration |  | • Rank 2: 2T8R |
|  |  | • Rank 4: 4T8R |
|  |  | • Rank 8: 8T8R |
| Antenna correlation |  | • Rank 2: ULA Medium B |
|  |  | • Rank 4: ULA Low |
|  |  | • Rank 8: ULA Low |
| Propagation conditions |  | • Rank 2: TDLC300-100 ,TDLA30-10 |
|  |  | • Rank 4 and 8: TDLA30-10 |
| MCS |  | Rank 2: - up to 256QAM: [MCS26] (table 2); (Low correlation) (Only forTDLA30-10) - MCS16 (Medium B); - MCS19(Medium B) - MCS23(Low) Rank 4: - up to 64QAM: MCS26; - MCS17; Rank 8: - up to 64QAM: MCS20; - MCS13; |
| Overhead |  | Overhead = 0 |
| TRS configuration |  | • Configure TRS symbols in symbol#{5, 9} |
|  |  | • NZP CSI-RS configuration of row 6 for 8 layers. |
| N1 and N2 configuration for 8Tx case |  | (N1,N2)=(4,1) |

Simulation results: SNR 70% throughput

The Table 2 gives the SNR at 70% throughput using the simulation parameters presented in the previous section.

Table 2: SNR at 70% for different number of layers and MCS (MCS table 2).

| Rank | MCS | Channel Correlation | Propagation Condition | SNR @ 70% of max TP |
| --- | --- | --- | --- | --- |
| 2 | 26 | ULA Low | TDLA 30-10 | 12.41918426 |
|  | 23 |  |  | 10.62430605 |
|  | 19 | ULA Medium B |  | 15.54287335 |
|  | 16 |  |  | 12.6520153 |
|  | 26 | ULA Low | TDLC 300-10 | 14.25106447 |
|  | 23 |  |  | 11.73272948 |
|  | 19 | ULA Medium B |  | - |
|  | 16 |  |  | - |
| 4 | 26 | ULA Low | TDLA 30-10 | 13.41172957 |
|  | 17 |  |  | 6.122539248 |
| 8 | 20 (Both CW) |  |  | 8.396243874 |
|  | 13 (Both CW) |  |  | 8.396243874 |

Conclusion

In this contribution we provided the results of our simulation campaign on 8 Rx based on simulation assumption agreed last meeting to define UE demodulation performance requirements.

References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023